Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 2 Changi North St 2 | Mobile/Lorry Crane Operation | 1 | Transport of lifting machinery into or out of site premise |  |
|  |  |  | 2 | Setting up of crane |  |
|  |  |  | 3 | Lifting operation |  |
|  |  |  | 4 | Working at height on top platform |  |
|  |  |  | 5 | Adverse weather condition |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Mobile/Lorry Crane Operation | RA Member: Zailani |  |  |  |
| Location: | No 2 Changi North St 2 ETLA Ltd | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2020 |  | Designation: | Director |  |
| Next Review Date | 30/06/2023 |  | Date: | 17/12/2021 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Transport of lifting machinery into or out of site premise | Collision of moving vehicle onto personnel’s or site property | Bodily Injury & Damage to Property | - Vehicle only to be moved into site with the aid of banks man guiding the driver - Clear all obstruction in way of moving path - Drive at slow speed when entering into customer premise | 3 | 2 | 6 | Vehicle must not be left unattended while engine is running | 3 | 1 | 3 | Supervisor on site | 28-31 /12/21 |  |
| 2 | Setting up of crane | Hit by moving parts of crane | Bodily Injury & Damage to Property | - Operator trained and competent person appointed - Crane operator and lifting supervisor to ensure operation area and crane swing path is clear from any obstruction - Crane operator to be on site when extending or retracting outriggers (must be able to see all operation procedure) - Crane operator to ensure all outrigger beams is fully extended and resting on steel plate/timber mats as base plate - PPE (safety shoe, safety helmet) provided and put on | 4 | 2 | 8 | Supervisor to ensure no unauthorised personnel or vehicle near moving parts of crane Barricade off the work area | 4 | 1 | 4 | Crane operator on site | 28-31 /12/21 |  |
| 3 | Lifting operation | Failing of lifting equipment | Bodily Injury & Damage to Property | - All lifting equipment are tested by approved person and has valid certificate - Ensure LM for mobile/ lorry crane are valid - Riggers to ensure lifting gears use are in compliance with weight of machine - No defective gears allowed - Never stand close or under the load - PPE (safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | While moving or slewing the crane, try not to jerk the load Ensure lifting supervisor to be 100% onsite to supervised the whole lifting operation | 4 | 1 | 4 | Lifting supervisor on site | 28-31 /12/21 |  |
|  |  | Overloading | Bodily Injury & Damage to Property | - No lifting beyond SWL of equipment - Load weight should be known | 4 | 2 | 8 | Ensure load, capacity of lifting arm and reach are within limit, when in doubt, always use a higher capacity crane | 4 | 1 | 4 | Lifting supervisor on site | 28-31 /12/21 |  |
|  |  | Struck by objects due to unsafe slinging, rigging | Bodily Injury & Damage to Property | - Entire lifting crew is trained and to have relevant skill, certificate and experiences required to carry out tasks - Machine must be securely slug, properly balanced and trimmed to prevent from dislodgment - Ensure machine is rig at it centre of gravity - Riggers to check and ensure safe rigging and lifting supervisor to confirm load are safe to be rig | 4 | 2 | 8 | Communication through hand signal or via walkie-talkie by Eric Wong personnel only | 4 | 1 | 4 | Lifting supervisor on site | 28-31 /12/21 |  |
|  |  | Hand caught by or in between loose part of sling or lifting gears | Arm or Hand Injury | - Keep hands off loose part of sling or lifting gears - Ensure signalman give proper means of signal to crane operator - Use glove | 3 | 2 | 6 | Signalman to command crane operator when to lift and supervisor to ensure all workers are hand off any loose parts of lifting gears | 3 | 1 | 3 | Lifting supervisor on site | 28-31 /12/21 |  |
|  |  | View obstruction or load swing | Bodily Injury & Damage to Property | - Appointed and trained signalman, riggers and lifting supervisor on site - Taglines should be used to control load from swinging so as to prevent bodily contact with load - Clear obstruction and unauthorised personnel in way of lifting path - Never stand close or under the suspended load and away from possible swing path - PPE (safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | Signalman to position himself to be visible to crane operator through hand signalling method or via walkie-talkie | 4 | 1 | 4 | Lifting supervisor on site | 28-31 /12/21 |  |
| 4 | Working at height on top platform | Falling from height | Bodily Injury | - Supervisor to ensure workers working at height on top platform to put on safety harness, safety helmet - Self Retracting Lifeline provided and properly secured onto building pillar to act as anchorage point - Ensure safety harness use are properly strap on and hook onto Self Retracting Lifeline while working on top platform at all time - To ensure fall prevention equipment are adequate for all personnel working on top platform - Supervisor to ensure removable side railing of platform must be in place - Practice good housekeeping | 4 | 2 | 8 | Full supervision and regular remainder by supervisor and to ensure all safety precaution are in place Check all PPE use are in good working condition and free from defects | 4 | 1 | 4 | Lifting supervisor on site | 28-31 /12/21 |  |
|  |  | Platform move or sway | Bodily Injury & Damage to Property | - Ensure platform 3ft metal plate protruding is fully lean and rested on the building opening slab - Ensure platform is stabilise effectively - Use strong fastening belt to secured front left and right of platform onto building railing/pillar to prevent platform swing outward | 4 | 2 | 8 | Close supervision by site supervisor and to ensure platform is fully stabilise | 4 | 1 | 4 | Lifting supervisor on site | 28-31 /12/21 |  |
| 5 | Adverse weather condition | Poor visibility due to heavy rain / lightning strike | Cause unexpected danger in term of workers slipping/strike by lightning | - To stop work immediately if there is heavy rain / lightning strike - To evacuate to nearest shelter immediately if there is heavy rain / lightning strike in the area | 4 | 2 | 8 | Supervisor to ensure all machineries are turned off and no activity during heavy rain | 4 | 1 | 4 | Supervisor on site | 28-31 /12/21 |  |