Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 17 Changi South St 2 | Lifting Operation | 1 | Transport of lifting machinery into or out of site premises |  |
|  |  |  | 2 | Setting up of mobile/lorry crane on site |  |
|  |  |  | 3 | Lifting operation |  |
|  |  |  | 4 | Working at height on top platform |  |
|  |  |  | 5 | Weather climate |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader : Kevin Wong | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Mobile/Lorry Crane Operation | RA Member : Zailani |  |  |  |
| Location: | 17 Changi South St 2 #04-00 | RA Member : Zaharin |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 15/05/2015 |  | Designation: | Director |  |
| Next Review Date | 15/05/2018 |  | Date: | 28/06/2016 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Transport of lifting machinery into or out of site premises | Collision of moving vehicle onto personnel’s or site property | Bodily Injury & Damage to Site Property | - Vehicle only to be moved into site with the aid of banks man guiding the driver - Clear all obstruction in way of moving path - Drive at slow speed when entering into customer premise | 3 | 2 | 6 | Vehicle must not be left unattended while engine is running | 3 | 2 | 6 | Supervisor on site | 02/07/16 |  |
| 2 | Setting up of mobile/lorry crane on site | Topple of crane | Bodily Injury & Damage to Site Property | - Operator trained and competent person appointed - Crane operation manual and service manual made available - Crane operator to ensure all outrigger beams is fully extended and resting on steel plate/timber mats - Lifting area to be barricade, only relevant personnel is allowed within the lifting zone | 4 | 2 | 8 | Ensure lifting machine is on firm and stable ground | 4 | 2 | 8 | Lifting Supervisor on site | 02/07/16 |  |
| 3 | Lifting operation | Failing of lifting equipment | Bodily Injury & Damage to Property | - All lifting equipment are tested by approved person and has valid certificate - No defective gears allowed - Ensure LM for crane are valid - Riggers to ensure lifting gears use are in compliance with weight of machine - Never stand close or under the load - Lifting zone to be barricade, only relevant personnel is allowed to enter the lifting zone - PPE (etc.. safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | While moving or slewing the crane, try not to jerk the load | 4 | 2 | 8 | Lifting supervisor on site | 02/07/16 |  |
|  |  | Overloading | Bodily Injury & Damage to Property | - No lifting beyond SWL of equipment - Load weight should be known | 4 | 2 | 8 | Ensure load, capacity of lifting arm and reach are within limit, when in doubt, always use a higher capacity crane | 4 | 2 | 8 | Lifting supervisor on site | 02/07/16 |  |
|  |  | Struck by objects due to unsafe slinging, rigging | Bodily Injury & Damage to Property | - Entire lifting crew is trained and to have relevant skill, certificate and experiences required to carry out tasks - Load shall be securely slug, properly balanced and trimmed to prevent dislodgment - Ensure machine is rig at it centre of gravity | 4 | 2 | 8 | Communication through hand signal or via walkie-talkie by Eric Wong personnel only | 4 | 2 | 8 | Lifting supervisor on site | 02/07/16 |  |
|  |  | Hand caught by or in between loose part of sling or lifting gears | Arm or Hand Injury | - Keep hands off loose part of sling or lifting gears - Ensure signalman give proper means of signal to crane operator - Use glove | 3 | 2 | 6 | Good coordination between workers | 3 | 2 | 6 | Lifting supervisor on site | 02/07/16 |  |
|  |  | View obstruction or poor visibility, load swing | Bodily Injury & Damage to Property | - Appointed and trained signalman, riggers and lifting supervisor on site - Tagline for load when necessary to prevent of load swing - Clear obstruction and unauthorised personnel in way of lifting path - Never stand close or under the suspended load and away from possible swing path - PPE (etc.. safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | Signalman to position himself to be visible to crane operator through hand signalling method or via walkie-talkie | 4 | 2 | 8 | Lifting supervisor on site | 02/07/16 |  |
| 4 | Working at height on top platform | Falling from height | Bodily Injury | - Workers working at height to put on safety harness, safety helmet - Ensure safety harness use is in good working condition, properly strapped on and hook to strong anchorage point at all time - Practice good housekeeping | 4 | 2 | 8 | Close supervision by supervisor at all time | 4 | 2 | 8 | Lifting supervisor on site | 02/07/16 |  |
|  |  | Platform move or sway | Bodily Injury & Damage to Property | - Ensure platform is fully rested on the opening slab - Ensure platform is stabilise effectively - Use strong fastening belt to secured front left and right of platform to building railing to prevent platform swing outward | 4 | 2 | 8 | Close supervision by supervisor | 4 | 2 | 8 | Lifting supervisor on site | 02/07/16 |  |
| 5 | Weather climate | Poor visibility due to heavy rain / lightning strike | Cause unexpected danger in term of workers slipping/strike by lightning | - To stop work immediately if there is heavy rain / lightning strike | 4 | 2 | 8 | Supervisor to ensure no activity during heavy rain | 4 | 2 | 8 | Supervisor on site | 02/07/16 |  |