Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 14 Senoko Way | Mobile crane operation | 1 | Transport of lifting machinery into or out of site premise |  |
|  |  |  | 2 | Setting up of crane on site |  |
|  |  |  | 3 | Attached sling to crane hook and secured to rigging point of chiller |  |
|  |  |  | 4 | Lifting operation |  |
|  |  |  | 5 | Signalling of load to location |  |
|  |  |  | 6 | Terror threat |  |
|  |  |  | 7 | Work execution during pandemic spreading period |  |
|  |  |  | 8 | Mental wellness |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Mobile Crane Operation | RA Member: Zailani |  |  |  |
| Location: | 14 Senoko Way | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2023 |  | Designation: | Director |  |
| Next Review Date: | 30/06/2026 |  | Date: | 22/12/2025 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Transport of lifting machinery into or out of site premise | Collision of moving vehicle onto personnel’s or site property | Bodily Injury & Damage to Property | - To coordinate with occupier for safe route access - Vehicle only to be moved into site with the aid of banks man guiding the driver - Clear all obstruction in way of moving path - Driver to drive at slow speed and observed speed limit when entering into customer premise | 4 | 2 | 8 | Vehicle must not be left unattended while engine is running | 4 | 1 | 4 | Supervisor on site | 27/12/25 |  |
| 2 | Setting up of crane on site | Hit by moving parts of crane | Bodily Injury & Damage to Property | - Operator trained and competent person appointed - Crane operator and lifting supervisor to ensure operation area and crane swing path is clear from any obstruction - Crane operator to be on site when extending or retracting outriggers (must be able to see all operation procedure) - Crane operator to ensure all outrigger beams is fully extended and resting on steel plate/timber mats as base plate - To determine the load radius and slewing part of the crane to ensure no hitting of nearby building or equipment - PPE (safety shoe, safety helmet) provided and put on | 5 | 2 | 10 | Ensure no unauthorised personnel or vehicle near moving parts of crane Barricade off the work area Signage to be up at the barricade area Assignment of traffic controller to control movement of vehicle and people within the vicinity | 5 | 1 | 5 | Lifting Supervisor on site | 27/12/25 |  |
| 3 | Attached sling to crane hook and secured to rigging point of chiller | Rigger hand caught in between the LG and hook | Minor fracture to hand | - Rigger to attached and secure sling with care - To coordinate well with crane operator to ensure no lifting are done till all sling are properly attached and secured onto machine - Use glove | 3 | 2 | 6 | Signalman to command crane operator when to lift and supervisor to ensure all worker are hand off any loose part of lifting gear | 3 | 1 | 3 | Lifting Supervisor on site | 27/12/25 |  |
|  |  | Pinch point | Injury to hand | - PPE safety glove) provided and put on - Never place hand where you can’t see them - If view is obstructed, do not proceed with the work until having a clearer view | 3 | 2 | 6 | Always stay vigilant and pay close attention to surrounding area | 3 | 1 | 3 | Lifting Supervisor on site | 27/12/25 |  |
| 4 | Lifting operation | Failing of lifting equipment | Bodily Injury & Damage to Property | - All lifting equipment are tested by approved person and has valid certificate - Ensure LM for mobile/ lorry crane are valid - Riggers to ensure lifting gears use are in compliance with weight of machine - No defective gears allowed - Never stand close or under the load - PPE (safety helmet, safety shoe) provided and put on | 5 | 2 | 10 | While moving or slewing the crane, do it smoothly without sudden acceleration or deceleration Ensure lifting supervisor to be 100% onsite to supervised the whole lifting operation No lifting is to be done during bad weather | 5 | 1 | 5 | Lifting Supervisor on site | 27/12/25 |  |
|  |  | Overloading | Bodily Injury & Damage to Property | - No lifting beyond SWL of equipment - Load weight should be known - To test lift load about 1ft above ground to check on stability and to ensure no overloading of load to be lifted - Implement of Lifting Plan to all lifting personnel on site to ensure that lifting operation is properly manage | 5 | 2 | 10 | Ensure load, capacity of lifting arm and reach are within limit, when in doubt, always use a higher capacity crane | 5 | 1 | 5 | Lifting Supervisor on site | 27/12/25 |  |
|  |  | Struck by objects due to unsafe slinging, rigging | Bodily Injury & Damage to Property | - Entire lifting crew is trained and to have relevant skill, certificate and experiences required to carry out tasks - Load shall be securely slug, properly balanced and trimmed to prevent dislodgment - Ensure machine is rig at it centre of gravity - Riggers to check and ensure safe rigging and lifting supervisor to confirm load are safe to be rig | 5 | 2 | 10 | Communication through hand signal or via walkie-talkie by Eric Wong personnel only To provide crane hook with safety latch to prevent displacement of sling or load from the hook | 5 | 1 | 5 | Lifting Supervisor on site | 27/12/25 |  |
|  |  | Hand caught by or in between loose part of sling or lifting gears | Arm or Hand Injury | - Keep hands off loose part of sling or lifting gears - Ensure signalman give proper means of signal to crane operator - Use glove | 3 | 2 | 6 | Signalman to command crane operator when to lift and to ensure all workers are hand off any loose parts of sling | 3 | 1 | 3 | Lifting Supervisor on site | 27/12/25 |  |
| 5 | Signalling of load to location | Load hit structure due to improper signalling | Bodily Injury & Damage to Property | - Only qualified signalman to execute the operation - Tagline should be used to control load from swinging so as to prevent bodily contact with load - Clear obstruction and unauthorised personnel in way of lifting path - Never stand close or under the suspended load and away from possible swing path - Signalman to position himself to be visible to crane operator through hand signalling method - PPE (safety helmet, safety shoe) provided and put on | 5 | 2 | 10 | Use of walkie-talkie to communicate whenever signalman is not visible to crane operator Close supervision by lifting supervisor | 5 | 1 | 5 | Lifting Supervisor on site | 27/12/25 |  |
| 6 | Terror threat | Suspicious parcel containing improvised explosive device | Death / Multiple Injury | - Evacuate quickly - Contact police as soon as possible - To submit all names and vehicle number to prevent unauthorised vehicular to access to workplace | 5 | 1 | 5 | NIL | 5 | 1 | 5 | Supervisor on site | 27/12/25 |  |
|  |  | Suspicious parcel containing chemical agent | Death / Multiple Injury | - Look out for symptoms from people who come into contact with substance - Locate contaminated area - Cover mouth, nose with damp doth, quickly leave the building without passing through contaminated area - Seek shelter a distance away from incident site - Contact police as soon as possible when safe | 5 | 1 | 5 | NIL | 5 | 1 | 5 | Supervisor on site | 27/12/25 |  |
|  |  | Weapon assault | Death / Multiple Injury | - Perform run, run, tell steps - Prevention of terrorist treat - Prevent unauthorised vehicular access to workplace premises | 5 | 1 | 5 | NIL | 5 | 1 | 5 | Supervisor on site | 27/12/25 |  |
|  |  | Vehicle ramming attack | Death / Multiple Injury | -Closed monitor CCTV by security guard - Prevent unauthorised vehicular access to workplace premises | 5 | 1 | 5 | NIL | 5 | 1 | 5 | Supervisor on site | 27/12/25 |  |
| 7 | Work execution during pandemic spreading period | Probability getting infected by pandemic | Body illness, severe breathing issue | -Workers should follow safe distance 1m - All working personnel should frequently wash hands properly - Sanitizers should be used frequently to keep hands hygienic - Should not shake hand or sharing foods and beverages - Wear face mask all the time | 5 | 1 | 5 | Avoid crowded places or gatherings | 5 | 1 | 5 | Supervisor on site | 27/12/25 |  |
| 8 | Mental wellness | Mental health and well being affected through stress | Stress, severe depression | - Discuss issues of fatigue with employees and make sure they take regular break, are encouraged to take leave - Work within working hours to have enough rest - Keep workers inform of what is happening so they feel reassured | 5 | 1 | 5 | Sharing of information and advice to worker on mental wellbeing Consider an occupational health referrer if personnel stress and anxiety are identified | 5 | 1 | 5 | Supervisor on site | 27/12/25 |  |