Inventory of Work Activities

| Company: Eric Wong Machinery Transportation Pte Ltd |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 3 Pioneer Walk | Forklift & moving operation | 1 | Transport of lifting machinery into or out of site premises |  |
|  |  |  | 2 | Forklift operation – to fork down machine from lorry chassis and transport to door entrance |  |
|  |  |  | 3 | Transport machine to designated location using pallet truck |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Forklift / Moving Operation | RA Member: Zailani |  |  |  |
| Location: | 3 Pioneer Walk Wieland Metals (S) Pte Ltd | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2023 |  | Designation: | Director |  |
| Next Review Date | 30/06/2026 |  | Date: | 28/07/2025 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Transport of lifting machinery into or out of site premises | Collision of moving vehicle onto personnel’s or site property | Bodily Injury & Damage to Property | - To coordinate with the occupier for safe route access - Vehicle only to be moved into site with the aid of banks man guiding the driver - Clear all obstruction in way of moving path - Driver to drive at slow speed and observe speed limit when entering into customer premise | 4 | 2 | 8 | Vehicle must not be left unattended while engine is running | 4 | 1 | 4 | Supervisor on site | 01/08/25 |  |
| 2 | Forklift operation – to fork down machine from lorry chassis and transport to door entrance | Falling load | Bodily Injury & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Ensure forklift fork is properly aligned directly to the lifting point of machine - Known the CG of the load - Ensure machine load are spread evenly and balanced well on the fork - Elevate the load slightly and fully tilt the mast backward - Ensure no unauthorised personnel is within the working zone | 4 | 2 | 8 | Use extension fork if needed to give greater stability of load | 4 | 1 | 4 | Supervisor on site | 01/08/25 |  |
|  |  | Restricted driver visibility | Bodily Injury & Damage to Property | - Forklift driver advise to be careful when moving forward or reversing - Ensure forklift driver is guided by other staff - Press horn if necessary to warn others that forklift is approaching - If required, reverse driving with reverse light on - Drive at slow transportation speed | 3 | 2 | 6 | Assignment of traffic controller to control movement of vehicle and personnel within the vicinity | 3 | 1 | 3 | Supervisor on site | 01/08/25 |  |
| 3 | Transport machine to designated location using pallet truck | Unstable load during moving or turning | Bodily Injury & Damage to Property if machine topper | - Ensure pallet truck are place accordingly on machine base - Ensure machine load are spread evenly and balanced well on the pallet truck before being transport to location - Workers to be alert and keep close lookout on route of travel from point to point | 4 | 2 | 8 | Slow moving speed | 4 | 1 | 4 | Supervisor on site | 01/08/25 |  |
|  |  | Trip & Fall | Sprains & strains | - Passage way and working area to be keep clear of tooling and materials before moving job - Proper housekeeping to be maintained during work - Keep the floor in the work area dry and clean - Oil/water spill must be clean up immediately - Tape down any power cords or cable if any to prevent from tripping | 2 | 2 | 4 | Supervisor to make sure route of transporting before moving and double check clearing of all material and tooling | 2 | 1 | 2 | Supervisor on site | 01/08/25 |  |
|  |  | Obstruction to vision while moving | Damage to Property if hit other obstacles | -To keep close lookout on route of travel - Ensure movement is guided - To alert personnel controlling the pallet truck steering if potential hitting of other obstacles | 1 | 2 | 2 | Provision of spotter at both end of machine so as to have a clearer view before hitting any other obstacles | 1 | 2 | 2 | Supervisor on site | 01/08/25 |  |