Inventory of Work Activities

| Company: Eric Wong Machinery Transportation Pte Ltd |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 6 Fusionopolis Way | Moving / forklift operation | 1 | Transport of lifting machinery into or out of site premise |  |
|  |  |  | 2 | Shift out machine to loading area |  |
|  |  |  | 3 | Using forklift to fork up machine onto wooden base |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Lorry Loader / Forklift Operation | RA Member: Zailani |  |  |  |
| Location: | 6 Fusionopolis Way, Synthesis Building B2-22 National Metrology Centre | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2020 |  | Designation: | Director |  |
| Next Review Date | 30/06/2023 |  | Date: | 27/05/2022 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Transport of lifting machinery into or out of site premises | Collision of moving vehicle onto personnel’s or site property | Bodily Injury & Damage to Property | - Vehicle to be move into site with the aid of bank’s man guiding the driver - Clear all obstruction in way of moving path - Drive at slow speed when entering into customer premise | 4 | 2 | 8 | Vehicle must not be left unattended while engine is running | 4 | 1 | 4 | Supervisor on site | 09/06/22 |  |
| 2 | Shift out machine to loading area using pallet truck | Unstable load during moving or turning | Bodily Injury & Damage to Property if machine topper | - Ensure pallet truck are place accordingly on machine base - Ensure machine load are spread evenly and balanced well on the pallet truck before being transport to location - Workers to be alert and keep close lookout on route of travel from point to point | 4 | 2 | 8 | Slow moving speed | 4 | 1 | 4 | Supervisor on site | 09/06/22 |  |
|  |  | Passage way block by materials or tooling | Trip & Fall | -Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Oil/water spill must be clean up immediately - Ensure travelling ground is well protected with plywood in contact with the pallet truck rollers at all time | 2 | 2 | 4 | Supervisor to make sure route of transporting before moving and to double check clearing of all materials and tooling | 2 | 1 | 2 | Supervisor on site | 09/06/22 |  |
|  |  | Obstruction to vision while moving | Damage to property if hit other obstacles | - To keep close lookout on route of travel - Ensure movement is guided - To alert personnel controlling the pallet truck steering if potential hitting of other obstacles | 2 | 2 | 4 | Provision of spotter at both end of compressor so as to have a clearer view before hitting any obstacles | 2 | 1 | 2 | Supervisor on site | 09/06/22 |  |
|  |  | Laying of canvas/metal sheet for floor protection | Cut from sharp edges | - To replace defective metal sheet with a new one - PPE (safety glove) to be provided and put on when carrying metal sheet - Buddy system-two worker to lift one piece of metal sheet and lay out onto flooring at a time | 2 | 2 | 4 | Regular check by storeman and ensure equipment issue is in good condition | 2 | 1 | 2 | Supervisor on site | 09/06/22 |  |
| 3 | Load machine onto lorry chassis | Falling load | Bodily Injury & Damage to Property | - - Ensure no unauthorised personnel is within the working area | 4 | 2 | 8 | Use extension fork if required to give greater stability of load | 4 | 1 | 4 | Supervisor on site | 09/06/22 |  |
|  |  | Hitting people or property | Bodily Injury & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Operator advised to be careful when moving forward or reversing - Ensure forklift driver is guided by other staff | 4 | 2 | 8 | Press horn if necessary to warn others that forklift is approaching | 4 | 1 | 4 | Supervisor on site | 09/06/22 |  |