Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | Lorong Halus Jetty | Shifting of container | 1 | Transport of lifting machinery into or out of site premises |  |
|  |  |  | 2 | Jack up container using hydraulic jack |  |
|  |  |  | 3 | Insert rollers under container bed |  |
|  |  |  | 4 | Shifting of container |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by: Eric Wong Signature | Reference Number: |
| --- | --- | --- | --- | --- |
| Process: | Shifting of container | RA Member: Zailani |  |  |
| Activity Location: | Lorong Halus Jetty | RA Member: Daniel Wong |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: Eric Wong |  |
| Last Review Date: | 01/07/2023 |  | Designation: Director |  |
| Next Review Date | 30/06/2026 |  | Date: 27/03/2025 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Transport of lifting machinery into or out of site premises | Collision of moving vehicle onto personnel’s or site property | Bodily Injury & Damage to Property | - To coordinate with the occupier for safe route access - Vehicle only to be moved into site with the aid of banks man guiding the driver - Clear all obstruction in way of moving path - Driver to observed speed limit when entering into customer premise | 4 | 2 | 8 | Vehicle must not be left unattended while engine is running | 4 | 1 | 4 | Supervisor on site | TBA |  |
| 2 | Jack up container using hydraulic jack | Container topple due to lost of balance upon lifted up | Bodily Injury & Damage to Property | - Workers to be alert when jack up container - Ensure container are jack up at the right angle of gravity - Jack up container to 4” high from one side to another is carried out in stages with the use of solid woodblock to be insert below container bed - Supervisor to ensure operation area is clear of unauthorised personnel | 5 | 2 | 10 | Only skill and experience worker to control the jack handling | 5 | 1 | 5 | Supervisor on site | TBA |  |
| 3 | Insert rollers under container bed | Sharp edges | Cuts form sharp edges | - To keep hand/finger clear of sharp edges - Insert rollers to below container with extra care - Use glove | 2 | 2 | 4 | Close supervision by supervisor | 2 | 2 | 4 | Supervisor on site | TBA |  |
|  |  | Pinch point | Injury to hand | - PPE (safety glove) provided and put on - Never place hand where you can’t see them - If view is obstructed, do not proceed with the work until having a clearer view - To standby solid woodblock under container and remove only after rollers are properly secured onto container bed | 3 | 2 | 6 | Always stay vigilant and pay close attention to surrounding area | 3 | 1 | 3 | Supervisor on site | TBA |  |
| 4 | Shifting of container | Unstable load during moving or turning | Bodily Injury & Damage to Property if load topper | - Ensure rollers are place accordingly on container bed - Ensure rollers are properly secured onto container bed with bolts and nuts - Tighten bolts and nuts with adjustable spanner - Ensure container load are spread evenly and balance well on the rollers before being transport to location - Workers to be alert and keep close lookout on route of travel from point to point and to ensure rollers are not moving out of position | 5 | 2 | 10 | Slow moving speed | 5 | 1 | 5 | Supervisor on site | TBA |  |
|  |  | Trip and fall | Sprains & strains | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Keep the floor in the work area dry and clean - Oil/water spill must be clean up immediately - Tape down power cords or cable if any to prevent from tripping | 2 | 2 | 4 | Supervisor to make sure route of transporting before moving and double check clearing of all material and tooling | 2 | 2 | 4 | Supervisor on site | TBA |  |
|  |  | Obstruction to vision while moving | Damage to Property when hitting other obstacles | -To keep close lookout on route of travel - Ensure movement is guided - To alert worker controlling the rollers steering if potential hitting of other obstacles | 1 | 2 | 2 | Provision of spotter on both end of machine so as to have a clearer view before hitting any obstacles | 1 | 2 | 2 | Supervisor on site | TBA |  |