Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 4 Gul Way | Lorry Loader Operation | 1 | Transport of lifting machinery into or out of site premises |  |
|  |  |  | 2 | Setting up of crane on site |  |
|  |  |  | 3 | Lifting Operation |  |
|  |  |  | 4 | Stuffing small cases into container using forklift |  |
|  |  |  | 5 | Using pallet truck to arrange cases inside container |  |
|  |  |  | 6 | Use of nail gun to secured choking wood |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Lorry Loader Operation | RA Member: Zailani |  |  |  |
| Location: | 4 Gul Way National Oilwell Varco Pte Ltd | RA Member: Zaharin |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 15/05/2015 |  | Designation: | Director |  |
| Next Review Date | 15/05/2018 |  | Date: | 19/11/2017 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Transport of lifting machinery into or out of site premises | Collision of moving vehicle onto personnel’s or site property | Bodily Injury & Damage to Site Property | - Vehicle only to be moved into site with the aid of banks man guiding the driver - Clear all obstruction in way of moving path - Drive at slow speed when entering into customer premise - To ensure height of entrance door is high enough for lorry to transport in and out of production area | 3 | 2 | 6 | Vehicle must not be left unattended while engine is running Provision of spotter to ensure height clearance of door entrance before hitting any obstacles | 3 | 1 | 3 | Supervisor on site | 21-27 /11/2018 |  |
| 2 | Setting up of crane | Hit by moving parts of crane | Bodily Injury & Damage to Site Property | - Operator trained and competent person appointed - Crane operator and lifting supervisor to ensure operation area and crane swing path is clear from any obstruction - Crane operator to be on site when extending or retracting outriggers (must be able to see all operation procedure) - Ensure all outrigger beams must be full extended and resting on steel plate/timber mat - PPE (safety shoe, safety helmet) provided and put on | 4 | 2 | 8 | Ensure no unauthorised personnel near moving parts of crane | 4 | 1 | 4 | Lifting supervisor on site | 21-27 /11/2018 |  |
| 3 | Lifting operation | Failing of lifting equipment | Bodily Injury & Damage to Property | - All lifting equipment are tested by approved person and has valid certificate - Ensure LM for lorry crane are valid - Riggers to ensure lifting gears use are in compliance with weight of machine - No defective gears allowed - Never stand close or under the load - PPE (safety shoe, safety helmet) provided and put on | 4 | 2 | 8 | While moving or slewing the crane, try not to jerk the load | 4 | 1 | 4 | Lifting supervisor on site | 21-27 /11/2018 |  |
|  |  | Overloading | Bodily Injury & Damage to Property | - No lifting beyond SWL of equipment - Load weight should be known | 4 | 2 | 8 | Ensure load, capacity of lifting arm and reach are within limit, when in doubt, always use a higher capacity crane | 4 | 1 | 4 | Lifting supervisor on site | 21-27 /11/2018 |  |
|  |  | Struck by objects due to unsafe slinging, rigging | Bodily Injury & Damage to Property | - Entire lifting crew is trained and to have relevant skill, certificate and experiences required to carry out tasks - Load shall be securely slug, properly balanced and trimmed to prevent from dislodgment - Ensure load is rig at the centre of gravity | 4 | 2 | 8 | Communication through hand signal or via walkie-talkie by Eric Wong personnel only | 4 | 1 | 4 | Lifting supervisor on site | 21-27 /11/2018 |  |
|  |  | Hand caught by or in between loose part of sling or lifting gears | Arm or Hand Injury | - Keep hands off loose part of sling or lifting gears - Ensure signalman give proper means of signal to crane operator - Use glove | 3 | 2 | 6 | Supervisor to ensure all workers are hand off any loose part of lifting gears | 3 | 1 | 3 | Lifting supervisor on site | 21-27 /11/2018 |  |
|  |  | View obstruction or load swing | Bodily Injury & Damage to Property | - Appointed and trained signalman, riggers and lifting supervisor on site - Tagline for load when necessary to prevent of load swing - Clear obstruction and unauthorised personnel in way of lifting path - Never stand close or under the suspended load and away from possible swing path - PPE (safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | Signalman to position himself to be visible to crane operator through hand signalling method or via walkie-talkie | 4 | 1 | 4 | Lifting supervisor on site | 21-27 /11/2018 |  |
| 4 | Stuffing small cases into container using forklift | Falling load | Bodily Injury & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Known the CG of the load and ensure load are balanced well on the fork - Elevate the cargo slightly and fully tilt the mask backward | 4 | 2 | 8 | Use extension fork if required to give greater stability of load | 4 | 1 | 4 | Supervisor on site | 21-27 /11/2018 |  |
|  |  | Restricted driver visibility | Bodily Injury & Damage to Property | - Forklift driver to ensure surrounding area is clear of unauthorised personnel or property before proceeding to drive - Forklift driver are advised to be careful when moving forward or reversing - To ensure forklift driver is guided and workers to be alert when stuffing cases into container so as not to hit side of container - Slow transportation speed | 4 | 2 | 8 | Assignment of traffic controller | 4 | 1 | 4 | Supervisor on site | 21-27 /11/2018 |  |
| 5 | Using pallet truck to arrange cases inside container | Excessive weight load | Sprain back when pushing heavy load | - Use the correct capacity pallet truck to reduce the effort of each man when pushing the load - Buddy system-one to control the pallet truck steering while the others to push the load | 3 | 2 | 6 | Supervisor to ensure there are adequate personnel to be deploy for the job | 3 | 1 | 3 | Supervisor on side | 21-27 /11/2018 |  |
| 6 | Use of nail gun to secured choking wood | Mishandling of nail gun | Bodily Injury | - Only trained and competent personnel are allowed to use tools that are issue to them - Understudy personnel need the supervision of a trained personnel - Personnel handling the nail gun to ensure route of projectiles is clear of unauthorised personnel in case of miss fired | 4 | 2 | 8 | Supervisor to ensure no unauthorised personnel is allowed to be near or within the working area | 4 | 1 | 4 | Supervisor on site | 21-27 /11/2018 |  |