Inventory of Work Activities

| Company: Eric Wong Machinery Transportation Pte Ltd |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 500 Dover Rd | Forklift & ‘A’ Frame operation | 1 | Fork up base together with gear box to 2nd floor corridor |  |
|  |  |  | 2 | Shift out base from platform |  |
|  |  |  | 3 | Lift up machine to 3rd floor using ‘A’ frame and chain block |  |
|  |  |  | 4 | Transportation of machine to designated location |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Forklift & A Frame Operation | RA Member: Zailani |  |  |  |
| Location: | 500 Dover Rd, Singapore Polytechnic W2 – W216 | RA Member: Zaharin |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2017 |  | Designation: | Director |  |
| Next Review Date | 30/06/2020 |  | Date: |  |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Fork up base together with gear box to 2nd floor corridor | Falling load | Bodily Injury & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Known the CG of the load - To ensure machine is firmly secured onto platform with strapping belt and balanced well before proceeding to fork up to 2nd floor - Elevate the load slightly and fully tilt the mast backward - To avoid sudden jerk of the load - To ensure no unauthorised personnel is below or within the working zone | 4 | 2 | 8 | Use extension fork if needed to give greater stability of load | 4 | 1 | 4 | Supervisor on site |  |  |
|  |  | Restricted driver visibility | Bodily Injury & Damage to Property | - Drive at slow speed - Forklift driver advised to be careful when moving forward or reversing - Ensure forklift driver is guided by other stuff | 3 | 2 | 6 | Ensure proper means of signal is given to forklift driver as when to lift up and lower down the platform when reaching 2nd floor | 3 | 1 | 3 | Supervisor on site |  |  |
| 2 | Shift out base from platform | Unstable load | Bodily Injury | - Ensure pallet truck are place accordingly on wooden base - Ensure machine load are spread evenly and balanced well on the pallet truck | 4 | 2 | 8 | Slow moving speed | 4 | 1 | 4 | Supervisor on site |  |  |
| 3 | Lift up machine to 3rd floor using A frame and chain block | Failing of lifting equipment | Bodily Injury & Damage to Property | - All lifting equipment use on site are tested by approved person and has valid certificate - To ensure all lifting gears use are in compliance with weight of machine to be lift - No lifting beyond SWL of equipment - Ensure no unauthorised personnel is below or near the load when being lift up | 4 | 2 | 8 | When lifting up, try not to jerk the load | 4 | 1 | 4 | Supervisor on site |  |  |
|  |  | Struck by load due to unsafe slinging or rigging | Bodily Injury & Damage to Property | - Entire crew is trained and have relevant skill and experience to carry out task - Load must be securely slug and properly balanced to prevent form dislodgment - To ensure load is rig at the centre of it gravity | 4 | 2 | 8 | Close supervision by supervisor | 4 | 1 | 4 | Supervisor on site |  |  |
|  |  | Manual pulling of chain block | Muscular strain | - Communicate between personnel ensuring everyone is clear of task - Supervisor to ensure there are adequate manpower to be deploy for the job - Buddy system-rotating of workers when pulling the chain block | 3 | 2 | 6 | Supervisor to be responsible for giving instruction to ensure load is pull up slowly and balanced well | 3 | 1 | 3 | Supervisor on site |  |  |
| 4 | Transportation of machine to designated location | Unstable load during moving or turning | Bodily Injury & Damage to Property if machine topper | - Ensure pallet truck/rollers are place accordingly on machine base - Ensure machine load are spread evenly and balanced well on the pallet truck/ rollers before being transport to location - Workers to be alert and keep close lookout on route of travel from point to point | 4 | 2 | 8 | Slow moving speed | 4 | 1 | 4 | Supervisor on site |  |  |
|  |  | Passage way block by tooling / materials | Trip & Fall | - Passage way and working area to be keep clear of tooling/materials before moving job - Proper housekeeping to be maintained during work - Oil/water spill must be clean up immediately | 2 | 2 | 4 | Supervisor to make sure route of transporting before moving and to double check clearing of all materials and tooling | 2 | 1 | 2 | Supervisor on site |  |  |
|  |  | Obstruction to vision while moving | Damage to Property when hitting other obstacles | - To keep close lookout on route of travel - Ensure movement is guided - To alert worker controlling the pallet truck/rollers steering if potential hitting of others obstacles | 2 | 2 | 4 | Provision of spotter on both side of machine so as to have a clearer view before hitting any obstacles | 2 | 1 | 2 | Supervisor on site |  |  |