Inventory of Work Activities

| Company: Eric Wong Machinery Transportation Pte Ltd |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 801 Toa Payoh Lorong 7 | Moving & Forklift Operation | 1 | Insert pallet truck |  |
|  |  |  | 2 | Transportation of machine to loading bay |  |
|  |  |  | 3 | Using forklift to load machine onto lorry chassis |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Moving & Forklift Operation | RA Member: Zailani |  |  |  |
| Location: | 801 Toa Payoh Lorong 7, WBL Building O’Connor’s Annex Block | RA Member: Zaharin |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2017 |  | Designation: | Director |  |
| Next Review Date | 30/06/2020 |  | Date: | 25/07/2018 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Insert pallet truck | Sharp edges of machine | Hand & Finger Injury | - Keep hand and finger clear of sharp edges - Insert pallet truck to machine base with care - Use glove | 2 | 2 | 4 | Close supervision by supervisor | 2 | 1 | 2 | Supervisor on site | 25/07/18 |  |
| 2 | Transportation of machine to loading bay | Unstable load while moving / turning | Bodily Injury & Damage to Property | - Ensure pallet truck are place accordingly on machine base - Ensure machine is stable on the pallet truck before being transport to loading bay - Buddy system-one to control the pallet truck steering while the others to push and hold onto the machine - Workers to be alert and keep close lookout on route of travel from point to point | 4 | 2 | 8 | Slow moving speed | 4 | 1 | 4 | Supervisor on site | 25/07/18 |  |
|  |  | Passage way block by material and tools | Trip & Fall | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Oil/water spill must be clean up immediately | 2 | 2 | 4 | Supervisor to ensure route of transporting before moving and double check clearing of all materials and tooling | 2 | 1 | 2 | Supervisor on site | 25/07/18 |  |
|  |  | Obstruction to vision while moving | Bodily Injury & Damage to Property | - To keep close lookout on route of travel - Ensure movement is guided - To alert the worker controlling the pallet truck steering if potential hitting of other obstacles | 2 | 2 | 4 | Provision of spotter on both end of machine somas to have clearer view before hitting any | 2 | 1 | 2 | Supervisor on site | 25/07/18 |  |
| 3 | Using forklift to load machine onto lorry chassis | Falling load | Bodily Injury & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Known the CG of the load - Ensure stability of load before proceeding to drive - Elevate the load slightly and fully tilt the mast backward - Ensure stability of load before driving off | 4 | 2 | 8 | Use extension fork if needed to give greater stability of load | 4 | 1 | 4 | Supervisor on site | 25/07/18 |  |
|  |  | Hitting people or property | Bodily Injury & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Operator advised to be careful when moving forward or reversing - Ensure working area is clear of unauthorised personnel or any obstruction before any moving job is done | 4 | 2 | 8 | Assignment of traffic controller to guide and control movement of vehicle or personnel within the vicinity | 4 | 1 | 4 | Supervisor on site | 25/07/18 |  |
|  |  | Restricted driver visibility | Bodily Injury & Damage to Property | - Forklift driver advised to be careful when moving forward or reversing - Ensure forklift driver is guided by other staff - Press horn if necessary to warn others that forklift is approaching - Drive at slow transportation speed | 4 | 2 | 8 | Assignment of traffic controller | 4 | 1 | 4 | Supervisor on site | 25/07/18 |  |