Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 51 Seletar Aerospace View | Installation of machine cover using ‘A’ frame | 1 | Fixing of ‘A’ frame |  |
|  |  |  | 2 | Lift up machine cover |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong | Approved by : Eric Wong Signature: | Reference Number: |
| --- | --- | --- | --- | --- |
| Process: | Installation of machine cover using ‘A’ Frame | RA Member: Zailani |  |  |
| Activity Location: | 51 Seletar Aerospace View Pratt & Whitney Singapore | RA Member: Zaharin |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: Eric Wong |  |
| Last Review Date: | 01/07/2017 |  | Designation: Director |  |
| Next Review Date | 30/06/2020 |  | Date:12/02/2018 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Fixing of ‘A’ frame | Manual handling of heavy equipment | Sprain back when carrying heavy load | - Supervisor to ensure there are adequate manpower to be deploy for the job - Buddy system when handling of heavy equipment | 3 | 2 | 6 | Use of mechanical aid such as lifter if required | 3 | 1 | 3 | Supervisor on site | 13/02/18 |  |
|  |  | Working at height to fix bolt and nut onto frame | Falling from height | - Use of ‘A’ frame ladder to work on which is unreachable from ground - Ladder use must be fully extended and stand on level ground - Buddy system- one to hold onto the ladder while the other is working on top of ladder | 4 | 2 | 8 | Close supervision by supervisor | 4 | 1 | 4 | Supervisor on site | 13/02/18 |  |
| 2 | Lift up machine cover | Failing of lifting equipment | Bodily injury & damage to property | - All lifting equipment use are tested by approved person and has valid certificate - To ensure lifting gears use are in compliance with weight of machine - No lifting beyond SWL of equipment - Ensure no unauthorised personnel near working area | 4 | 2 | 8 | When lift up machine cover, try not to jerk the load | 4 | 1 | 4 | Supervisor on site | 13/02/18 |  |
|  |  | Struck by load due to unsafe slinging or rigging | Bodily injury & damage to property | - Entire crew is trained and to have relevant skill and experience to carry out task - Load must be securely slug and properly balanced to prevent from dislodgment - Ensue load is rig at the centre of gravity | 4 | 2 | 8 | Close supervision by supervisor | 4 | 1 | 4 | Supervisor on site | 13/02/18 |  |