Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 22 Pioneer Crescent | Lift up machine using ‘A’ frame structure | 1 | Fixing of ‘A’ frame structure |  |
|  |  |  | 2 | Lift up machine onto stand |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by : Eric Wong Signature: | Reference Number: |
| --- | --- | --- | --- | --- |
| Process: | Lift up machine using ‘A’ Frame structure | RA Member: Zailani |  |  |
| Activity Location: | 22 Pioneer Crescent Dyson | RA Member: Daniel Wong |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: Eric Wong |  |
| Last Review Date: | 01/07/2020 |  | Designation: Director |  |
| Next Review Date | 30/06/2023 |  | Date:16/12/2020 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Fixing of ‘A’ frame | Manual handling of heavy equipment | Sprain back when carrying heavy load | - Supervisor to ensure there are adequate manpower to be deploy for the job - Buddy system when handling of heavy equipment - PPE (safety helmet, safety shoe) provided and put on | 3 | 2 | 6 | Use of mechanical aid such as lifter if require | 3 | 1 | 3 | Supervisor on site Ibrahim / Zahari | 19/12/20 |  |
|  |  | Working at height to fix bolt and nut onto frame | Falling from height | - Use of ‘A’ frame ladder to work on which is unreachable from ground - Ladder use must be fully extended and stand on level ground - Buddy system- one to hold onto the ladder while the other is working on top of ladder | 4 | 2 | 8 | Close supervision by supervisor | 4 | 1 | 4 | Supervisor on site Ibrahim / Zahari | 19/12/20 |  |
| 2 | Lift up machine onto stand | Failing of lifting equipment | Bodily injury & damage to property | - All lifting equipment use are tested by approved person and has valid certificate - To ensure lifting gears use are in compliance with weight of machine - No lifting beyond SWL of equipment - Ensure no unauthorised personnel is below or near the load when being lift up | 4 | 2 | 8 | When lift up machine, try not to jerk the load | 4 | 1 | 4 | Supervisor on site Ibrahim / Zahari | 19/12/20 |  |
|  |  | Struck by load due to unsafe slinging or rigging | Bodily injury & damage to property | - Entire crew is trained and to have relevant skill and experience to carry out task - Load must be securely slug and properly balanced to prevent from dislodgment - To ensure load is rig at the centre of gravity | 4 | 2 | 8 | Close supervision by supervisor | 4 | 1 | 4 | Supervisor on site Ibrahim / Zahari | 19/12/20 |  |
|  |  | Manual pulling of chain block | Muscular strain | - Communicate between workers ensuring everyone is clear of task - Supervisor to ensure there are adequate manpower to be deploy for the job - Buddy system-rotating of workers when pulling the chain block | 3 | 2 | 6 | Supervisor to be responsible for giving instruction to ensure load is being pull up slowly and balanced well | 3 | 1 | 3 | Supervisor on site Ibrahim / Zahari | 19/12/20 |  |