Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 23 Changi North Crescent | Dismantling jig crane | 1 | Working at height to assist engineer dismantle jig crane |  |
|  |  |  | 2 | Forklift to fork down load |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Dismantling jig crane | RA Member: Zailani |  |  |  |
| Location: | 23 Changi North Crescent UMS | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2023 |  | Designation: | Director |  |
| Next Review Date | 30/06/2026 |  | Date: | 03/11/2025 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Working at height to assist engineer dismantle jig crane | Fall from height | Bodily Injury | - Provision of boom lift as working platform to work at height - Only certified and competent boom lift operator are allowed to operate the boom lift - Worker working at height on top boom lift to put on proper PPE (safety harness, safety helmet) - Ensure safety harness use is in good working condition, properly strap on and hook to strong anchorage point of the boom lift at all time | 4 | 2 | 8 | All trained and authorised operator record to be paste at the boom lift | 4 | 1 | 4 | Supervisor on site | 08/12/25 |  |
| 2 | Forklift to fork down load | Falling load | Bodily injury & damage to property | - Only certified and competent forklift driver are allowed to operate the forklift - Known the CG of the load - Ensure forklift fork are properly aligned directly to the CG of load - Ensure load is well balanced on the fork - Elevate the load slightly and fully tilt the mast backward - To ensure no unauthorised personnel is within the working area | 4 | 2 | 8 | Use extension fork when needed to give greater stability of load | 4 | 1 | 4 | Supervisor on site | 08/12/25 |  |
|  |  | Hitting people or property | Bodily injury & damage to property | - Only certified and competent forklift driver are allowed to operate the forklift - Operator advised to be careful when moving forward or reversing - Ensure forklift driver is guided by other staff - To ensure no unauthorised personnel is within the working area | 4 | 2 | 8 | Press horn when necessary to warn others that forklift is approaching | 4 | 1 | 4 | Supervisor on site | 08/12/25 |  |