Inventory of Work Activities

| Company: Eric Wong Machinery Transportation Pte Ltd |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 273 Jalan Ahmad Ibrahim | Forklift operation with Extension boom | 1 | Transport of lifting machinery into or out of site premises |  |
|  |  |  | 2 | Lifting operation using forklift |  |
|  |  |  | 3 | Unloading of Spindle head onto Pallet |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Lorry Loader / Forklift Operation | RA Member: Zailani |  |  |  |
| Location: | 273 Jalan Ahmad Ibrahim / Baker Hudges | RA Member: Zaharin |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2017 |  | Designation: | Director |  |
| Next Review Date | 30/06/2020 |  | Date: | 30/05/2018 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Transport of lifting machinery into or out of site premises | Collision of moving vehicle onto personnel’s or site property | Bodily Injury & Damage to Property | - Vehicle to be move into site with the aid of bank’s man guiding the driver - Clear all obstruction in way of moving path - Drive at slow speed when entering into customer premise | 4 | 2 | 8 | Vehicle must not be left unattended while engine is running | 4 | 1 | 4 | Supervisor on site | 30/05/18 |  |
| 2 | Lifting operation using forklift | Failing of lifting equipment | Bodily Injury & Damage to Property | - Operator trained and competent person appointed - All lifting equipment are tested by approved person and has valid certificate - Riggers to ensure lifting gears use are within working load - No defective gears allowed - Never stand close or under the load | 4 | 2 | 8 | While moving or slewing the crane, try not to jerk the load | 4 | 1 | 4 | Lifting supervisor on site | 30/05/18 |  |
|  |  | Overloading | Bodily Injury & Damage to Property | - Load weight should be known | 4 | 2 | 8 | Ensure load, capacity of lifting arm and reach are within limit. | 4 | 1 | 4 | Lifting supervisor on site | 30/05/18 |  |
|  |  | Struck by object due to unsafe slinging, rigging | Bodily Injury & Damage to Property | - Entire lifting crew is trained and to have relevant skill, certificate and experiences required to carry out tasks - Load shall be securely slug, properly balanced and trimmed to prevent from dislodgment - Ensure load is rig at the centre of gravity | 4 | 2 | 8 | Communication through hand signal or via walkie-talkie by Eric Wong personnel only | 4 | 1 | 4 | Lifting supervisor on site | 30/05/18 |  |
| 3 | Unloading of Spindle head onto Pallet | Falling load | Bodily Injury & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Known the CG of the load - Elevate the load slightly and fully tilt the mast backward - Ensure machine load are spread evenly and is stable on the fork before driving off - Ensure no unauthorised personnel within the working zone | 4 | 2 | 8 | Use extension fork/boom if needed to give greater stability of load | 4 | 1 | 4 | Supervisor on site | 30/05/18 |  |
|  |  | Hitting people or property | Bodily Injury & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Operator advised to be careful when moving forward or reversing - Ensure working area is clear of unauthorised personnel - Ensure forklift driver is guided by other staff - Press horn if necessary to warn others that forklift is approaching | 4 | 2 | 8 | Assignment of traffic controller to control movement of vehicle and people within the vicinity | 4 | 1 | 4 | Supervisor on site | 30/05/18 |  |
|  |  | Restricted driver visibility | Bodily Injury & Damage to Property | - Forklift equipped with side mirror and reverse warning alarm to cautioned nearby personnel - Ensure forklift driver is guided by other staff - Drive at slow transportation speed | 3 | 2 | 6 | Assignment of traffic controller | 3 | 1 | 3 | Supervisor on site | 30/05/18 |  |