Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 25 A.M.K St 65 | Fixing ‘A’ frame structure | 1 | Fixing of ‘A’ frame structure |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by : Eric Wong Signature: | Reference Number: |
| --- | --- | --- | --- | --- |
| Process: | Fixing ‘A’ Frame structure | RA Member: Zailani |  |  |
| Activity Location: | 25 A.M.K St 65 Sanden International | RA Member: Daniel Wong |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: Eric Wong |  |
| Last Review Date: | 01/07/2020 |  | Designation: Director |  |
| Next Review Date | 30/06/2023 |  | Date: 13/05/2022 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Fixing of ‘A’ frame | Manual handling of heavy equipment | Sprain back when carrying heavy load | - Adopt proper technique when carrying out task - Ensure sufficient rest and recovery time is taken during repeated lift - Supervisor to ensure there are adequate manpower to be deploy for the job - Apply buddy system when handling of heavy equipment - PPE (safety helmet, safety shoe) provided and put on | 3 | 2 | 6 | Use of mechanical aid such as lifter if require | 3 | 1 | 3 | Supervisor on site | 17/05/22 |  |
|  |  | Working at height to fix bolt and nut onto frame | Falling from height | - Use of ‘A’ frame ladder to work on which is unreachable from ground - Ladder use must be fully extended and stand on level ground - Apply buddy system- one to hold onto the ladder while the other is working on top of ladder | 4 | 2 | 8 | Close supervision by supervisor | 4 | 1 | 4 | Supervisor on site | 17/05/22 |  |