Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 39 Changi North Crescent | Forklift operation | 1 | Operating of forklift into factory premise |  |
|  |  |  | 2 | Securing of sling onto equipment using ladder |  |
|  |  |  | 3 | Forklift operation – to lift up equipment |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Forklift Operation | RA Member: Zailani |  |  |  |
| Location: | 39 Changi North Crescent Collins Goodrich Aerospace | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2023 |  | Designation: | Director |  |
| Next Review Date | 30/06/2026 |  | Date: | 23/11/2023 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Operating of forklift into factory premise | Hitting people or property | Bodily injury & damage to property | - Only certified and competent forklift lift operator are allowed to operate the forklift - Operator advised to be careful when moving forward or reversing - To ensure no unauthorised personnel is within the working area - To clear off all material or tooling that is obstructing the work area - Proper PPE to be put when entering the site | 4 | 2 | 8 | Slow moving speed | 4 | 1 | 4 | Supervisor on site | 05/12/23 |  |
| 2 | Securing of sling onto equipment using ladder | Fall from height | Bodily Injury | - Use ‘A’ frame ladder to work on which is unreachable from ground - Ladder use must be fully extended and stand on ground level - Buddy system-one to hold the ladder while the other is working on the ladder | 4 | 2 | 8 | Close supervision and reminder by supervisor | 4 | 1 | 4 | Supervisor on site | 05/12/23 |  |
| 3 | Forklift operation – to lift up equipment | Falling load | Bodily Injury | - Only certified and competent forklift operator are allowed to operate the forklift - Ensure load is rig at the centre of gravity - Known the CG of the load - Elevate the load slightly and fully tilt the mast backward | 4 | 2 | 8 | Use of extension fork when needed | 4 | 1 | 4 | Supervisor on site | 05/12/23 |  |
|  |  | Hitting people or property | Bodily Injury & damage to property | - Only certified and competent forklift driver are allowed to operate the forklift - Operator advised to be careful when moving forward or reversing - To ensure working area is clear of unauthorised personnel - Ensure forklift operator is guided by other stuff | 4 | 2 | 8 | Press horn if necessary to warn others | 4 | 1 | 4 | Supervisor on site | 05/12/23 |  |