Inventory of Work Activities

| Company: Eric Wong Machinery Transportation Pte Ltd |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 2 Seletar Aerospace Link | Forklift operation | 1 | Using forklift to fork up machine to mezzanine floor |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Forklift Operation | RA Member: Zailani |  |  |  |
| Location: | 2 Seletar Aerospace Link Avimac Pte Ltd | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2023 |  | Designation: | Director |  |
| Next Review Date | 30/06/2026 |  | Date: | 26/08/2025 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Using forklift to fork up machine to mezzanine floor | Falling load | Bodily Injury & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Known the CG of the load - Worker to guide forklift driver to ensure forklift fork is properly adjust and aligned directly to the lifting point of machine - Elevate the load slightly and fully tilt the mast backward - To ensure machine load are spread evenly and balanced well on the fork before proceeding to fork up machine to 2nd floor - Ensure proper means of signal is given to forklift driver when reaching 2nd floor as when to lift up and lower down the fork - To avoid sudden jerk of the load - To ensure no unauthorised personnel is under or within the working area | 4 | 2 | 8 | Use extension fork if needed to give greater stability of load | 4 | 1 | 4 | Supervisor on site | 29/09/25 |  |
|  |  | Hitting people or property | Bodily Injury & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Drive at slow speed - Forklift driver advised to be careful when moving forward or reversing - Ensure forklift driver is guided by other stuff - To ensure no unauthorised personnel is within the working area | 4 | 2 | 8 | Assign worker to controller and guide movement around the operation area | 4 | 1 | 4 | Supervisor on site | 29/09/25 |  |