Inventory of Work Activities

| Company: Eric Wong Machinery Transportation Pte Ltd |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 1 Engineering Dr 2 | Forklift operation | 1 | Unloading of case from lorry chassis |  |
|  |  |  | 2 | Un-crate case |  |
|  |  |  | 3 | Fork up machine to 2nd floor |  |
|  |  |  | 4 | Transportation of machine to designated location |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Forklift Operation | RA Member: Zailani |  |  |  |
| Location: | NUS, 1 Engineering Drive 2 Department of Civil & Environmental Engineering, Block EW1 | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2020 |  | Designation: | Director |  |
| Next Review Date | 30/06/2023 |  | Date: | 28/01/2021 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Using forklift to unload case from lorry chassis | Falling load | Bodily Injury & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Known the CG of the load - Elevate the load slightly and fully tilt the mast backward - Ensure no unauthorised personnel within the working zone | 4 | 2 | 8 | Use extension fork if needed to give greater stability of load | 4 | 1 | 4 | Supervisor on site | 02/02/21 |  |
|  |  | Hitting people or property | Bodily Injury & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Operator advised to be careful when moving forward or reversing - Ensure working area is clear of unauthorised personnel or any obstruction | 4 | 2 | 8 | Assignment of traffic controller to control movement of vehicle and personnel within the vicinity | 4 | 1 | 4 | Supervisor on site | 02/02/21 |  |
| 2 | Un-crate case | Falling of wooden plank | Bodily Injury | - PPE (safety shoe, safety helmet) provided and put on - Uncrate and dismantle case in the correct sequence - Remove immediately any loose plank - Proper housekeeping to be maintained during work - Ensure no unauthorised personnel near working area | 3 | 2 | 6 | Good coordination between workers | 3 | 1 | 3 | Supervisor on site | 02/02/21 |  |
|  |  | Protruding nails on loose timber | Bodily Injury | - PPE (safety shoe, safety helmet) provided and put on - Remove immediately any loose timber with protruding nails from working area - Proper housekeeping to be maintained during work | 3 | 2 | 6 | Close supervision and regular reminder by supervisor | 3 | 1 | 3 | Supervisor on site | 02/02/21 |  |
| 3 | Fork up equipment to 2nd floor | Falling load | Bodily Injury & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Known the CG of the load - To ensure equipment is secured firmly with strapping belt and balanced well on the fork before proceeding to fork up to 2nd floor - Elevate the load slightly and fully tilt the mast backward - To avoid sudden jerk of the load - Supervisor to ensure no unauthorised personnel is below the load or within the working zone | 4 | 2 | 8 | Use extension fork if needed to give greater stability of load | 4 | 1 | 4 | Supervisor on site | 02/02/21 |  |
|  |  | Restricted driver visibility | Bodily Injury & Damage to Property | - Ensure forklift driver is guided by other stuff when required - Drive at slow speed - Forklift driver advised to be careful when moving forward or reversing | 3 | 2 | 6 | Ensure proper means of signal is given to forklift driver when reaching 2nd floor as when to lift up and lower down the equipment onto woodblock | 3 | 1 | 3 | Supervisor on site | 02/02/21 |  |
| 4 | Transport of equipment to designated location | Unstable load during moving or turning | Bodily Injury & Damage to Property if machine topper | - Ensure pallet truck are place accordingly on equipment base - Ensure equipment load are spread evenly and balanced well on the pallet truck before being transport to location - Workers to be alert and keep close lookout on route of travel from point to point | 4 | 2 | 8 | Slow moving speed | 4 | 1 | 4 | Supervisor on site | 02/02/21 |  |
|  |  | Passage way block by tooling / materials | Trip & Fall | - Passage way and working area to be keep clear of tooling/materials before moving job - Proper housekeeping to be maintained during work - Oil/water spill must be clean up immediately | 2 | 2 | 4 | Supervisor to make sure route of transporting before moving and to double check clearing of all materials and tooling | 2 | 1 | 2 | Supervisor on site | 02/02/21 |  |
|  |  | Obstruction to vision while moving | Damage to Property when hitting other obstacles | - To keep close lookout on route of travel - Ensure movement is guided - To alert worker controlling the pallet truck steering if potential hitting of others obstacles | 2 | 2 | 4 | Provision of spotter on both side of machine so as to have a clearer view before hitting any obstacles | 2 | 1 | 2 | Supervisor on site | 02/02/21 |  |