Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | No 12 Penjuru Lane | Installation & lifting using gantry crane | 1 | Lay out I-beam onto gantry crane structure using overhead crane |  |
|  |  |  | 2 | Working at height to tighten nuts |  |
|  |  |  | 3 | Gantry crane to lift up machine and load onto cometto |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader : Kevin Wong | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Hoisting using gantry crane | RA Member : Zailani |  |  |  |
| Location: | No 12 Penjuru Lane AEI Corporation Ltd | RA Member :Zaharin |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 15/05/2015 |  | Designation: | Director |  |
| Next Review Date | 15/05/2018 |  | Date: | 26/09/2016 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Fixed I-beam onto gantry structure | Load topper | Bodily Injury & Damage Property | - Ensure beam is securely slug, properly balanced and trimmed to prevent from dislodgment - Ensure beam is rig at it centre of gravity - Never stand close or under the suspended load - PPE (etc.. safety helmet, safety shoe) provided and put on | 4 | 2 | 6 | Workers to be briefed before commencement of work | 4 | 2 | 6 | Lifting Supervisor on site | TBA |  |
| 2 | Working at height to tighten nuts | Falling from height | Bodily Injury & Damage to Site Property | - Use ‘A’ frame ladder to work on which is unreachable from ground - Ensure ladder must be fully extended and stand on ground level - Ensure buddy system – one to hold onto the ladder while the other is working on the ladder - If needed, use of safety harness with short lanyard and anchored it to strong anchorage point at time | 4 | 2 | 8 | Provision of boom lift as working platform if needed | 4 | 2 | 8 | Lifting Supervisor on site | TBA |  |
| 3 | Gantry crane to lift up machine and load onto cometto | Failing of lifting equipment | Bodily Injury & Damage to Property | - All lifting equipment are tested by approved person and has valid certificate - Riggers to ensure lifting gears use are in compliance with weight of machine - Ensure hoist connected to gantry crane are in good working condition and properly secured - Only skill personnel are allowed to operate the gantry crane - Load to be lift up at slow speed - Never stand close or under the suspended load - Ensure operation area are clear of unauthorised personnel | 4 | 2 | 8 | While being lift up, try not to jerk the load | 4 | 2 | 8 | Lifting Supervisor on site | TBA |  |
|  |  | Struck by objects due to unsafe slinging, rigging | Bodily Injury & Damage to Property | - Entire lifting crew is trained and to have relevant skill, certificate and experiences required to carry out tasks - Load shall be securely slug, properly balanced and trimmed to prevent dislodgment - Ensure load is rig at it centre of gravity | 4 | 2 | 8 | Communication through hand signal or via walkie-talkie by Eric Wong personnel only | 4 | 2 | 8 | Lifting Supervisor on site | TBA |  |
|  |  | Hand caught by or in between loose part of sling or lifting gears | Arm or Hand Injury | - Keep hands off loose part of sling or lifting gears - Glove to be provided and worn | 3 | 2 | 6 | Good coordination between workers | 3 | 2 | 6 | Lifting Supervisor on site |  |  |
|  |  | View obstruction or load swing | Bodily Injury & Damage to Property | - Appointed and trained signalman, riggers and lifting supervisor to on site - Clear all obstruction and unauthorised personnel in way of lifting path - Tagline for load when necessary to prevent of load swing - Never stand close or under the suspended load and away from possible swing path | 4 | 2 | 8 |  | 4 | 2 | 8 | Lifting Supervisor on site | TBA |  |