Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 8 Tuas West Ave | Un-crate/moving operation | 1 | Un-crate case |  |
|  |  |  | 2 | Jack up machine base |  |
|  |  |  | 3 | Transportation of machine to location |  |

| Company : | Eric Wong Machinery Tptn Pte Ltd | RA Leader : Kevin Wong | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Un-crate Case/Moving Operation | RA Member : Zailani |  |  |  |
| Location: | No 8 Tuas West Ave Hunting Energy Sevices Pte Ltd | RA Member :Zaharin |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 15/05/2015 |  | Designation: | Director |  |
| Next Review Date | 15/05/2018 |  | Date: | 11/07/2017 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Un-crate case | Sharp edges/ wooden splinters | Cut | - Keep hand and finger clear of sharp edges - Remove sharp edges (etc...nails, metal strips) from working area as soon as possible - PPE (etc.. safety shoe, safety helmet, glove) provided and put on | 2 | 3 | 6 | Close supervision by supervisor | 2 | 2 | 4 | Supervisor on site | 13/07/17 |  |
|  |  | Falling of wooden plank/timber | Bodily Injury | - PPE (etc.. safety shoe, safety helmet) provided and put on - Un-crate and dismantle case in the correct sequence - Remove immediately any loose plank/timber - Proper housekeeping to be maintained during work | 3 | 2 | 6 | Good coordination between workers | 3 | 1 | 3 | Supervisor on site | 13/07/17 |  |
|  |  | Working at height | Falling from height | - Use ‘A’ frame ladder to work on which is unreachable from ground - Ensure ladder must be fully extended and stand on ground level - Buddy system-one to hold onto the ladder while the other is working on top of ladder - Use of safety harness with short lanyard and anchored it to strong anchorage point | 4 | 2 | 8 | Provision of boom lift as working platform if required | 4 | 1 | 4 | Supervisor on site | 13/07/17 |  |
| 2 | Jack up machine using manual / hydraulic jack | Machine topper due to lost of balance upon lifted up | Bodily Injury & Damage to Property | - Worker to be alert when jack up machine - Ensure machine are jack up at the right angle of gravity - Jack up machine from one side to another is carried out in stages with the use of hard wooden block | 4 | 2 | 8 | Only skill and experience worker to control the jack handling | 4 | 1 | 4 | Supervisor on site | 13/07/17 |  |
| 3 | Transportation of machine to designated location | Unstable load while moving | Bodily Injury & Damage to Property | - Ensure rollers are place accordingly on machine base - Ensure machine load are spread evenly on rollers for stability before being transport to location - Workers to be alert and keep close lookout on route of travel from point to point | 3 | 2 | 6 | Slow moving speed | 3 | 1 | 3 | Supervisor on site | 13/07/17 |  |
|  |  | Excessive weight load | Sprain back | - Use the correct capacity rollers to reduce the effort of each man when pushing the load - Deploy adequate personnel | 3 | 2 | 6 | Use manual puller or forklift if possible to assist in pushing the load | 3 | 1 | 3 | Supervisor on site | 13/07/17 |  |
|  |  | Passageway block by material and tools | Trip & Fall | - Passageway and working area to be keep clear of material and tooling before moving job - Proper housekeeping to be maintained during work - Oil/water spill must be clean up immediately | 2 | 2 | 4 | Supervisor to make sure route of transporting before moving and double check clearing of all materials or tooling | 2 | 1 | 2 | Supervisor on site | 13/07/17 |  |
|  |  | Obstruction to vision while moving | Damage to Property | -To keep close lookout on route of travel - Ensure movement is guided - To alert worker controlling the rollers steering if potential hitting of other obstacles | 3 | 2 | 6 | Provision of spotter on both end of machine side so as to have a clearer view before it touches any obstacles | 3 | 1 | 3 | Supervisor on site | 13/07/17 |  |