Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 51 Corporation Rd | Gantry crane operation | 1 | Insufficient working space |  |
|  |  |  | 2 | Fixing beam onto gantry structure |  |
|  |  |  | 3 | Working at height to tighten nuts |  |
|  |  |  | 4 | Lift up machine column |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Gantry crane operation | RA Member: Zailani |  |  |  |
| Location: | 51 Corporation Rd Avitron Pte Ltd | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2020 |  | Designation: | Director |  |
| Next Review Date | 30/06/2023 |  | Date: | 22/03/2023 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Insufficient working space | Hitting personnel or property | Bodily Injury & Damage to Property | - To ensure there is enough working space for setting up of gantry crane - To keep working area clear with no object or unauthorised personnel obstructing the pathway | 4 | 2 | 8 | Close supervision by supervisor | 4 | 2 | 8 | Supervisor on site | 24/03/23 |  |
| 2 | Fixing beam onto gantry crane using overhead crane | Load topple | Bodily Injury & Damage to Property | - To ensure load is stable and balanced well while being lift up - To avoid sudden jerk of the load - Supervisor to ensure no unauthorised personnel is under the suspended load - PPE (safety helmet, safety shoe) provided and put | 4 | 2 | 8 | Workers are to be briefed before commencement of work | 4 | 2 | 8 | Supervisor on site | 24/03/23 |  |
| 3 | Working at height to tighten nuts | Falling from height | Bodily Injury | - Use ‘A’ frame ladder to work on which is unreachable from ground - Ensure ladder must be fully extended and stand on ground level - Buddy system/one to hold onto the ladder while the other is working on the ladder - Use of safety harness with short lanyard and anchored it to strong anchorage point at all time | 4 | 2 | 8 | Provision of scissor lift as working platform if required | 4 | 1 | 4 | Lifting supervisor on site | 24/03/23 |  |
| 4 | Lift up machine column | Falling of lifting equipment | Bodily Injury & Damage to Property | - Ensure hoist connected to gantry crane is in good condition and properly secured - Only experience and skill personnel are allowed to operate the gantry crane - Riggers to ensure lifting gears use are in compliance with weight on machine - Load to be lift up at slow speed - Never stand close or under the suspended load | 4 | 2 | 8 | While being lift up, try not to jerk the load | 4 | 1 | 4 | Lifting supervisor on site | 24/03/23 |  |
|  |  | Overloading | Bodily Injury & Damage to Property | - No lifting beyond SWL of equipment - Load weight should be known | 4 | 2 | 8 | Ensure load, capacity of lifting equipment are within limit, when in doubt, always use a higher capacity crane | 4 | 1 | 4 | Lifting supervisor on site | 24/03/23 |  |
|  |  | Struck by objects due to unsafe slinging, rigging | Bodily Injury & Damage to Property | - Entire lifting crew is trained and to have relevant skill, certificate and experiences required to carry out tasks - Load shall be securely slug, properly balanced and trimmed to prevent from dislodgment - Ensure load is rig up at slow pace | 4 | 2 | 8 | Use of chain block to perform stable lifting when required | 4 | 1 | 4 | Lifting supervisor on site | 24/03/23 |  |