Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  |  | Lift up machine part using hydraulic gantry crane | 1 | Hoist down gantry crane from low-bed using lorry loader |  |
|  |  |  | 2 | Fixing I-beam onto gantry crane structure |  |
|  |  |  | 3 | Working at height to tighten nuts and lock beam onto gantry crane |  |
|  |  |  | 4 | Gantry crane to lift up machine parts |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Lift up machine part using hydraulic gantry crane | RA Member: Zailani |  |  |  |
| Location: |  | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2020 |  | Designation: | Director |  |
| Next Review Date | 30/06/2023 |  | Date: | 16/03/2022 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Hoist down gantry crane from low-bed chassis using lorry loader | Failing of lifting equipment | Bodily Injury & Damage to Property | - Trained and competent lorry loader operator appointed - All lifting equipment use are tested by approved person and has valid certificate - Riggers to ensure lifting gears use are in compliance with weight of load to be to be lifted - No defective gears allowed - Never stand close or under the load - PPE (safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | While slewing the crane, try not to jerk the load | 4 | 1 | 4 | Wong Yen Siong |  |  |
| 2 | Fixing I-beam onto gantry crane structure | Load topple | Bodily Injury & Damage to Property | - To ensure I-beam is securely slug, properly balanced and trimmed to prevent load from dislodgment - To ensure I-beam is rig at it centre of gravity - To avoid sudden jerk of the load - Supervisor to ensure no unauthorised personnel is near or under the suspended load - PPE (safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | Workers are to be briefed before commencement of work | 4 | 1 | 4 | Wong Yen Siong |  |  |
| 3 | Working at height to tighten nuts and lock beam onto gantry crane | Falling from height | Bodily Injury | - Use ‘A’ frame ladder to work on which is unreachable from ground - Ensure ladder must be fully extended and stand on ground level - Buddy system/one to hold onto the ladder while the other is working on the ladder - Use of safety harness with short lanyard and anchored it to strong anchorage point at all time | 4 | 2 | 8 | Provision of scissor lift as working platform if required | 4 | 1 | 4 | Wong Yen Siong |  |  |
| 4 | Gantry crane to lift up machine parts | Failing of lifting equipment | Bodily Injury & Damage to Property | - Appointed and trained lifting supervisor, rigger and signalman to be on site - All lifting equipment are tested by approved person and has valid certificate - Riggers to ensure lifting gears use are in compliance with weight od machine - Ensure hoist connected to gantry crane are in good working condition and properly secured - Only experienced and skill personnel are allowed to operate the gantry crane - Load to be lift up at slow speed - Never stand close or under the suspended load - Ensure operation area are clear of unauthorised personnel | 4 | 2 | 8 | While being lower down, try not to jerk the load | 4 | 1 | 4 | Wong Yen Siong |  |  |
|  |  | Overloading | Bodily Injury & Damage to Property | - No lifting beyond SWL of equipment - Load weight should be known | 4 | 2 | 8 | Ensure load, capacity of lifting equipment are within limit, when in doubt, always use a higher capacity equipment | 4 | 1 | 4 | Wong Yen Siong |  |  |
|  |  | Struck by objects due to unsafe slinging, rigging | Bodily Injury & Damage to Property | - Entire lifting crew is trained and have relevant skill, certificate and experiences require to carry out the tasks - Load shall be securely slug, properly balanced and trimmed to prevent from dislodgment - Ensure load is rig at it centre of gravity - Riggers to check and ensure safe rigging and lifting supervisor to confirm load are safe to rig | 4 | 2 | 8 | Use of chain block to perform stable lifting when required | 4 | 1 | 4 | Wong Yen Siong |  |  |
|  |  | Hand caught by or in between loose parts of sling or lifting gears | Arm or Hand Injury | - Keep hands off loose part of sling or lifting gears - Ensure proper signal are given to gantry crane personnel - Glove to be provided and use | 3 | 2 | 6 | Good coordination between workers | 3 | 1 | 3 | Wong Yen Siong |  |  |