Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 8 Tractor Rd | Manual lifter operation | 1 | Using manual lifter to lift down/up polishing plate |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Manual lifter operation | RA Member: Zailani |  |  |  |
| Location: | 8 Tractor Rd Excelitas Technologies (S) Pte Ltd | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2020 |  | Designation: | Director |  |
| Next Review Date | 30/06/2023 |  | Date: | 06/04/2021 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Using manual lifter to lift down/up polishing plate | Falling load | Bodily Injury & Damage to Property | - Known the CG of the load - Supervisor to ensure plate is well balanced on the lifter fork - To avoid sudden jerk of the load while being lifted up/down - To lower down/up plate at slow speed - Ensure no unauthorised personnel is near or within the working area - Supervisor to ensure there are adequate personnel to be deploy for the job | 4 | 2 | 8 | Close supervision by supervisor to ensure all control and precaution are in place | 4 | 1 | 4 | Supervisor on site | TBA |  |
|  |  | Overloading of load | Damage to Property | - Load weigh should be known - Ensure polishing plate to be lifted up/down is within the lifting capacity of the manual lifter, if in doubt, use a higher capacity lifter | 3 | 2 | 6 | While being lift up or down the plate, try not to jerk the load | 3 | 1 | 3 | Supervisor on site | TBA |  |