Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 164 Gul Circle | Lift up structure using chain block | 1 | Lift up structure using chain block |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by : Eric Wong Signature: | Reference Number: |
| --- | --- | --- | --- | --- |
| Process: | Lift up structure using chain block | RA Member: Zailani |  |  |
| Activity Location: | 164 Gul Circle | RA Member: Daniel Wong |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: Eric Wong |  |
| Last Review Date: | 01/07/2020 |  | Designation: Director |  |
| Next Review Date | 30/06/2023 |  | Date: 13/05/2022 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 2 | Lift up structure | Failing of anchorage point of lifting equipment | Bodily injury & damage to property | - Customer to ensure anchorage point of the chain block is able to handle the structure weight when being pull up - All lifting equipment use are tested by approved person and has valid certificate - To ensure lifting gears use are in compliance with weight of load - No lifting beyond SWL of equipment - Ensure no unauthorised personnel is below or near the load when being lift up | 4 | 2 | 8 | When lift up the structure, try not to jerk the load | 4 | 1 | 4 | Supervisor on site |  |  |
|  |  | Struck by load due to unsafe slinging or rigging | Bodily injury & damage to property | - Entire crew is trained and to have relevant skill and experience to carry out task - Load must be securely slug and properly balanced to prevent from dislodgment - To ensure load is rig at the centre of gravity | 4 | 2 | 8 | Close supervision by supervisor | 4 | 1 | 4 | Supervisor on site |  |  |
|  |  | Manual pulling of chain block | Muscular strain | - Communicate between workers ensuring everyone is clear of task - Supervisor to ensure there are adequate manpower to be deploy for the job - Buddy system-rotating of workers when pulling the chain block to required height | 3 | 2 | 6 | Supervisor to be responsible for giving instruction to ensure load is being pull up slowly and balanced well | 3 | 1 | 3 | Supervisor on site |  |  |