Inventory of Work Activities

| Company: Eric Wong Machinery Transportation Pte Ltd |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 100 Woodlands Ave 7 | Forklift & moving operation | 1 | Transport of lifting machinery into or out of site premises |  |
|  |  |  | 2 | Unloading of machine using 16ton forklift and transport to loading bay |  |
|  |  |  | 3 | Transportation of machine to designated location using rollers |  |
|  |  |  | 4 | Unload machine from rollers using jack |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Forklift / Moving Operation | RA Member: Zailani |  |  |  |
| Location: | 100 Woodlands Ave 7 3M Singapore Pte Ltd | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2020 |  | Designation: | Director |  |
| Next Review Date | 30/06/2023 |  | Date: | 13/12/2022 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Transport of lifting machinery into or out of site premises | Collision of moving vehicle onto personnel’s or site property | Multiple Injuries & Damage to Property | - Vehicle only to be moved into site with the aid of banks man guiding the driver - To clear all obstruction in way of moving path - Drive at slow speed when entering into customer premise | 4 | 2 | 8 | Vehicle must not be left unattended while engine is running | 4 | 1 | 4 | Supervisor on site | 07/01/23 |  |
| 2 | Unloading of machine using 16ton forklift and transport to loading bay | Falling load | Multiple Injuries & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Known the CG of the load - Ensure load weight are spread evenly and balanced well on the fork - Elevate the load slightly and fully tilt the mast backward - Ensure no unauthorised personnel within the working zone | 4 | 2 | 8 | Use extension fork if needed to give greater stability of load | 4 | 1 | 4 | Supervisor on site | 07/01/23 |  |
|  |  | Hitting people or property | Multiple Injuries & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Operator advised to be careful when moving forward or reversing - Ensure forklift driver is guided by other staff - Ensure working area is clear of unauthorised personnel or any obstruction | 4 | 2 | 8 | Assignment of traffic controller to control movement of vehicle and personnel within the vicinity | 4 | 1 | 4 | Supervisor on site | 07/01/23 |  |
|  |  | Restricted driver visibility | Multiple Injuries & Damage to Property | - Forklift driver advise to be careful when moving forward or reversing - Ensure forklift driver is guided by other staff - Press horn if necessary to warn others that forklift is approaching - If required, reverse driving with reverse light on - Drive at slow transportation speed | 4 | 2 | 8 | Assignment of traffic controller to control movement of vehicle and personnel within the vicinity | 4 | 1 | 4 | Supervisor on site | 07/01/23 |  |
| 3 | Transport machine to designated location using rollers | Unstable load during moving or turning | Multiple Injuries & Damage to Property if machine topper | - Ensure rollers are place accordingly on machine bed - Ensure rollers are properly secured to machine bed with bolt and nuts - Tighten bolts and nuts with adjustable spanner - Ensure machine load are spread evenly and balanced well on the rollers before being transport to location - Workers to be alert and keep close lookout on route of travel from point to point and to ensure rollers are not moving out of position | 4 | 2 | 8 | Slow moving speed | 4 | 1 | 4 | Supervisor on site | 07/01/23 |  |
|  |  | Excessive weight load | Sprain back if force moving the load | - Use the correct capacity rollers to ensure roller wheel and bearing can sustained the weight of load - Supervisor to ensure there are adequate personnel to be deploy for the job - Use of waist belt if needed to prevent sprain back or backache - To ensure travelling ground is well protected with canvas/metal plate in contact with the rollers at all time | 3 | 2 | 6 | Use machine puller to assist in pulling the machine when required | 3 | 1 | 3 | Supervisor on site | 07/01/23 |  |
|  |  | Trip & Fall | Sprains & strains | - Passage way and working area to be keep clear of tooling and materials before moving job - Proper housekeeping to be maintained during work - Keep the floor in the work area dry and clean - Oil/water spill must be clean up immediately - Tape down any power cords or cable if any to prevent from tripping | 2 | 2 | 4 | Supervisor to make sure route of transporting before moving and to double check clearing of all materials and tooling | 2 | 1 | 2 | Supervisor on site | 07/01/23 |  |
| 4 | Unload machine from rollers using jack | Machine topple due to lost of balanced upon lifted up | Multiple Injuries & Damage to Property | - Workers to be alert when jack up machine - Ensure machine are jack up at the right angle of gravity - Jack up machine to 4” high from one side to another is carried out in stages with the use of solid woodblock to be insert below machine bed - Supervisor to ensure operation area is clear of unauthorised personnel | 4 | 2 | 8 | Only skill and experienced worker to control the jack handling | 4 | 1 | 4 | Supervisor on site | 07/01/23 |  |