Inventory of Work Activities

| Company: Eric Wong Machinery Transportation Pte Ltd |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 51 Seletar Aerospace View | Forklift / moving operation | 1 | Unloading of container using 2 x 10 ton forklift |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Forklift Operation | RA Member: Zailani |  |  |  |
| Location: | 51 Seletar Aerospace View Pratt & Whitney NGPF Manufacturing Company (S) Pte Ltd | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2017 |  | Designation: | Director |  |
| Next Review Date | 30/06/2020 |  | Date: | 30/12/2019 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Unloading of container using 2 x 10 ton forklift | Falling load | Bodily Injury & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Known the CG of the load - Elevate the load slightly and fully tilt the mast backward - Ensure proper command is given to both forklift driver to ensure container when being fork up or lower down remain stable on the fork - Ensure no unauthorised personnel is within the working zone | 4 | 2 | 8 | Use extension fork if needed to give greater stability of load | 4 | 1 | 4 | Supervisor on site | 31/12/19 |  |
|  |  | Hitting people or property | Bodily Injury & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Operator advised to be careful when moving forward or reversing - Ensure working area is clear of unauthorised personnel or any obstruction | 4 | 2 | 8 | Assignment of traffic controller to control movement of vehicle and personnel within the vicinity | 4 | 1 | 4 | Supervisor on site | 31/12/19 |  |
|  |  | Restricted driver visibility | Bodily Injury & Damage to Property | - Forklift driver advise to be careful when moving forward or reversing - Ensure forklift driver is guided by other staff - Press horn if necessary to warn others that forklift is approaching - If required, reverse driving with reverse light on - Drive at slow transportation speed | 3 | 2 | 6 | Assignment of traffic controller to control movement of vehicle and personnel within the vicinity | 3 | 1 | 3 | Supervisor on site | 31/12/19 |  |