Inventory of Work Activities

| Company: Eric Wong Machinery Transportation Pte Ltd |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 249 Jalan Boon Lay | Forklift & moving operation | 1 | Transport of lifting machinery into or out of site premises |  |
|  |  |  | 2 | Unloading of case from container using 7ton forklift |  |
|  |  |  | 3 | Uncrate case |  |
|  |  |  | 4 | Fork up machine from base and transport to door entrance |  |
|  |  |  | 5 | Insert rollers under machine base |  |
|  |  |  | 6 | Laying of canvas/metal plate onto flooring for floor protection |  |
|  |  |  | 7 | Transportation of machine to designated location using rollers |  |
|  |  |  | 8 | Unload machine from rollers using jack |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Forklift / Moving Operation | RA Member: Zailani |  |  |  |
| Location: | 249 Jalan Boon Lay ST Engineering Advanced Material Engineering Pte Ltd | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2023 |  | Designation: | Director |  |
| Next Review Date | 30/06/2026 |  | Date: | 11/06/2025 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Transport of lifting machinery into or out of site premises | Collision of moving vehicle onto personnel’s or site property | Multiple Injuries & Damage to Property | - To coordinate with the occupier for safe route access - Vehicle only to be moved into site with the aid of banks man guiding the driver - Clear all obstruction in way of moving path - Driver to drive at slow speed and observe speed limit when entering into customer premise | 4 | 2 | 8 | Vehicle must not be left unattended while engine is running | 4 | 1 | 4 | Supervisor on site | 20/06/25 |  |
| 2 | Unloading of cases from container using 7ton forklift | Falling load | Multiple Injuries & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Known the CG of the load - Ensure load weight are spread evenly and balanced well on the fork - Elevate the load slightly and fully tilt the mast backward - Ensure no unauthorised personnel within the working zone | 4 | 2 | 8 | Use extension fork if needed to give greater stability of load | 4 | 1 | 4 | Supervisor on site | 20/06/25 |  |
|  |  | Hitting people or property | Multiple Injuries & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Operator advised to be careful when moving forward or reversing - Ensure forklift driver is guided by other staff - Ensure working area is clear of unauthorised personnel or any obstruction | 4 | 2 | 8 | Assignment of traffic controller to control movement of vehicle and personnel within the vicinity | 4 | 1 | 4 | Supervisor on site | 20/06/25 |  |
|  |  | Restricted driver visibility | Multiple Injuries & Damage to Property | - Forklift driver advise to be careful when moving forward or reversing - Ensure forklift driver is guided by other staff - Press horn if necessary to warn others that forklift is approaching - If required, reverse driving with reverse light on - Drive at slow transportation speed | 3 | 2 | 6 | Assignment of traffic controller to control movement of vehicle and personnel within the vicinity | 3 | 1 | 3 | Supervisor on site | 20/06/25 |  |
| 3 | Uncrate case | Sharp edges | Hand / Finger injury | - P.P.E (safety shoe, safety helmet, glove) provided and put on - To keep hand and finger clear of sharp edges - Uncrate and dismantle case in the correct sequence - Use of proper tools for the job - Proper housekeeping to be maintained during work - Ensure no unauthorised personnel near working area | 3 | 2 | 6 | Good coordination between workers and close supervision by supervisor | 3 | 1 | 3 | Supervisor on site | 20/06/25 |  |
| 4 | Fork up machine from base and transport to door entrance | Falling load | Multiple Injuries & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Ensure forklift fork is properly aligned directly to the lifting point of machine - Known the CG of the load - Ensure machine load are spread evenly and balanced well on the fork - Elevate the load slightly and fully tilt the mast backward - Ensure no unauthorised personnel is within the working zone | 4 | 2 | 8 | Use extension fork if needed to give greater stability of load | 4 | 1 | 4 | Supervisor on site | 20/06/25 |  |
| 5 | Insert rollers under machine bed | Sharp edges of machine | Cuts from sharp edges | - To keep hand/finger clear of sharp edges - Insert rollers to below machine base with extra care - Use glove - Tighten bolts and nuts onto rollers with adjustable spanner | 2 | 2 | 4 | Close supervision by supervisor | 2 | 1 | 2 | Supervisor on site | 20/06/25 |  |
|  |  | Pinch point | Injury to hand | - PPE (safety glove) provided and put on - Never place hand where you can’t see them - If view is obstructed, do not proceed with the work until having a clearer view - To standby solid woodblock below machine and remove only after rollers are properly secured onto machine bed | 3 | 2 | 6 | Always stay vigilant and pay close attention to surrounding area | 3 | 1 | 3 | Supervisor on site | 20/06/25 |  |
| 6 | Laying of canvas/metal plate onto flooring for floor protection | Sharp edges | Cuts | - To replace defective sheet with a new one - Deploy extra manpower to assist in laying metal sheets to minimize weight carry on each individual - Buddy system-two worker to lift one piece of metal sheet and lay out onto flooring one at a time - Wear glove | 2 | 2 | 4 | Regular check by storeman and ensure equipment issue is in good condition | 2 | 1 | 2 | Supervisor on site | 20/06/25 |  |
| 7 | Transport machine to designated location using rollers | Unstable load during moving or turning | Multiple Injuries & Damage to Property if machine topper | - Ensure rollers are place accordingly on machine base - Ensure rollers are properly secured onto machine base with bolt and nuts - Tighten bolts and nuts with adjustable spanner - Ensure machine load are spread evenly and balanced well on the rollers before being transport to location - Workers to be alert and keep close lookout on route of travel from point to point and to ensure rollers are not moving out of position | 4 | 2 | 8 | Slow moving speed Supervisor to take note of the pathway which machine is moving towards and to make sure machine load always while being transport to location | 4 | 1 | 4 | Supervisor on site | 20/06/25 |  |
|  |  | Obstruction to vision while moving | Damage to Property when hitting other obstacles | - Supervisor to ensure operation area is clear of unauthorised personnel - To keep close lookout on route of travel - Ensure movement is guided - To alert personnel controlling the pallet truck steering if potential hitting of other obstacles | 1 | 2 | 2 | Provision of spotter at both end of machine so as to have a clearer view of machine before hitting any property | 1 | 2 | 2 | Supervisor on site | 20/06/25 |  |
|  |  | Trip & Fall | Sprains & strains | - Passage way and working area to be keep clear of tooling and materials before moving job - Proper housekeeping to be maintained during work - Keep the floor in the work area dry and clean - Oil/water spill must be clean up immediately - Tape down any power cords or cable if any to prevent from tripping | 2 | 2 | 4 | Supervisor to make sure route of transporting before moving and double check clearing of all material and tooling | 2 | 1 | 2 | Supervisor on site | 20/06/25 |  |
| 8 | Unload machine from rollers using jack | Machine topple due to lost of balance upon lifted up | Multiple Injuries & Damage to Property | - Workers to be alert when jack up machine - Ensure machine are jack up at the right angle of gravity - Jack up machine to 4” high from one side to another is carried out in stages with the use of solid woodblock to be insert below machine bed - Supervisor to ensure operation area is clear of unauthorised personnel | 4 | 2 | 8 | Only skill and experienced worker to control the jack handling | 4 | 1 | 4 | Supervisor on site | 20/06/25 |  |