Inventory of Work Activities

| Company: Eric Wong Machinery Transportation Pte Ltd |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 10 Tuas South St 5 | Forklift/moving operation | 1 | Using forklift to unload machine from lorry chassis to unloading area |  |
|  |  |  | 2 | Transportation of machine to designated location using rollers |  |
|  |  |  | 3 | Unload machine from rollers |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader : Kevin Wong | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Forklift / Moving Operation | RA Member : Zailani |  |  |  |
| Location: | No 10 Tuas South St 5 CDI Multiseals Pte Ltd | RA Member : Zaharin |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2017 |  | Designation: | Director |  |
| Next Review Date | 30/06/2020 |  | Date: | 08/01/2018 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Using forklift to unload machine from lorry chassis to unloading area | Falling load | Bodily Injury & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Known the CG of the load - Ensure machine are stable on the fork before proceeding to drive - Elevate the load slightly and fully tilt the mast backward - Ensure no unauthorised personnel within the working zone | 4 | 2 | 8 | Use extension fork if needed to give greater stability of load | 4 | 1 | 4 | Supervisor on site | 10/01/18 |  |
|  |  | Hitting people or property | Bodily Injury & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Operator advised to be careful when moving forward or reversing - Ensure working area is clear of unauthorised personnel or any obstruction before any moving job is done | 4 | 2 | 8 | Assignment of traffic controller to guide and control movement of vehicle or personnel within the vicinity | 4 | 1 | 4 | Supervisor on site | 10/01/18 |  |
|  |  | Restricted driver visibility | Bodily Injury & Damage to Property | - Forklift driver advise to be careful when moving forward or reversing - Ensure forklift driver is guided by other staff - Press horn if necessary to warn others that forklift is approaching - If required, reverse driving with reverse light on - Drive at slow transportation speed | 3 | 2 | 6 | Assignment of traffic controller | 3 | 1 | 3 | Supervisor on site | 10/01/18 |  |
| 2 | Transportation of machine to designated location using rollers | Unstable load during moving or turning | Bodily Injury & Damage to Property | - Ensure rollers are place accordingly on machine base - Ensure rollers are secured to machine base with bolts and nuts to prevent from rolling out - Ensure machine load are spread evenly and is stable on the rollers before being transport to location - Workers to be alert and keep close lookout on route of travel from point to point and to ensure rollers are not moving out of position | 4 | 2 | 8 | Slow moving speed | 4 | 1 | 4 | Supervisor on site | 10/01/18 |  |
|  |  | Excessive weight load | Sprain back when pushing heavy load | - Use the correct capacity rollers to reduce the effort of each man when pushing the machine - Supervisor to ensure there are adequate manpower for the job | 3 | 3 | 9 | Use machine puller or forklift if possible to assist in pulling/ pushing the machine | 3 | 2 | 6 | Supervisor on site | 10/01/18 |  |
|  |  | Passage way block by materials or tooling | Trip & Fall | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Oil/water spill must be clean up immediately | 2 | 2 | 4 | Supervisor to make sure route of transporting before moving and double check clearing of all materials and tooling | 2 | 1 | 2 | Supervisor on site | 10/01/18 |  |
|  |  | Obstruction to vision while moving | Damage to Property | - To keep close lookout on route of travel - Ensure movement is guided - To alert worker controlling the pallet truck steering if potential hitting of other obstacles | 2 | 2 | 4 | Provision of spotter at both end so as to have a clearer view before machine hit any obstacles | 2 | 1 | 2 | Supervisor on site | 10/01/18 |  |
| 3 | Unload machine from rollers | Machine topple due to lost of balance upon lifted up | Bodily Injury & Damage to Property | - Workers to be alert when lift up machine - Ensure machine are jack up at the right angle of gravity - Jack up and lower down machine from one side to another is carried out in stages with the use of hard wooden block | 4 | 2 | 8 | Only skill and experience worker to control the jack handling | 4 | 1 | 4 | Supervisor on site | 10/01/18 |  |