Inventory of Work Activities

| Company: Eric Wong Machinery Transportation Pte Ltd |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 12 Jalan Tukang Innovation | Forklift operation | 1 | Unload conveyor from lorry chassis |  |
|  |  |  | 2 | Transport of conveyor into factory using rollers/pallet truck |  |
|  |  |  | 3 | Load conveyor into trench using forklift |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader : Kevin Wong | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Forklift Operation | RA Member : Zailani |  |  |  |
| Location: | No 12 Jalan Tukang Innovation Dorma Production GmbH Co KG | RA Member : Zaharin |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2017 |  | Designation: | Director |  |
| Next Review Date | 30/06/2020 |  | Date: | 16/10/2017 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Unload conveyor from lorry chassis | Falling load | Bodily Injury & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Known the CG of the load - Elevate the load slightly and fully tilt the mast backward - Ensure no unauthorised personnel is near the working zone | 4 | 2 | 8 | Use extension fork to give greater stability of load | 4 | 1 | 4 | Supervisor/ forklift driver on site | As per required schedule |  |
| 2 | Transport of conveyor into factory using rollers/pallet truck | Unstable load while moving or turning | Bodily Injury & Damage to Property | - Ensure rollers/pallet truck are place accordingly on conveyor base - Ensure load is stable on the rollers/pallet truck before being transport to location - Buddy system-one to control the rollers/pallet truck steering while the others to push and hold onto the load - Workers to be alert and keep a close lookout on route of travel from point to point | 4 | 2 | 8 | Slow moving speed | 4 | 1 | 4 | Supervisor on site | As per required schedule |  |
| 3 | Load conveyor into trench using forklift | Hitting people or property | Bodily Injury & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Operator advised to be careful when moving forward or reversing - Ensure working area is clear of unauthorised personnel or any obstruction | 4 | 2 | 8 | Assignment of traffic controller to give proper command to forklift operator when required | 4 | 1 | 4 | Supervisor on site | As per required schedule |  |
|  |  | Restricted driver visibility | Bodily Injury & Damage to Property | - Forklift driver advise to be careful when moving forward or reversing - Ensure forklift driver is guided by other staff - Press horn if necessary to warn others that forklift is approaching - Drive at slow speed | 3 | 2 | 6 | Assignment of traffic controller to give proper command to forklift operator when to move forward and lower down conveyor into trench | 3 | 1 | 3 | Supervisor on site | As per required schedule |  |
|  |  | Failing of lifting gears | Bodily Injury & Damage to Property | - All lifting gears use are tested by approved person and has valid certificate - Riggers to ensure lifting gear use are in compliance with weight of machine - No defective gears allowed - Load must be securely slug, properly balanced and trimmed to prevent from dislodgment | 4 | 2 | 8 | While forklift is being move forward, try not to jerk the load | 4 | 1 | 4 | Supervisor on site | As per required schedule |  |