Inventory of Work Activities

| Company: Eric Wong Machinery Transportation Pte Ltd |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 501 Airport Rd | Loading/unloading by forklift | 1 | Loading/unloading by forklift |  |
|  |  |  | 2 | Transportation of load to location |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader : Kevin Wong | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Loading/unloading by forklift | RA Member : Zailani |  |  |  |
| Location: | 501 Airport Rd | RA Member : Zaharin |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 15/05/2015 |  | Designation: | Director |  |
| Next Review Date | 15/05/2018 |  | Date: | 18/03/2016 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Loading / unloading using forklift | Falling load | Bodily Injury & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Known the CG of the load - Elevate the load slightly and fully tilt the mast backward - Ensure stability of load before driving off - Ensure no unauthorised personnel within the working zone | 4 | 2 | 8 | Use extension fork if needed to give greater stability of load | 4 | 2 | 8 | Supervisor / forklift driver on site | To be advised |  |
|  |  | Hitting people or property | Bodily Injury & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Operator advised to be careful when moving forward or reversing - Ensure working area is clear of unauthorised personnel - Ensure forklift driver is guided by other staff if needed - Press horn if necessary to warn others that forklift is approaching | 4 | 2 | 8 | Assignment of traffic controller | 4 | 2 | 8 | Supervisor / forklift driver on site | To be advised |  |
| 2 | Transporting of load to location | Restricted driver visibility | Bodily Injury & Damage to Property | - Forklift equipped with side mirror and reverse warning alarm to cautioned nearby personnel - Ensure forklift driver is guided by other staff if needed - Drive at slow transportation speed | 3 | 2 | 6 | Assignment of traffic controller | 3 | 2 | 6 | Supervisor / forklift driver on site | To be advised |  |
|  |  | Blind spots | Bodily Injury & Damage to Property | - Forklift driver advise to be careful when moving forward and reversing - Press horn if necessary to warn others that forklift is approaching - Drive at slow transportation speed | 3 | 2 | 6 | Assignment of traffic controller | 3 | 2 | 6 | Supervisor / forklift driver on site | To be advised |  |