Inventory of Work Activities

| Company: Eric Wong Machinery Transportation Pte Ltd |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 7 Benoi Crescent | Forklift / moving operation | 1 | Transport of lorry crane into or out of site premise |  |
|  |  |  | 2 | Hoist down forklift from lorry chassis |  |
|  |  |  | 3 | Unload case using forklift |  |
|  |  |  | 4 | Uncrate case |  |
|  |  |  | 5 | Detach conveyor from base and transport to door entrance |  |
|  |  |  | 6 | Using pallet truck to shift conveyor into production area |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader : Kevin Wong | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Forklift/Moving Operation | RA Member : Zailani |  |  |  |
| Location: | No 7 Benoi Crescent | RA Member : Zaharin |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 15/05/2015 |  | Designation: | Director |  |
| Next Review Date | 15/05/2018 |  | Date: | 10/11/2016 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Transport of lorry crane into or out of site premise | Collision of moving vehicle onto personnel or site property | Bodily Injury & Damage to Property | - Vehicle to be move into site with the aid of banks man guiding the driver - Drive at slow speed when entering into customer premise - Clear all obstruction in way of moving vehicle | 4 | 2 | 8 | Vehicle must not be left unattended while engine is running | 4 | 2 | 8 | Supervisor on site | TBA | Reda personnel to ensure area of unloading are clear on moving day |
| 2 | Hoist down forklift from lorry chassis | Failing of lifting equipment | Bodily Injury & Damage to Property | - All lifting equipment are tested by approved person and has valid certificate - Ensure LM for lorry crane are valid - Riggers to ensure lifting gears use are in compliance with weight of load - Never stand close or under the load - PPE (etc…safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | While moving or slewing the crane, try not to jerk the load | 4 | 2 | 8 | Lifting supervisor on site | TBA |  |
| 3 | Unload case using forklift | Falling load | Bodily Injury & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Known the CG of the load - Elevate the load slightly and fully tilt the mast backward - Ensure no unauthorised personnel within the working zone | 4 | 2 | 8 | Use extension fork if needed to give greater stability of load | 4 | 2 | 8 | Supervisor on site | TBA |  |
| 4 | Uncrate case | Sharp edges/ wooden splinters | Cut | - Keep hand and finger clear of sharp edges - Remove sharp objects (etc…nails, metal strip) from working area as soon as possible - Use glove | 3 | 2 | 6 | Close supervision by supervisor | 3 | 2 | 6 | Supervisor on site | TBA |  |
|  |  | Falling of wooden plank/timber | Bodily Injury | - Remove immediately any loose plank/timber - PPE (etc… safety helmet, safety shoe) provided and put on - Proper housekeeping to be maintained during work | 3 | 2 | 6 | Good coordination between workers | 3 | 2 | 6 | Supervisor on site | TBA |  |
| 5 | Detach conveyor from base and transport to door entrance | Falling load | Bodily Injury & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Known the CG of the load - Elevate the load slightly and fully tilt the mast backward - Ensure stability of load before driving off | 4 | 2 | 8 | Use extension fork if needed to give greater stability of load | 4 | 2 | 8 | Supervisor on site | TBA |  |
|  |  | Restricted driver visibility | Bodily Injury & Damage to Property | - Forklift driver advise to be careful when moving forward or reversing - Ensure forklift driver is guided by other staff - Press horn if necessary to warn others that forklift is approaching - If required, reverse driving with reverse light on - Drive at slow transportation speed | 3 | 2 | 6 | Assignment of traffic controller | 3 | 2 | 6 | Supervisor on site | TBA |  |
| 6 | Using pallet truck to shift conveyor into production area | Unstable load during moving or turning | Bodily Injury & Damage to Property | - Ensure pallet truck are place accordingly on conveyor base - Ensure conveyor load are spread evenly and is stable on the pallet truck before being transport to location - Buddy system- one to control the pallet truck steering while the others to push and hold onto the load | 4 | 2 | 8 | Slow moving speed | 4 | 2 | 8 | Supervisor on site | TBA |  |
|  |  | Passage way block by materials or tooling | Trip & Fall | - To ensure passage way and working area are to be keep clear of materials or tooling before moving job - Proper housekeeping to be maintained during job - Oil/water spill must be clean up immediately | 2 | 2 | 4 | Good coordination between workers | 2 | 2 | 4 | Supervisor on site | TBA |  |
|  |  | Obstruction to vision while moving | Damage to Property | - To keep close lookout on route of travel from point to point - Ensure movement is guided - To alert personnel controlling the pallet truck steering if potential hitting of other obstacles | 2 | 2 | 4 | Provision of spotter | 2 | 2 | 4 | Supervisor on site | TBA |  |