Inventory of Work Activities

| Company: Eric Wong Machinery Transportation Pte Ltd |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | NUS, Blk SDE3 #01-11 | Forklift & lorry loader operation | 1 | Using forklift to fork down machine from mezzanine floor |  |
|  |  |  | 2 | Transport machine to loading bay and load onto lorry chassis |  |
|  |  |  | 3 | Lifting operation – to unload machine from lorry chassis |  |
|  |  |  | 4 | Transportation of machine to designate location |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Forklift & Lorry Loader Operation | RA Member: Zailani |  |  |  |
| Location: | NUS, Blk SDE3, #01-11 Design Fabrication Workshop, Department of Architecture | RA Member: Zaharin |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2017 |  | Designation: | Director |  |
| Next Review Date | 30/06/2020 |  | Date: | 20/08/2019 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Using forklift to fork down machine from mezzanine floor | Falling load | Bodily Injury & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Known the CG of the load - To ensure machine is secured firmly and balanced well on the fork before proceeding to fork down machine to ground floor - Use extension fork if needed to give greater stability of load - Elevate the load slightly and fully tilt the mast backward - To avoid sudden jerk of the load - To ensure no unauthorised personnel is under or within the working area | 4 | 2 | 8 | Ensure proper means of signal is given to forklift driver when reaching mezzanine floor as when to lift up and lower down the fork | 4 | 1 | 4 | Supervisor on site | 21/08/19 |  |
|  |  | Hitting people or property | Bodily Injury & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Drive at slow speed - Forklift driver advised to be careful when moving forward or reversing - Ensure forklift driver is guided by other stuff - To ensure no unauthorised personnel is within the working area | 3 | 2 | 6 | Assignment of traffic controller to guide movement around operation area | 3 | 1 | 3 | Supervisor on site | 21/08/19 |  |
| 2 | Transport machine to loading bay and load onto lorry chassis | Restricted driver visibility | Bodily Injury & Damage to Property | - Forklift equipped with side mirror and reverse warning alarm to caution nearby people - Ensure forklift is guided by other staff when required - Slow transportation speed | 4 | 2 | 8 | Assignment of traffic controller to guide movement around operation area | 4 | 1 | 4 | Supervisor on site | 21/08/19 |  |
|  |  | Blind spot | Damage to Property when hitting other obstacles | - Forklift driver advised to be careful when moving forward or reversing - Press horn if necessary to warn others that forklift is approaching | 2 | 2 | 4 | Assignment of traffic controller to guide movement around operation area | 2 | 1 | 2 | Supervisor on site | 21/08/19 |  |
| 3 | Lifting operation – to unload machine from lorry chassis | Falling of lifting equipment | Bodily Injury & Damage to Property | - All lifting equipment are tested by approved person and has valid certificate - Rigger to ensure lifting equipment use are in compliance with weight of machine - Never stand close or under the load - PPE (etc.. safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | While moving or slewing the crane, try not to jerk the load | 4 | 1 | 4 | Lifting supervisor on site | 21/08/19 |  |
|  |  | Struck by objects due to unsafe rigging | Bodily Injury & Damage to Property | - Entire lifting crew is trained and have relevant skill, certificate and experiences required to carry out tasks - Load must be securely slug, properly balanced and trimmed to prevent from dislodgment - Ensure load is rig at the centre of gravity | 4 | 2 | 8 | Communication through hand signal or via walkie-talkie by Eric Wong personnel only | 4 | 1 | 4 | Lifting supervisor on site | 21/08/19 |  |
|  |  | Hand caught by or in between loose part of sling | Arm or Hand Injury | - Keep hands off loose part of sling or lifting gears - Ensure signalman give proper means of signal to crane operator - Use glove | 3 | 2 | 6 | Signalman to command crane operator when to lift and supervisor to ensure all worker are hand off any loose part of lifting gears | 3 | 1 | 3 | Lifting supervisor on site | 21/08/19 |  |
| 4 | Transportation of machine to designated location | Unstable load during moving or turning | Bodily Injury & Damage to Property if machine topper | - Ensure rollers/pallet truck are place accordingly on machine base - Ensure machine load are spread evenly and balanced well on the rollers/pallet truck before being transport to location - Buddy system-one to control the rollers/pallet truck steering while the others push and hold on onto the machine - Supervisor to ensure there are adequate manpower to be deploy for the job - Workers to be alert and keep close lookout on route of travel from point to point | 4 | 2 | 8 | Slow moving speed | 4 | 1 | 4 | Supervisor on site | 21/08/19 |  |
|  |  | Passageway block by material or tooling | Trip & Fall | - Passageway and working area to be keep clear of material and tooling before moving job - Proper housekeeping to be maintained during work | 2 | 2 | 4 | Supervisor to ensure route of transporting before moving and to double check clearing of all materials and tooling | 2 | 1 | 2 | Supervisor on site | 21/08/19 |  |
|  |  | Obstruction to vision while moving | Damage to Property when hit other obstacles | - To keep close lookout on route of travel - Ensure movement is guided - To alert personnel controlling the rollers/pallet truck steering if potential hitting other obstacles | 2 | 2 | 4 | Provision of spotter at both end of machine so as to have a clearer view before hitting other obstacles | 2 | 1 | 2 | Supervisor on site | 21/08/19 |  |