Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 2 A.M.K Dr | Lorry loader operation & assemble ‘A’ frame | 1 | Transport of lifting machinery into or out of site premises |  |
|  |  |  | 2 | Lifting operation – to hoist down ‘A’ frame structure from lorry chassis |  |
|  |  |  |  | Transport structure into workshop using pallet truck |  |
|  |  |  | 3 | Fixing ‘A’ frame |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by: Eric Wong Signature: | Reference Number: |
| --- | --- | --- | --- | --- |
| Process: | Lorry Loader Operation & Assemble ‘A’ Frame | RA Member: Zailani |  |  |
| Activity Location: | 2 A.M.K Drive ITE College Central | RA Member: Daniel Wong |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: Eric Wong |  |
| Last Review Date: | 01/07/2023 |  | Designation: Director |  |
| Next Review Date | 30/06/2026 |  | Date: 22/12/2025 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Transport of lifting machinery into or out of site premises | Collision of moving vehicle onto personnel’s or site property | Bodily Injury & Damage to Property | - To coordinate with the occupier for safe route access - Vehicle only to be moved into site with the aid of banks man guiding the driver - Clear all obstruction in way of moving path - Driver to observe speed limit and drive at slow speed when entering into customer premise | 4 | 2 | 8 | Vehicle must not be left unattended while engine is running | 4 | 1 | 4 | Supervisor on site | TBA |  |
| 2 | Lifting operation – hoist down ‘A’ frame structure from lorry chassis | Failing of lifting equipment | Bodily Injury & Damage to Property | - All lifting equipment are tested by approved person and has valid certificate - Ensure LM for lorry crane are valid - Riggers to ensure lifting gears use are in compliance with weight of machine - No defective gears allowed - Never stand close or under the load - PPE (safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | While moving or slewing the crane, try not to jerk the load | 4 | 1 | 4 | Lifting supervisor on site | TBA |  |
|  |  | Overloading | Bodily Injury & Damage to Property | - No lifting beyond SWL of equipment - Load weight should be known - Implement of Lifting Plan to all personnel on site to ensure that the lifting operation is properly manage | 4 | 2 | 8 | Ensure load, capacity of lifting arm and reach are within limit, when in doubt, always use a higher capacity crane | 4 | 1 | 4 | Lifting supervisor on site | TBA |  |
|  |  | Struck by objects due to unsafe slinging, rigging | Bodily Injury & Damage to Property | - Entire lifting crew is trained and to have relevant skill, certificate and experiences required to carry out tasks - Load must be securely slug, properly balanced and trimmed to prevent from dislodgment - Ensure load is rig at the centre of gravity - Riggers to check and ensure safe rigging and lifting supervisor to confirm load are safe to be rig | 4 | 2 | 8 | Communication through hand signal or via walkie-talkie by Eric Wong personnel only | 4 | 1 | 4 | Lifting supervisor on site | TBA |  |
| 3 | Transport structure into workshop using pallet truck | Unstable load while moving or turning | Bodily Injury & Damage to Property | - Ensure pallet truck are place accordingly on structure bed - Buddy system-one to control the pallet truck steering while the others to push and hold onto the structure - Workers to be alert and keep close lookout on route of travel from point to point | 4 | 2 | 8 | Slow moving speed Supervisor to take note of the pathway which structure is moving toward and to make sure load always stable on the pallet truck | 4 | 1 | 4 | Supervisor on site | TBA |  |
|  |  | Trip & Fall | Sprains & Strains | - Passageway and working area to be keep clear of any obstruction before moving job - Proper housekeeping to be maintained during work - Oil/water spill must be clean up immediately | 2 | 2 | 4 | Good coordination between workers to ensure any obstruction are move aside before any moving are done | 2 | 1 | 2 | Supervisor on site | TBA |  |
|  |  | Obstruction to vision while moving | Damage to Property when hitting other obstacle | - Supervisor to ensure operation area is clear of unauthorised personnel - To keep close lookout on route of travel - Ensure movement is guided - To alert personnel controlling the pallet truck steering if potential hitting of other obstacles | 1 | 2 | 2 | Provision of spotter at both end of structure so as to have a clearer view before hitting any obstacles | 1 | 2 | 2 | Supervisor on site | TBA |  |
| 4 | Fixing ‘A’ frame | Manual handling of heavy equipment | Sprain back when carrying heavy load | - Adopt proper technique when carrying out task - Ensure sufficient rest and recovery time is taken during repeated lift - Supervisor to ensure there are adequate manpower to be deploy for the job - Apply buddy system when handling of heavy equipment - PPE (safety helmet, safety shoe) provided and put on | 3 | 2 | 6 | Use of mechanical aid such as lifter if require | 3 | 1 | 3 | Supervisor on site | TBA |  |
|  |  | Fix bolt and nut onto frame | Fall from height | - Use of ‘A’ frame ladder to work on which is unreachable from ground - Ladder use must be fully extended and stand on level ground - Apply buddy system - one to hold onto the ladder while the other is working on the ladder | 4 | 2 | 8 | Close supervision by supervisor | 4 | 1 | 4 | Supervisor on site | TBA |  |
|  |  | Pinch point | Injury to hand | - PPE (glove) provided and put on - Never place hand where you can’t see them - Pay close attention around any moving parts of the equipment you’re working on - If view is obstructed, do not proceed with the work until having a clearer view | 3 | 2 | 6 | Always stay vigilant and pay close attention to surrounding area | 3 | 1 | 3 | Supervisor on site | TBA |  |