Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 70 Joo Koon Circle | Assist engineer dismantle machine parts | 1 | Operating of boom lift into workshop |  |
|  |  |  | 2 | Working at height to assist engineer dismantle parts |  |
|  |  |  | 3 | Lifting operation |  |
|  |  |  | 4 | Signalling of load to drop off point |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Dismantle of machine parts | RA Member: Zailani |  |  |  |
| Location: | 70 Joo Koon Circle Nakano Metal Press (S) Pte Ltd | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2020 |  | Designation: | Director |  |
| Next Review Date | 30/06/2023 |  | Date: | 05/12/2022 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Operating of boom lift into workshop | Hitting people or property | Bodily injury & damage to property | - Only certified and competent boom lift operator are allowed to operate the boom lift - Operator advised to be careful when moving forward or reversing - To ensure no unauthorised personnel is within the working area - To clear off all material or tooling that is obstructing the work area - Ensure proper PPE (safety helmet, safety shoe) to be put on all the time when entering the site | 4 | 2 | 8 | Slow moving speed | 4 | 1 | 4 | Supervisor on site | 07/12/22 |  |
| 2 | Working at height to assist engineer dismantle parts | Fall from height | Bodily Injury | - Provision of boom lift as working platform to work at height - Only certified and competent boom lift operator are allowed to operate the boom lift - Worker working at height on top boom lift to put on proper PPE (safety harness, safety helmet) - Ensure safety harness use is in good working condition, properly strap on and hook to strong anchorage point of boom lift at all time | 4 | 2 | 8 | All trained and authorised operator record to be paste at the boom lift | 4 | 1 | 4 | Supervisor on site | 07/12/22 |  |
| 3 | Lifting operation | Failing of lifting equipment | Bodily Injury & Damage to Property | - All lifting equipment are tested by approved person and has valid certificate - Ensure LM for crane are valid - Ensure lifting gears use are in compliance with weight of load - Never stand close or under the load - PPE (safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | While moving or slewing the crane, try not to jerk the load | 4 | 1 | 4 | Supervisor on site | 07/12/22 |  |
|  |  | Struck by load due to unsafe slinging, rigging | Bodily Injury & Damage to Property | - Load shall be securely slug, properly balanced and trimmed to prevent from dislodgement - Ensure load is rig at the centre of gravity | 4 | 2 | 8 | Communication through hand signal or walkie-talkie by Eric Wong personnel only | 4 | 1 | 4 | Supervisor on site | 07/12/22 |  |
| 4 | Signalling of load to drop off point | Load hit structure due to improper signalling | Bodily Injury & Damage to Property | - Tagline for load when necessary to prevent of load swing - Clear all obstruction and unauthorised personnel in way of lifting path - Never stand close or under the suspended load and away from possible swing path - PPE (safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | Close supervision by supervisor | 4 | 1 | 4 | Supervisor on site | 07/12/22 |  |