Inventory of Work Activities

| Company: Eric Wong Machinery Transportation Pte Ltd |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | N3.1, 65 Nanyang Drive | Lorry loader/forklift operation | 1 | Transport of lifting machinery into or out of site premises |  |
|  |  |  | 2 | Setting up of crane |  |
|  |  |  | 3 | Lifting operation / to unload forklift from lorry chassis |  |
|  |  |  | 4 | Unloading of case from forwarder lorry using forklift |  |
|  |  |  | 5 | Uncrate case |  |
|  |  |  | 6 | Transport machine to designated location |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Lorry Loader / Forklift Operation | RA Member: Zailani |  |  |  |
| Location: | NTU, N3.1, 65 Nanyang Drive HP-NTU Digital Manufacturing Corporate Lab, B4b-02 | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2020 |  | Designation: | Director |  |
| Next Review Date | 30/06/2023 |  | Date: | 07/10/2021 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Transport of lifting machinery into or out of site premises | Collision of moving vehicle onto personnel’s or site property | Bodily Injury & Damage to Property | - Vehicle to be move into site with the aid of bank’s man guiding the driver - Clear all obstruction in way of moving path - Drive at slow speed when entering into customer premise | 4 | 2 | 8 | Vehicle must not be left unattended while engine is running | 4 | 1 | 4 | Supervisor on site | 13/10/21 |  |
| 2 | Setting up of crane | Hit by moving parts of crane | Bodily Injury & Damage to Property | - Operator trained and competent person appointed - Crane operator to be on site when extending or retracting outriggers (must be able to see all operation procedure) - Crane operator to ensure outriggers beam is fully extended and resting on steel plate/timber mats as base plate - PPE (safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | Ensure no unauthorised personnel or vehicle near moving parts of crane | 4 | 1 | 4 | Crane operator on site | 13/10/21 |  |
| 3 | Lifting operation / to unload forklift from lorry chassis | Failing of lifting equipment | Bodily Injury & Damage to Property | - All lifting equipment are tested by approved person and has valid certificate - Ensure LM for lorry crane are valid - Riggers to ensure lifting gears use are in compliance with weight of load - Never stand close or under the load - PPE (safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | While moving or slewing the crane, try not to jerk the load | 4 | 1 | 4 | Lifting supervisor on site | 13/10/21 |  |
|  |  | Overloading | Bodily Injury & Damage to Property | - No lifting beyond SWL of crane - Load weight should be known | 4 | 2 | 8 | Ensure load, capacity of lifting arm and reach are within limit, when in doubt, always use a higher capacity crane | 4 | 1 | 4 | Lifting supervisor on site | 13/10/21 |  |
|  |  | Struck by object due to unsafe slinging, rigging | Bodily Injury & Damage to Property | - Entire lifting crew is trained and to have relevant skill, certificate and experiences required to carry out tasks - Load must be securely slug, properly balanced and trimmed to prevent from dislodgment - Ensure load is rig at the centre of gravity | 4 | 2 | 8 | Communication through hand signal or via walkie-talkie by Eric Wong personnel only | 4 | 1 | 4 | Lifting supervisor on site | 13/10/21 |  |
| 4 | Unloading of case from forwarder lorry using forklift | Falling load | Bodily Injury & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Known the CG of the load - Elevate the load slightly and fully tilt the mast backward - Ensure machine load are spread evenly and balanced well on the fork before - Ensure no unauthorised personnel is within the working zone | 4 | 2 | 8 | Use extension fork if needed to give greater stability of load | 4 | 1 | 4 | Supervisor on site | 13/10/21 |  |
|  |  | Hitting people or property | Bodily Injury & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Operator advised to be careful when moving forward or reversing - Ensure working area is clear of unauthorised personnel - Ensure forklift driver is guided by other staff - Press horn if necessary to warn others that forklift is approaching | 4 | 2 | 8 | Assignment of traffic controller to control movement of vehicle and people within the vicinity | 4 | 1 | 4 | Supervisor on site | 13/10/21 |  |
|  |  | Restricted driver visibility | Bodily Injury & Damage to Property | - Forklift driver advise to be careful when moving forward or reversing - Ensure forklift driver is guided by other staff - Press horn if necessary to warn others that forklift is approaching - If required, reverse driving with reverse light on - Drive at slow transportation speed | 3 | 2 | 6 | Assignment of traffic controller to control movement of vehicle and personnel within the vicinity | 3 | 1 | 3 | Supervisor on site | 13/10/21 |  |
| 5 | Uncrate case | Falling of wooden plank | Bodily Injury | - PPE (safety helmet, safety shoe) provided and put on - Uncrate and dismantle case in the correct sequence - Remove immediately any loose plank - Proper housekeeping to be maintained during work - Ensure no unauthorised personnel is near the working area | 3 | 2 | 6 | Good coordination between workers | 3 | 1 | 3 | Supervisor on site | 13/10/21 |  |
| 6 | Transport machine to designated location | Unstable load during moving or turning | Bodily Injury & Damage to Property if machine topper | - Ensure pallet truck are place accordingly on machine base - Ensure machine load are spread evenly and is stable on the pallet truck before being transport to location - Workers to be alert and keep close lookout on route of travel from point to point | 4 | 2 | 8 | Slow moving speed | 4 | 1 | 4 | Supervisor on site | 13/10/21 |  |
|  |  | Passage way block by materials or tooling | Trip & Fall | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Oil/water spill must be clean up immediately | 2 | 2 | 4 | Supervisor to make sure route of transporting before moving and to double check clearing of all materials and tooling | 2 | 1 | 2 | Supervisor on site | 13/10/21 |  |
|  |  | Obstruction to vision while moving | Damage to Property when hitting other obstacles | - To keep close lookout on route of travel - Ensure movement is guided - To alert personnel controlling the pallet truck steering if potential hitting of others obstacles | 2 | 2 | 4 | Provision of spotter at both end of machine so as to have a clearer view before hitting any obstacles | 2 | 1 | 2 | Supervisor on site | 13/10/21 |  |