Inventory of Work Activities

| Company: Eric Wong Machinery Transportation Pte Ltd |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 249 Jalan Boon Lay | Lorry loader / forklift operation | 1 | Transport of lifting machinery into or out of site premises |  |
|  |  |  | 2 | Setting up of crane |  |
|  |  |  | 3 | Attached sling to crane hook and secured to rigging point of forklift |  |
|  |  |  | 4 | Lifting operation – to hoist down forklift from lorry chassis |  |
|  |  |  | 5 | Signalling of load |  |
|  |  |  | 6 | Forklift operation – to fork down machine from lorry chassis and transport to door entrance |  |
|  |  |  | 7 | Transport machine to designated location using pallet truck |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Lorry Loader / Forklift Operation | RA Member: Zailani |  |  |  |
| Location: | 249 Jalan Boon Lay ST Engineering Advanced Material Pte Ltd | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2023 |  | Designation: | Director |  |
| Next Review Date | 30/06/2026 |  | Date: | 24/09/2025 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Transport of lifting machinery into or out of site premises | Collision of moving vehicle onto personnel’s or site property | Multiple Injuries & Damage to Property | - To coordinate with the occupier for safe route access - Vehicle only to be moved into site with the aid of bank’s man guiding the driver - Clear all obstruction in way of moving path - Driver to drive at slow speed and observe speed limit when entering into customer premise | 4 | 2 | 8 | Vehicle must not be left unattended while engine is running | 4 | 1 | 4 | Supervisor on site | 09-10 /10/25 |  |
| 2 | Setting up of crane | Hit by moving parts of crane | Multiple Injuries & Damage to Property | - Operator trained and competent person appointed - Crane operator and lifting supervisor to ensure operation area and crane swing path is clear from any obstruction - Crane operator to be on site when extending or retracting outriggers (must be able to see all operation procedure) - Ensure all outrigger beams to be fully extended and resting on steel plate/timber mats as base plate - PPE (safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | Ensure no unauthorised personnel or vehicle near moving parts of crane Barricade off the work area | 4 | 1 | 4 | Lifting supervisor on site | 09-10 /10/25 |  |
| 3 | Attached sling to crane hook and secured to rigging point of forklift | Rigger hand caught in between the LG and hook | Minor fracture to hand | - Riggers to attached and secured sling with care - To coordinate well with crane operator to ensure no lifting are done until all sling are properly attached and secured - Use glove | 3 | 2 | 6 | Signalman to command crane operator when to lift and supervisor to ensure all worker are hand off and loose part of lifting gears | 3 | 1 | 3 |  | 09-10 /10/25 |  |
|  |  | Pinch point | Injury to hand | - PPE (safety glove) provided and put on - Never place hand where you can’t see them - If view is obstructed, do not proceed with the work until having a clearer view | 3 | 2 | 6 | Always stay vigilant and pay close attention to surrounding area | 3 | 1 | 3 |  | 09-10 /10/25 |  |
| 4 | Lifting operation – to hoist down forklift from lorry chassis | Failing of lifting equipment | Multiple Injuries & Damage to Property | - All lifting equipment are tested by approved person and has valid certificate - Ensure LM for lorry crane are valid - Riggers to ensure lifting gears use are in compliance with weight of load - No defective gears allowed - Never stand close or under the load - PPE (safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | While moving or slewing the crane, do it smoothly without sudden acceleration or deceleration Ensure lifting supervisor to be 100% onsite to supervised the whole lifting operation No lifting is to be done during bad weather | 4 | 1 | 4 | Lifting supervisor on site | 09-10 /10/25 |  |
|  |  | Overloading | Multiple Injuries & Damage to Property | - No lifting beyond SWL of crane - Load weight should be known - To test lift load about 1ft to check on stability and to ensure no overloading of load to be lifted - Implement of Lifting Plan to all lifting personnel on site to ensure that the lifting operation is properly manage | 4 | 2 | 8 | Ensure load, capacity of lifting arm and reach are within limit, when in doubt, always use a higher capacity crane | 4 | 1 | 4 | Lifting supervisor on site | 09-10 /10/25 |  |
|  |  | Struck by object due to unsafe slinging, rigging | Multiple Injuries & Damage to Property | - Entire lifting crew is trained and to have relevant skill, certificate and experiences required to carry out tasks - Load must be securely slug, properly balanced and trimmed to prevent from dislodgment - Ensure load is rig at the centre of gravity - Riggers to check and ensure safe rigging and lifting supervisor to confirm load are safe to be rig | 4 | 2 | 8 | Communication through hand signal or via walkie-talkie by Eric Wong personnel only To provide crane hook with safety latch to prevent displacement of sling or load from the hook | 4 | 1 | 4 | Lifting supervisor on site | 09-10 /10/25 |  |
|  |  | Hand caught by or in between loose parts of slings | Minor fracture to arm and hand | - Keep hands off loose part of sling or lifting gears - Ensure signalman give proper means of signal to crane operator - Use glove | 3 | 2 | 6 | Signalman to command crane operator when to lift and supervisor to ensure all workers are hand off any loose part of lifting gears | 3 | 1 | 3 | Lifting supervisor on site | 09-10 /10/25 |  |
| 5 | Signalling of load | View obstruction or load swing | Multiple Injuries & Damage to Property | - Appointed and trained signalman, rigger and lifting supervisor on site - Tagline for load when necessary to prevent of load swing - Clear obstruction and unauthorised personnel in way of lifting path - Never stand close or under the suspended load and away from possible swing path - PPE (safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | Signalman to position himself to be visible to crane operator through hand signalling method or via walkie-talkie Close supervision by lifting supervisor | 4 | 1 | 4 | Lifting supervisor on site | 09-10 /10/25 |  |
| 6 | Forklift operation - to fork down machine from lorry chassis and transport to door entrance | Falling load | Multiple Injuries & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Ensure forklift fork are properly aligned directly to the lifting point of load - Known the CG of the load - Ensure equipment load are spread evenly and balanced well on the fork - Elevate the load slightly and fully tilt the mast backward - Ensure no unauthorised personnel is within the working area | 4 | 2 | 8 | Use extension fork if required to give greater stability of load | 4 | 1 | 4 | Supervisor on site | 09-10 /10/25 |  |
|  |  | Restricted driver visibility | Multiple Injuries & Damage to Property | - Forklift driver advised to be careful when moving forward or reversing - Ensure forklift driver is guided by other staff - Press horn if necessary to warn others that forklift is approaching - If required, reverse driving with reverse light on - Drive at slow transportation speed | 4 | 2 | 8 | Assignment of traffic controller to control movement of vehicle and people within the vicinity | 4 | 1 | 4 | Supervisor on site | 09-10 /10/25 |  |
| 7 | Transport machine to designated location using pallet truck | Unstable load during moving or turning | Multiple Injuries & Damage to Property | - Ensure pallet truck are place accordingly on machine base - Ensure machine load are spread evenly and balanced well on the pallet truck before being transport to location - Workers to be alert and keep close lookout on route of travel from point to point | 4 | 2 | 8 | Slow moving speed Supervisor to take note of the pathway which machine is moving towards and to make sure machine always stable on the pallet truck while being transport to location | 4 | 1 | 4 | Supervisor on site | 09-10 /10/25 |  |
|  |  | Trip & Fall | Sprains & strains | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Keep the floor in the work area dry and clean - Oil/water spill must be clean up immediately - Tape down any power cords or cable is any to prevent from tripping | 2 | 2 | 4 | Supervisor to make sure route of transporting before moving and to double check clearing of all materials and tooling | 2 | 1 | 2 | Supervisor on site | 09-10 /10/25 |  |
|  |  | Obstruction to vision while moving | Damage to Property when hitting other obstacles | - To keep close lookout on route of travel - Ensure movement is guided - To alert personnel controlling the pallet truck steering if potential hitting of others obstacles | 1 | 2 | 2 | Provision of spotter at both end of machine so as to have a clearer view before hitting any obstacles | 1 | 2 | 2 | Supervisor on site | 09-10 /10/25 |  |