Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 10 Dover Dr | Lorry Loader & Moving Operation | 1 | Transport of lifting machinery into or out of site premises |  |
|  |  |  | 2 | Setting up of crane |  |
|  |  |  | 3 | Un-crate case |  |
|  |  |  | 4 | Lifting operation |  |
|  |  |  | 5 | Transportation of machine to designated location |  |
|  |  |  | 6 | Unload machine from rollers |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by : Eric Wong Signature | Reference Number: |
| --- | --- | --- | --- | --- |
| Process: | Lorry Loader & Moving Operation | RA Member: Zailani |  |  |
| Activity Location: | 10 Dover Drive SR-B1B SIT @ Dover | RA Member: Daniel Wong |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: Eric Wong |  |
| Last Review Date: | 01/07/2017 |  | Designation: Director |  |
| Next Review Date | 30/06/2020 |  | Date: 30/02/2020 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Transport of lifting machinery into or out of site premises | Collision of moving vehicle onto personnel’s or site property | Bodily Injury & Damage to Property | - Vehicle only to be moved into site with the aid of banks man guiding the driver - Clear all obstruction in way of moving path - Drive at slow speed when entering into customer premise | 4 | 2 | 8 | Vehicle must not be left unattended while engine is running | 4 | 1 | 4 | Supervisor on site | 26/02/20 |  |
| 2 | Setting up of crane | Hit by moving parts of crane | Bodily Injury & Damage to Property | - Operator trained and competent person appointed - Crane operator and lifting supervisor to ensure operation area and crane swing path is clear from any obstruction - Crane operator to be on site when extending or retracting outriggers (must be able to see all operation procedure) - Ensure all outrigger beams must be fully extended and resting on steel plate/timber mat - PPE (safety shoe, safety helmet) provided and put on | 3 | 2 | 6 | Ensure no unauthorised personnel or vehicle near moving parts of crane Barricade off the work area | 3 | 1 | 3 | Lifting supervisor on site | 26/02/20 |  |
| 3 | Un-crate case | Falling of wooden plank | Bodily Injury | - PPE (safety shoe, safety helmet) provided and put on - Un crate and dismantle case in the correct sequence - Remove immediately any loose plank - Proper housekeeping to be maintained during work - Ensure no unauthorised personnel near working area - Use ‘A’ frame ladder to work on which is unreachable from ground | 3 | 2 | 6 | Good coordination between workers | 3 | 1 | 3 | Supervisor on site | 26/02/20 |  |
|  |  | Protruding nails on loose timber | Bodily Injury | - PPE (safety shoe, safety helmet) provided and put on - Remove immediately any loose timber with protruding nails from working area - Proper housekeeping to be maintained during work | 3 | 2 | 6 | Close supervision and regular reminder by supervisor | 3 | 1 | 3 | Supervisor on site | 26/02/20 |  |
| 4 | Lifting operation | Failing of lifting equipment | Bodily Injury & Damage to Property | - All lifting equipment are tested by approved person and has valid certificate - Ensure LM for crane are valid - Riggers to ensure lifting gears use are in compliance with weight of equipment - No defective gears allowed - Never stand close or under the load - PPE (safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | While moving or slewing the crane, try not to jerk the load Ensure lifting supervisor to be 100% onsite to supervisor the whole lifting operation | 4 | 1 | 4 | Lifting supervisor on site | 26/02/20 |  |
|  |  | Overloading | Bodily Injury & Damage to Property | - No lifting beyond SWL of equipment - Load weight should be known - To test lift load about 1ft above ground to check on stability and to ensure no overloading of load to be lifted | 4 | 2 | 8 | Ensure load, capacity of lifting arm and reach are within limit, when in doubt, always use a higher capacity crane | 4 | 1 | 4 | Lifting supervisor on site | 26/02/20 |  |
|  |  | Struck by objects due to unsafe slinging, rigging | Bodily Injury & Damage to Property | - Entire lifting crew is trained and to have relevant skill, certificate and experiences required to carry out tasks - Machine must be securely slug, properly balanced and trimmed to prevent from dislodgment - Ensure machine is rig at the centre of gravity - Riggers to check and ensure safe rigging and lifting supervisor to confirm load are safe to be rig | 4 | 2 | 8 | Communication through hand signal or via walkie-talkie by Eric Wong personnel only | 4 | 1 | 4 | Lifting supervisor on site | 26/02/20 |  |
|  |  | Hand caught by or in between loose part of sling or lifting gears | Arm or Hand Injury | - Keep hands off loose part of sling or lifting gears - Ensure signalman give proper means of signal to crane operator - Use glove | 3 | 2 | 6 | Signalman to command crane operator when to lift and supervisor to ensure all worker are hand off any loose part of lifting gears | 3 | 1 | 3 | Lifting supervisor on site | 26/02/20 |  |
|  |  | View obstruction or load swing | Bodily Injury & Damage to Property | - Appointed and trained signalman, rigger and lifting supervisor on site - Tagline should be used to control load from swinging so as to prevent bodily contact with load - Clear obstruction and unauthorised personnel in way of lifting path - Never stand close or under the suspended load and away from possible swing path | 4 | 2 | 8 | Signalman to position himself to be visible to crane operator through hand signalling method or via walkie-talkie | 4 | 1 | 4 | Lifting supervisor on site | 26/02/20 |  |
| 5 | Transportation of machine to designated location | Unstable load during moving or turning | Bodily Injury & Damage to Property if machine topper | - Ensure rollers are place accordingly on machine base - Ensure machine load are spread evenly and balance well on the rollers before being transport to location - Workers to be alert and keep close lookout on route of travel from point to point and to ensure rollers are not moving out of position | 4 | 2 | 8 | Slow moving speed | 4 | 1 | 4 | Supervisor on site | 26/02/20 |  |
|  |  | Excessive weight load | Sprain back when pushing heavy load | - Use the correct capacity rollers to reduce the effort of each man when pushing the machine - Supervisor to ensure there are adequate personnel to be deploy for the job | 3 | 3 | 9 | Use of machine puller or forklift if possible to assist in pulling / pushing the machine | 3 | 2 | 6 | Supervisor on site | 26/02/20 |  |
|  |  | Passage way block by materials and tooling | Trip & Fall | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Oil/water spill must be clean up immediately | 2 | 2 | 4 | Supervisor to make sure route of the transporting before moving and to double check clearing of all materials and tooling | 2 | 1 | 2 | Supervisor on site | 26/02/20 |  |
|  |  | Obstruction to vision while moving | Damage to Property when hitting other obstacles | - To keep close lookout on route of travel - Ensure movement is guided - To alert personnel controlling the rollers steering if potential hitting of other obstacles | 2 | 2 | 4 | Provision of spotter at both end of machine so as to have a clearer view before hitting any obstacles | 2 | 1 | 2 | Supervisor on site | 26/02/20 |  |
| 6 | Unload machine from rollers using jack | Machine topple due to lost of balance upon lifted up | Bodily Injury & Damage to Property | - Ensure jack use is in good working condition - Workers to be alert when jack up machine - Ensure machine are jack up at the right angle of gravity - Jack up and lower down machine from one side to another is carried out in stages with the use of hard wooden block | 4 | 2 | 8 | Only skill and experience worker to control the jack handling | 4 | 1 | 4 | Supervisor on site | 26/02/20 |  |