Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 249 Jalan Boon Lay | Lorry Crane / Forklift & Moving Operation | 1 | Transport of lifting machinery into or out of site premises |  |
|  |  |  | 2 | Setting up of crane |  |
|  |  |  | 3 | Open up container canvas |  |
|  |  |  | 4 | Using lorry crane to pull out case |  |
|  |  |  | 5 | Unloading of cases using forklift |  |
|  |  |  | 6 | Lifting operation – to unload case from FR container |  |
|  |  |  | 7 | Signalling of load |  |
|  |  |  | 8 | Uncrate case |  |
|  |  |  | 9 | Transport base to designated location using rollers |  |
|  |  |  | 10 | Unload case from rollers using jack |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by : Eric Wong Signature | Reference Number: |
| --- | --- | --- | --- | --- |
| Process: | Lorry Crane / Forklift & Moving Operation | RA Member: Zailani |  |  |
| Activity Location: | 249 Jalan Boon Lay ST Engineering | RA Member: Daniel Wong |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: Eric Wong |  |
| Last Review Date: | 01/07/2023 |  | Designation: Director |  |
| Next Review Date | 30/06/2026 |  | Date: 09/11/2023 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Transport of lifting machinery into or out of site premises | Collision of moving vehicle onto personnel’s or site property | Multiple Injuries & Damage to Property | - To coordinate with the occupier for safe route access - Vehicle only to be moved into site with the aid of banks man guiding the driver - Clear all obstruction in way of moving path - Drive at slow speed when entering into customer premise | 4 | 2 | 8 | Vehicle must not be left unattended while engine is running | 4 | 1 | 4 | Supervisor on site | Estimate: 14-17 /11/23 |  |
| 2 | Setting up of crane | Hit by moving parts of crane | Multiple Injuries & Damage to Property | - Operator trained and competent person appointed - Crane operator and lifting supervisor to ensure operation area and crane swing path is clear from any obstruction - Crane operator to be on site when extending or retracting outriggers (must be able to see all operation procedure) - Ensure all outrigger beams must be fully extended and resting on steel plate/timber mat as base plate - PPE (safety shoe, safety helmet) provided and put on | 4 | 2 | 8 | Ensure no unauthorised personnel or vehicle near moving parts of crane Barricade off the work area | 4 | 1 | 4 | Lifting supervisor on site | Estimate: 14-17 /11/23 |  |
| 3 | Open up container canvas | Fall from height | Multiple Injuries | - Provision of scissor lift as working platform to work at height - Only certified and competent scissor lift operator are allowed to operate the scissor lift - Worker working at height on top scissor lift to put on proper PPE (safety harness, safety helmet) - Ensure safety harness use is in good working condition, properly strap on and hook onto strong anchorage point of the scissor lift | 4 | 2 | 8 | All trained and authorised operator record to be paste on the scissor lift | 4 | 1 | 4 | Supervisor on site | Estimate: 14-17 /11/23 |  |
| 4 | Using lorry crane to pull out cases from container onto lorry chassis | Hit by moving parts of crane | Multiple Injuries & Damage to Property | - Operator trained and competent person appointed - Ensure sling use to pull out case has valid certificate - To ensure proper command are given to crane operator - To ensure no unauthorised personnel is near the working area - Workers to be alert and ensure case is being pull out slowly - PPE (safety shoe, safety helmet) provided and put on | 4 | 2 | 8 | Do not force pull the case out of container to minimize damage if hit side of container | 4 | 1 | 4 | Lifting supervisor on site | Estimate: 14-17 /11/23 |  |
| 5 | Unloading of cases using forklift | Falling load | Multiple Injuries & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Known the CG of the load - Ensure forklift fork is aligned directly to the lifting point of case - Ensure load are spread evenly and balanced well on the fork - Elevate the load slightly and fully tilt the mast backward - To ensure no unauthorised personnel is within the working zone | 4 | 2 | 8 | Use extension fork if needed to give greater stability of load | 4 | 1 | 4 | Supervisor on site | Estimate: 14-17 /11/23 |  |
|  |  | Hitting people or property | Multiple Injuries & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Forklift driver advised to be careful when moving forward or reversing - Ensure forklift driver is guided by other staff - Ensure working area is clear of unauthorised personnel or any obstruction | 4 | 2 | 8 | Assignment of traffic controller to control movement of vehicle and personnel within the vicinity | 4 | 1 | 4 | Supervisor on site | Estimate: 14-17 /11/23 |  |
| 6 | Lifting operation – to unload case from FR container | Failing of lifting equipment | Multiple Injuries & Damage to Property | - All lifting equipment are tested by approved person and has valid certificate - Ensure LM for crane are valid - Riggers to ensure lifting gears use are in compliance with weight of equipment - No defective gears allowed - Never stand close or under the load - PPE (safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | While moving or slewing the crane, try not to jerk the load Ensure lifting supervisor to be 100% onsite to supervisor the whole lifting operation | 4 | 1 | 4 | Lifting supervisor on site | Estimate: 14-17 /11/23 |  |
|  |  | Overloading | Multiple Injuries & Damage to Property | - No lifting beyond SWL of equipment - Load weight should be known - To test lift load about 1ft above ground to check on stability and to ensure no overloading of load to be lifted - Implement of Lifting Plan to all lifting personnel on site to ensure that the lifting operation is properly manage | 4 | 2 | 8 | Ensure load, capacity of lifting arm and reach are within limit, when in doubt, always use a higher capacity crane | 4 | 1 | 4 | Lifting supervisor on site | Estimate: 14-17 /11/23 |  |
|  |  | Struck by objects due to unsafe slinging, rigging | Multiple Injuries & Damage to Property | - Entire lifting crew is trained and to have relevant skill, certificate and experiences required to carry out tasks - Machine must be securely slug, properly balanced and trimmed to prevent from dislodgment - Ensure machine is rig at the centre of gravity - Riggers to check and ensure safe rigging and lifting supervisor to confirm load are safe to be rig | 4 | 2 | 8 | Communication through hand signal or via walkie-talkie by Eric Wong personnel only | 4 | 1 | 4 | Lifting supervisor on site | Estimate: 14-17 /11/23 |  |
|  |  | Hand caught by or in between loose part of sling or lifting gears | Minor fracture o hand and arm | - Keep hands off loose part of sling or lifting gears - Ensure signalman give proper means of signal to crane operator - Use glove | 3 | 2 | 6 | Signalman to command crane operator when to lift and supervisor to ensure all worker are hand off any loose part of lifting gears | 3 | 1 | 3 | Lifting supervisor on site | Estimate: 14-17 /11/23 |  |
| 7 | Signalling of load | View obstruction or load swing | Multiple Injuries & Damage to Property | - Appointed and trained signalman, rigger and lifting supervisor on site - Tagline for load when necessary to prevent of load swing - Clear obstruction and unauthorised personnel in way of lifting path - Never stand close or under the suspended load and away from possible swing path - PPE (safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | Signalman to position himself to be visible to crane operator through hand signalling method or via walkie-talkie | 4 | 1 | 4 | Lifting supervisor on site | Estimate: 14-17 /11/23 |  |
| 8 | Uncrate case | Falling of wooden plank | Multiple Injuries | - PPE (safety shoe, safety helmet) provided and put on - Uncrate and dismantle case in the correct sequence - Remove immediately any loose plank from site - Ensure no unauthorised personnel near working area | 3 | 2 | 6 | Good coordination between workers | 3 | 1 | 3 | Supervisor on site | Estimate: 14-17 /11/23 |  |
|  |  | Protruding nail on loose timber | Multiple Injuries | - PPE (safety shoe, safety helmet) provided and put on - Remove immediately any loose timber with protruding nails from working area - Proper housekeeping to be maintained during work | 3 | 2 | 6 | Close supervision and regular reminder by supervisor | 3 | 1 | 3 | Supervisor on site | Estimate: 14-17 /11/23 |  |
| 9 | Transport base to designated location using rollers | Unstable load during moving or turning | Multiple Injuries & Damage to Property if machine topper | - Ensure rollers are place accordingly on machine base - Ensure rollers are properly secured onto machine base with bolts and nuts - Tighten bolts and nuts with adjustable spanner - Ensure case load are spread evenly and balance well on the rollers before being transport to location - Workers to be alert and keep close lookout on route of travel from point to point and to ensure rollers are not moving out of position | 4 | 2 | 8 | Slow moving speed | 4 | 1 | 4 | Supervisor on site | Estimate: 14-17 /11/23 |  |
|  |  | Excessive weight load | Sprain back if force moving the load | - Use the correct capacity rollers to ensure roller wheel and bearing can sustained the weight of load - Proper tool such as manual puller to be brought to site - Supervisor to ensure there are adequate personnel to be deploy for the job - Use of waist belt, if needed, to prevent sprain back or backache | 3 | 2 | 6 | Use of manual puller to assist in pulling the machine when required | 3 | 1 | 3 | Supervisor on site | Estimate: 14-17 /11/23 |  |
|  |  | Trip & fall | Sprains & strains | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Keep the floor in the work area dry and clean - Oil/water spill must be clean up immediately - Tape down power cords or cable if any to prevent from tripping | 2 | 2 | 4 | Supervisor to make sure route of transporting before moving and double check clearing of all material and tooling | 2 | 1 | 2 | Supervisor on site | Estimate: 14-17 /11/23 |  |
| 10 | Unload case from rollers using jack | Machine topple due to lost of balance upon lifted up | Multiple Injuries & Damage to Property | - Workers to be alert when jack up load - Ensure load are jack up at the right angle of gravity - Jack up load to 4” high from one side to another is carried out in stages with the use of solid woodblock - Supervisor to ensure operation area is clear of unauthorised personnel | 4 | 2 | 8 | Only skill and experience worker to control the jack handling | 4 | 1 | 4 | Supervisor on site | Estimate: 14-17 /11/23 |  |