Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 2 Gul Circle | Mobile/Overhead Crane & Moving Operation | 1 | Transport of lifting machinery into or out of site premises |  |
|  |  |  | 2 | Setting up of crane |  |
|  |  |  | 3 | Lifting operation |  |
|  |  |  | 4 | Transportation of machine to designated location |  |
|  |  |  | 5 | Using forklift to assist in pulling/pushing machine |  |
|  |  |  | 6 | Working at height assemble machine |  |
|  |  |  | 7 | Using factory 20 ton overhead crane to install machine parts |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by : Eric Wong Signature | Reference Number: |
| --- | --- | --- | --- | --- |
| Process: | Mobile/Lorry Crane & Moving Operation | RA Member: Zailani |  |  |
| Activity Location: | 2 Gul Circle Cooper Cameron (S) Pte Ltd | RA Member: Daniel Wong |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: Eric Wong |  |
| Last Review Date: | 01/07/2020 |  | Designation: Director |  |
| Next Review Date | 30/06/2023 |  | Date: 21/08/2020 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Transport of lifting machinery into or out of site premises | Collision of moving vehicle onto personnel’s or site property | Bodily Injury & Damage to Property | - Vehicle only to be moved into site with the aid of banks man guiding the driver - Clear all obstruction in way of moving path - Drive at slow speed and observe speed limit when entering into customer premise | 4 | 2 | 8 | Vehicle must not be left unattended while engine is running | 4 | 1 | 4 | Wong Yen Siong / Koh Ban San | TBA |  |
| 2 | Setting up of crane | Hit by moving parts of crane | Bodily Injury & Damage to Property | - Operator trained and competent person appointed - Crane operator and lifting supervisor to ensure operation area and crane swing path is clear from any obstruction - Crane operator to be on site when extending or retracting outriggers (must be able to see all operation procedure) - Ensure all outrigger beams must be fully extended and resting on steel plate/timber mats as base plate - PPE (safety shoe, safety helmet) provided and put on | 3 | 2 | 6 | Ensure no unauthorised personnel or vehicle near moving parts of crane Barricade off the work area | 3 | 1 | 3 | Wong Yen Siong / Koh Ban San | TBA |  |
| 3 | Lifting operation | Failing of lifting equipment | Bodily Injury & Damage to Property | - All lifting equipment are tested by approved person and has valid certificate - Ensure LM for crane are valid - Riggers to ensure lifting gears use are in compliance with weight of machine - No defective gears allowed - Never stand close or under the load - PPE (safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | While moving or slewing the crane, try not to jerk the load Ensure lifting supervisor to be 100% on site to supervised the whole lifting operation | 4 | 1 | 4 | Wong Yen Siong / Koh Ban San | TBA |  |
|  |  | Overloading | Bodily Injury & Damage to Property | - No lifting beyond SWL of equipment - Load weight should be known - To test lift load about 1ft above ground to check on stability and to ensure no overloading of load to be lifted | 4 | 2 | 8 | Ensure load, capacity of lifting arm and reach are within limit, when in doubt, always use a higher capacity crane | 4 | 1 | 4 | Wong Yen Siong / Koh Ban San | TBA |  |
|  |  | Struck by objects due to unsafe slinging, rigging | Bodily Injury & Damage to Property | - Entire lifting crew is trained and to have relevant skill, certificate and experiences required to carry out tasks - Load must be securely slug, properly balanced and trimmed to prevent from dislodgment - Ensure machine is rig at the centre of gravity - Riggers to check and ensure safe rigging and lifting supervisor to confirm load are safe to be rig | 4 | 2 | 8 | Communication through hand signal or via walkie-talkie by Eric Wong personnel only | 4 | 1 | 4 | Wong Yen Siong / Koh Ban San | TBA |  |
|  |  | Hand caught by or in between loose part of sling or lifting gears | Arm or Hand Injury | - Keep hands off loose part of sling or lifting gears - Ensure signalman give proper means of signal to crane operator - Use glove | 3 | 2 | 6 | Signalman to command crane operator when to lift and supervisor to ensure all worker are hand off any loose part of lifting gears | 3 | 1 | 3 | Wong Yen Siong / Koh Ban San | TBA |  |
|  |  | View obstruction or load swing | Bodily Injury & Damage to Property | - Appointed and trained signalman, rigger and lifting supervisor on site - Tagline for load when necessary to prevent of load swing - Clear obstruction and unauthorised personnel in way of lifting path - Never stand close or under the suspended load and away from possible swing path | 4 | 2 | 8 | Signalman to position himself to be visible to crane operator through hand signalling method or via walkie-talkie | 4 | 1 | 4 | Wong Yen Siong / Koh Ban San | TBA |  |
| 4 | Transportation of machine to designated location | Unstable load during moving or turning | Bodily Injury & Damage to Property if machine topper | - Ensure rollers are place accordingly on machine base - Ensure rollers are properly secured to machine base with bolt and nut to prevent from rolling out - Ensure machine load are spread evenly and balanced well on the rollers before being transport to location - Workers to be alert and keep close lookout on route of travel from point to point and to ensure rollers are not moving out of position | 4 | 2 | 8 | Slow moving speed | 4 | 1 | 4 | Wong Yen Siong / Koh Ban San | TBA |  |
|  |  | Excessive weight load | Sprained back while pushing heavy load | - Use the correct capacity rollers to reduce the effort of each man when pushing the machine - Supervisor to ensure there are adequate manpower to be deploy for the job | 3 | 3 | 9 | Use machine puller or forklift if possible to assist in pulling/ pushing the machine | 3 | 2 | 6 | Wong Yen Siong / Koh Ban San | TBA |  |
|  |  | Passage way block by materials and tooling | Trip & Fall | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Oil/water spill must be clean up immediately | 2 | 2 | 4 | Supervisor to make sure route of the transporting before moving and to double check clearing of all materials and tooling | 2 | 1 | 2 | Wong Yen Siong / Koh Ban San | TBA |  |
|  |  | Obstruction to vision while moving | Damage to Property when hitting other obstacles | - To keep close lookout on route of travel - Ensure movement is guided - To alert personnel controlling the rollers steering if potential hitting of other obstacles | 2 | 2 | 4 | Provision of spotter at both end of machine so as to have a clearer view before hitting any obstacles | 2 | 1 | 2 | Wong Yen Siong / Koh Ban San | TBA |  |
| 5 | Using forklift to assist in pulling / pushing machine | Hitting people or property | Bodily Injury & Damage to Property | - Only certified and competent forklift operator are allowed to operate the forklift - Ensure working area is clear of obstruction or unauthorised personnel - Forklift operator to check whether anyone is nearby the forklift before proceeding to drive - Provision of spotter to guide the forklift operator when required | 4 | 2 | 8 | Press horn when necessary to warn others that forklift is approaching | 4 | 1 | 4 | Wong Yen Siong / Koh Ban San | TBA |  |
| 6 | Working at height to assemble machine | Fall from height | Bodily Injury | - Provision of boom lift as working platform to work at height - Only certified and competent boom lift operator are allowed to operate the boom lift - Worker working at height are trained and must have WAH course - Worker working at height on top boom lift to put on proper PPE (safety harness, safety helmet) - Ensure safety harness use are properly strapped on and hook to strong anchorage point at all time | 4 | 2 | 8 | All trained and authorised operator record to be paste on the boom lift | 4 | 1 | 4 | Wong Yen Siong / Koh Ban San | TBA |  |
| 7 | Using factory 20 ton overhead crane to install machine parts | Failing of lifting equipment | Bodily Injury & Damage to Property | - All lifting equipment use are tested by approved person and has valid certificate - Riggers to ensure lifting gears use are in compliance with weight of machine - No defective gears allowed - Never stand close or under the load - PPE (safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | While moving or slewing the crane, try not to jerk the load | 4 | 1 | 4 | Wong Yen Siong / Koh Ban San | TBA |  |
|  |  | Overloading | Bodily Injury & Damage to Property | - Ensure overloading alarm device for factory overhead crane is functioning - No lifting beyond SWL of overhead crane - Load weight should be known | 4 | 2 | 8 | Ensure load, capacity of overhead crane are within limit, when in doubt, always use a higher capacity crane | 4 | 1 | 4 | Wong Yen Siong / Koh Ban San | TBA |  |
|  |  | Struck by load due to unsafe slinging, rigging | Bodily Injury & Damage to Property | - Entire lifting crew is trained and to have relevant skill, certificate and experiences required to carry out tasks - Load must be securely slug, properly trimmed to prevent from dislodgment - Ensure load is rig at the centre of gravity - Riggers to check and ensure safe rigging and lifting supervisor to confirm load are safe to be rig | 4 | 2 | 8 | Communication through hand signal or via walkie-talkie by Eric Wong personnel only | 4 | 1 | 4 | Wong Yen Siong / Koh Ban San | TBA |  |
|  |  | Load swing | Bodily Injury & Damage to Property | - Tagline for load when necessary to prevent of load swing - Clear obstruction and unauthorised personnel in way of lifting path - Never stand close or under the suspended load and away from possible swing path - PPE (safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | Signalman to position himself to be visible to crane operator through hand signalling method or via walkie-talkie | 4 | 1 | 4 | Wong Yen Siong / Koh Ban San | TBA |  |