Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 5 Engineering Drive | Lifting / Moving Operation | 1 | Transport of lifting machinery into or out of site premises |  |
|  |  |  | 2 | Setting up of crane |  |
|  |  |  | 3 | Lifting operation |  |
|  |  |  | 4 | Transportation of machine to designated location |  |
|  |  |  | 5 | Unload machine using crowbar |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader : Kevin Wong | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Lifting & Moving Operation | RA Member : Zailani |  |  |  |
| Location: | 5 Engineering Drive, Blk EA, Room E4 Blk EA, Room EA-03-02 | RA Member :Zaharin |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 15/05/2015 |  | Designation: | Director |  |
| Next Review Date | 15/05/2018 |  | Date: | 15/05/2017 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Transport of lifting machinery into or out of site premises | Collision of moving vehicle onto personnel’s or site property | Bodily Injury & Damage to Property | - Vehicle only to be moved into site with the aid of banks man guiding the driver - Clear all obstruction in way of moving path - Drive at slow speed when entering into customer premise | 4 | 2 | 8 | Vehicle must not be left unattended while engine is running | 4 | 1 | 4 | Supervisor on site | 18/05/17 | NUS personnel to ensure loading/ unloading area are clear on moving day |
| 2 | Setting up of crane | Hit by moving parts of crane | Bodily Injury & Damage to Property | - Operator trained and competent person appointed - Crane operator to be on site when extending or retracting outriggers (must be able to see all operation procedure) - Ensure all outrigger beams must be fully extended and resting on steel plate/timber mats - PPE (etc.. safety shoe, safety helmet) provided and put on | 3 | 2 | 6 | Ensure no unauthorised personnel or vehicle near moving parts of crane | 3 | 1 | 3 | Lifting supervisor on site | 18/05/17 |  |
| 3 | Lifting operation | Failing of lifting equipment | Bodily Injury & Damage to Property | - All lifting equipment are tested by approved person and has valid certificate - Ensure LM for lorry crane are valid - Riggers to ensure lifting gears use are in compliance with weight of machine - No defective gears allowed - Never stand close or under the load - Lifting zone to be barricade, only relevant personnel is allowed to enter the lifting zone - PPE (etc… safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | While moving or slewing the crane, try not to jerk the load | 4 | 1 | 4 | Lifting supervisor on site | 18/05/17 |  |
|  |  | Overloading | Bodily Injury & Damage to Property | - No lifting beyond SWL of equipment - Load weight should be known | 4 | 2 | 8 | Ensure load, capacity of lifting arm and reach are within limit, when in doubt, always use a higher capacity crane | 4 | 1 | 4 | Lifting supervisor on site | 18/05/17 |  |
|  |  | Struck by objects due to unsafe slinging, rigging | Bodily Injury & Damage to Property | - Entire lifting crew is trained and to have relevant skill, certificate and experiences required to carry out tasks - Load shall be securely slug, properly balanced and trimmed to prevent dislodgment - Ensure machine is rig at it centre of gravity | 4 | 2 | 8 | Communication through hand signal or via walkie-talkie by Eric Wong personnel only | 4 | 1 | 4 | Lifting supervisor on site | 18/05/17 |  |
|  |  | Hand caught by or in between loose part of sling or lifting gears | Hand Injury | - Keep hands off loose part of sling or lifting gears - Ensure signalman give proper means of signal to crane operator - Use cotton glove | 3 | 2 | 6 | Signalman to command crane operator when to lift and to ensure all workers are hands off loose part of lifting gears | 3 | 1 | 3 | Lifting supervisor on site | 18/05/17 |  |
|  |  | View obstruction or poor visibility, load swing | Bodily Injury & Damage to Property | - Appointed and trained signalman, riggers and lifting supervisor on site - Tagline for load when necessary to prevent of load swing - Clear obstruction and unauthorised personnel in way of lifting path - Never stand close or under the suspended load and away from possible swing path - PPE (etc.. safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | Signalman to position himself to be visible to crane operator through hand signalling method or via walkie-talkie | 4 | 1 | 4 | Lifting supervisor on site | 18/05/17 |  |
| 4 | Transportation of machine to designated location | Unstable load during moving, turning | Bodily Injury & Damage to Property when machine fall | - Ensure pallet truck are place accordingly on machine base - Ensure machine load are spread evenly and is stable on the pallet truck before being transport to location - Workers to be alert and keep close lookout on route of travel from point to point | 4 | 2 | 8 | Move slow and steady | 4 | 1 | 4 | Supervisor on site | 18/05/17 |  |
|  |  | Excessive weight load | Sprained back while pushing machines | - Use the correct capacity pallet truck to reduce the effort of each man when pushing the machine - Deploy adequate personnel | 3 | 3 | 9 | Use machine puller or forklift if possible to assist in pulling/pushing the machine | 3 | 2 | 6 | Supervisor on site | 18/05/17 |  |
|  |  | Passage way block by materials and tooling | Trip & Fall | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Oil/water spill must be clean up immediately | 2 | 2 | 4 | Supervisor to make sure of the route of the transporting before moving and to double check clearing of all materials and tooling | 2 | 1 | 2 | Supervisor on site | 18/05/17 |  |
|  |  | Obstruction to vision while moving | Damage to Property when hitting other machine or obstacles | - To keep close lookout on route of travel - Ensure movement is guided - To alert personnel controlling the rollers/pallet truck steering if potential hitting of other obstacles | 2 | 2 | 4 | Provision of spotter on both end of machine so as to have a clearer view before it touches any obstacles | 2 | 1 | 2 | Supervisor on site | 18/05/17 |  |
| 5 | Unload machine using crowbar | Machine topple due to lost of balance upon lifted up | Bodily Injury & Damage to Property | - Workers to be alert when lift up machine - Ensure machine are lift up at the right angle of gravity - Lower down machine from one side to another is carried out in stages with the use of hard wooden block | 4 | 2 | 8 | Only skill and experience worker to control the crowbar handling | 4 | 1 | 4 | Supervisor on site | 18/05/17 |  |