Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | No 18 Tractor Rd | Lifting / Moving Operation | 1 | Transport of lifting machinery into or out of site premises |  |
|  |  |  | 2 | Un-crating of case |  |
|  |  |  | 3 | Setting up of crane outriggers |  |
|  |  |  | 4 | Lifting operation |  |
|  |  |  | 5 | Transportation of machine to designated location |  |
|  |  |  | 6 | Unload machine using crowbar |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Lifting & Moving Operation | RA Member: Zailani |  |  |  |
| Location: | No 18 Tractor Rd NOK Precision Component (S) Pte Ltd | RA Member: Zaharin |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2017 |  | Designation: | Director |  |
| Next Review Date | 30/06/2020 |  | Date: | 07/03/2018 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Transport of lifting machinery into or out of site premises | Collision of moving vehicle onto personnel’s or site property | Bodily Injury & Damage to Property | - Vehicle only to be moved into site with the aid of banks man guiding the driver - Clear all obstruction in way of moving path - Drive at slow speed when entering into customer premise | 4 | 2 | 8 | Vehicle must not be left unattended while engine is running. | 4 | 1 | 4 | Supervisor on site (Zaharin / Ibrahim) | 15/03/18 |  |
| 2 | Un-crate case | Contact with sharp edges/ wooden splinters | Cut | - Keep hand and finger clear of sharp edges - Remove sharp edges (etc...nails, metal strips) from working area as soon as possible - PPE (safety shoe, safety helmet, cotton glove) provided and put on | 2 | 3 | 6 | Close supervision by supervisor to advise workers where the sharp edges are before uncrating. | 2 | 1 | 2 | Supervisor on site (Zaharin / Ibrahim) | 15/03/18 |  |
|  |  | Falling of wooden plank/timber | Bodily Injury | - PPE (safety shoe, safety helmet) provided and put on - Un-crate and dismantle case in the correct sequence - Proper housekeeping to be maintained during work - Ensure no unauthorised personnel near working area | 3 | 2 | 6 | Communicate between workers before removing any plank or timber | 3 | 1 | 3 | Supervisor on site (Zaharin / Ibrahim) | 15/03/18 |  |
| 3 | Setting up of crane | Hit by moving parts of crane | Bodily Injury & Damage to Property | - Operator trained and competent person appointed - Crane operator to be on site when extending or retracting outriggers (must be able to see all operation procedure) - Ensure all outrigger beams must be fully extended and resting on steel plate/timber mats - PPE (safety shoe, safety helmet) provided and put on | 3 | 2 | 6 | Ensure no unauthorised personnel or vehicle near moving parts of crane. Cordon off the work area | 3 | 1 | 3 | Lifting supervisor on site (Zaharin / Ibrahim) | 15/03/18 |  |
| 4 | Lifting operation | Failing of lifting equipment | Bodily Injury & Damage to Property | - All lifting equipment are tested by approved person and has valid certificate - Ensure LM for lorry crane are valid - Riggers to ensure lifting gears use are in compliance with weight of machine - No defective gears allowed - Never stand close or under the load - Lifting zone to be barricade, only relevant personnel is allowed to enter the lifting zone - PPE (safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | While moving or slewing the crane, try not to jerk the load | 4 | 1 | 4 | Lifting supervisor on site (Zaharin / Ibrahim) | 15/03/18 |  |
|  |  | Overloading | Bodily Injury & Damage to Property | - No lifting beyond SWL of equipment - Load weight should be known | 4 | 2 | 8 | Ensure load, capacity of lifting arm and reach are within limit, when in doubt, always use a higher capacity crane To lower down machine if overloading siren is on | 4 | 1 | 4 | Lifting supervisor on site (Zaharin / Ibrahim) | 15/03/18 |  |
|  |  | Struck by objects due to unsafe slinging, rigging | Bodily Injury & Damage to Property | - Entire lifting crew is trained and to have relevant skill, certificate and experiences required to carry out tasks - Load shall be securely slug, properly balanced and trimmed to prevent dislodgment - Ensure machine is rig at it centre of gravity | 4 | 2 | 8 | Communication through hand signal or via walkie-talkie by Eric Wong personnel only | 4 | 1 | 4 | Lifting supervisor on site (Zaharin / Ibrahim) | 15/03/18 |  |
|  |  | Hand caught by or in between loose part of sling or lifting gears | Arm or Hand Cut or Abrasion | - Keep hands off loose part of sling or lifting gears - Ensure signalman give proper means of signal to crane operator - Use cotton glove | 3 | 2 | 6 | Signalman to command crane operator when to lift and to ensure all workers are hand off any loose part of lifting gears | 3 | 1 | 3 | Lifting supervisor on site (Zaharin / Ibrahim) | 15/03/18 |  |
|  |  | View obstruction or poor visibility, load swing | Bodily Injury & Damage to Property when Machine Topple | - Appointed and trained signalman, riggers and lifting supervisor on site - Tagline for load when necessary to prevent of load swing - Clear obstruction and unauthorised personnel in way of lifting path - Never stand close or under the suspended load and away from possible swing path - PPE (safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | Signalman to position himself to be visible to crane operator through hand signalling method or via walkie-talkie Cordon off hoisting area and to make sure only lifting team are in sight of the lifting. | 4 | 1 | 4 | Lifting supervisor on site (Zaharin / Ibrahim) | 15/03/18 |  |
| 5 | Transportation of machine to designated location | Unstable load during moving, turning and uneven grounds | Bodily Injury & Damage to Property when machine fall | - Ensure pallet truck are place accordingly on machine base - Ensure machine load are spread evenly and is stable on the pallet truck before being transport to location - Workers to be alert and keep close lookout on route of travel from point to point | 4 | 2 | 8 | Slow and steady moving speed Supervisor to take note of the pathway which machine is moving towards and to make sure machines load always to be spread evenly and stable while being transported to location | 4 | 1 | 4 | Supervisor on site (Zaharin / Ibrahim) | 15/03/18 |  |
|  |  | Excessive weight load | Sprained back while pushing heavy load | - Use the correct capacity pallet truck to reduce the effort of each man when pushing the machine - Supervisor to ensure adequate personnel are deploy for the job | 3 | 3 | 9 | Use machine puller or forklift if possible to assist in pulling/pushing the machine | 3 | 2 | 6 | Supervisor on site (Zaharin / Ibrahim) | 15/03/18 |  |
|  |  | Passage way block by materials and tooling | Trip & Fall | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Oil/water spill must be clean up immediately | 2 | 2 | 4 | Close supervision by supervisor Supervisor to make sure of the route of the transporting before moving and to double check clearing of all materials and tooling | 2 | 1 | 2 | Supervisor on site (Zaharin / Ibrahim) | 15/03/18 |  |
|  |  | Obstruction to vision while moving | Damage to Property while pushing / pulling to machines | - To keep close lookout on route of travel - Ensure movement is guided - To alert personnel controlling the roller steering if potential hitting of other obstacles | 2 | 2 | 4 | Provision of spotter on both side so as to have a clearer view on the machine before hitting any obstacles | 2 | 1 | 2 | Supervisor on site (Zaharin / Ibrahim) | 15/03/18 |  |
| 6 | Unload machine using crowbar | Machine topple due to lost of balance upon lifted up | Bodily Injury & Damage to Property | - Workers to be alert when lift up machine - Ensure machine are lift up at the right angle of gravity - Lower down machine from one side to another is carried out in stages with the use of hard wooden block | 4 | 2 | 8 | Only skill and experience worker to control the crowbar handling | 4 | 1 | 4 | Supervisor on site (Zaharin / Ibrahim) | 15/03/18 |  |