Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | Blk 5 A.M.K Ind Pk 2A | Moving operation via loading bay | 1 | Transport of lifting machinery into or out of site premise |  |
|  |  |  | 2 | Reverse lorry to loading bay |  |
|  |  |  | 3 | Use crane to assist machine down slope |  |
|  |  |  | 4 | Shift out machine from lorry chassis and transport to location via cargo lift |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Moving operation via loading bay | RA Member: Zailani |  |  |  |
| Location: | Blk 5 A.M.K Ind Pk 2A #02-15 Yi Fong Precision Pte Ltd | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2023 |  | Designation: | Director |  |
| Next Review Date | 30/06/2026 |  | Date: | 10/12/2025 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Transport of lifting machinery into or out of site premises | Collision of moving vehicle onto personnel or site property | Bodily injury & damage to property | - To coordinate with the occupier for safe route access - Vehicle only to be moved into site with the aid of banks man guiding the driver - Clear all obstruction in way of moving path - Driver to drive at slow speed and observe speed limit when entering into customer premise | 4 | 2 | 8 | Vehicle must not be left unattended while engine is running | 4 | 1 | 4 | Supervisor on site | 20/12/25 |  |
| 2 | Reverse lorry to loading bay | Restricted driver visibility | Bodily injury & damage to property | - Clear all obstruction in way of reversing path - Slow speed when reversing lorry to loading bay - Ensure driver is guided by other staff to ensure no hitting of any obstruction while reversing to loading bay | 3 | 2 | 6 | Assign traffic controller to control movement of vehicle or personnel within the vicinity when required | 3 | 1 | 3 | Supervisor on site | 20/12/25 |  |
| 3 | Use crane to assist machine down slope | Hitting people or object | Bodily injury & damage to property | - Wire/webbing sling are attached to crane hook and machine to control the moving motion while moving down the slope from lorry chassis to loading bay - Sling use are tested and in good working condition - To ensure no unauthorised personnel is within the working area | 4 | 2 | 8 | Close monitoring and supervision by supervisor | 4 | 1 | 4 | Supervisor on site | 20/12/25 |  |
| 4 | Shift out machine from lorry chassis and transport to 2nd floor via cargo lift | Unstable load while moving or turning | Bodily injury & damage to property if machine topper | - Ensure pallet truck are place accordingly on machine bed - Ensure machine load are spread evenly and balanced well on the pallet truck - Buddy system/one to control the pallet truck steering while the others to push and hold onto the machine - To ensure adequate personnel are to be deploy for the job - Workers to be alert and keep close lookout on route of travel from point to point | 4 | 2 | 8 | Slow moving speed Supervisor to take note of the pathway which machine is moving towards and to make sure machine load always stable on the pallet truck while being transport to location | 4 | 1 | 4 | Supervisor on site | 20/12/25 |  |
|  |  | Trip & fall | Sprains & strains | - Passage way and working area are to be keep clear of any materials and tooling before moving job - Proper housekeeping to be maintained during work - Keep the floor in the work area dry and clean - Oil/water spill must be clean up immediately | 2 | 2 | 4 | Good coordination between workers to ensure all materials or tooling are move aside before any moving are done | 2 | 1 | 2 | Supervisor on site | 20/12/25 |  |
|  |  | Obstruction to vision while moving | Damage to property when hitting other obstacle | - Supervisor to ensure operation area is clear of unauthorised personnel - To keep close lookout on route of travel - Ensure movement is guided - To alert personnel controlling the pallet truck steering if potential hitting of other obstacles | 1 | 2 | 2 | Provision of spotter at both end of machine so as to have a clearer view before hitting any obstacles | 1 | 2 | 2 | Supervisor on site | 20/12/25 |  |