Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 110 Tuas South Ave 3 | Mobile Crane & Moving Operation | 1 | Transport of lifting machinery into or out of site premises |  |
|  |  |  | 2 | Setting up of crane |  |
|  |  |  | 3 | Lifting operation – to hoist down case from container |  |
|  |  |  | 4 | Uncrate case |  |
|  |  |  | 5 | Lifting operation – to hoist and detach machine from base |  |
|  |  |  | 6 | Signalling of load |  |
|  |  |  | 7 | Insert roller under machine bed |  |
|  |  |  | 8 | Transport machine to designated location using rollers |  |
|  |  |  | 9 | Unload machine from rollers using jack |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by : Eric Wong Signature | Reference Number: EW/Q/0908/ 24 |
| --- | --- | --- | --- | --- |
| Process: | Mobile Crane & Moving Operation | RA Member: Zailani |  |  |
| Activity Location: | 110 Tuas South Ave 3 #01-06 RTHT Technologies Pte Ltd | RA Member: Daniel Wong |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: Eric Wong |  |
| Last Review Date: | 01/07/2023 |  | Designation: Director |  |
| Next Review Date | 30/06/2026 |  | Date: 26/08/2024 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Transport of lifting machinery into or out of site premises | Collision of moving vehicle onto personnel’s or site property | Multiple Injuries & Damage to Property | - To coordinate with the occupier for safe route access - Vehicle only to be moved into site with the aid of banks man guiding the driver - Clear all obstruction in way of moving path - Driver to observe speed limit when entering into customer premise | 4 | 2 | 8 | Vehicle must not be left unattended while engine is running | 4 | 1 | 4 | Supervisor on site | 29/08/24 |  |
| 2 | Setting up of crane | Hit by moving parts of crane | Multiple Injuries & Damage to Property | - Operator trained and competent person appointed - Crane operator and lifting supervisor to determine the load radius and slewing part of the crane to ensure no hitting of nearby building or equipment - To ensure operation area are clear from any obstruction - Crane operator to be on site when extending or retracting outriggers (must be able to see all operation procedure) - Ensure all outrigger beams must be fully extended and resting on steel plate/timber mat as base plate - PPE (safety shoe, safety helmet) provided and put on | 5 | 2 | 10 | Ensure no unauthorised personnel or vehicle near moving parts of crane Barricade off the work area Signage to be put up at the barricade area and if necessary a stand-by man to direct traffic or personnel | 5 | 1 | 5 | Lifting supervisor on site | 29/08/24 |  |
| 3 | Lifting operation – to hoist down case from container | Failing of lifting equipment | Multiple Injuries & Damage to Property | - All lifting equipment are tested by approved person and has valid certificate - Ensure LM for mobile/ lorry crane use are valid - Riggers to ensure lifting gears use are in compliance with weight of equipment - No defective gears allowed - Never stand close or under the load - PPE (safety helmet, safety shoe) provided and put on | 5 | 2 | 10 | While moving or slewing the crane, try not to jerk the load Ensure lifting supervisor to be 100% onsite to supervisor the whole lifting operation No lifting is to be carried out during bad weather | 5 | 1 | 5 | Lifting supervisor on site | 29/08/24 |  |
|  |  | Overloading | Multiple Injuries & Damage to Property | - No lifting beyond SWL of equipment - Load weight should be known - To test lift load about 1ft above ground to check on stability and to ensure no overloading of load to be lifted - Implement of Lifting Plan to all lifting personnel on site to ensure that the lifting operation is properly manage | 5 | 2 | 10 | Ensure load, capacity of lifting arm and reach are within limit, when in doubt, always use a higher capacity crane | 5 | 1 | 5 | Lifting supervisor on site | 29/08/24 |  |
|  |  | Struck by objects due to unsafe slinging, rigging | Multiple Injuries & Damage to Property | - Entire lifting crew is trained and to have relevant skill, certificate and experiences required to carry out tasks - Machine must be securely slug, properly balanced and trimmed to prevent from dislodgment - Ensure machine is rig at the centre of gravity - Riggers to check and ensure safe rigging and lifting supervisor to confirm load are safe to be rig | 5 | 2 | 10 | Communication through hand signal or via walkie-talkie by Eric Wong personnel only To provide crane hook with safety latch to prevent displacement of sling or load from the hook | 5 | 1 | 5 | Lifting supervisor on site | 29/08/24 |  |
|  |  | Hand caught by or in between loose part of sling or lifting gears | Minor fracture to hand and arm | - Keep hands off loose part of sling or lifting gears - Ensure signalman give proper means of signal to crane operator - Use glove | 3 | 2 | 6 | Signalman to command crane operator when to lift and supervisor to ensure all worker are hand off any loose part of lifting gears | 3 | 1 | 3 | Lifting supervisor on site | 29/08/24 |  |
| 4 | Uncrate case | Falling of wooden plank | Multiple Injuries & Damage to Property | - PPE (safety shoe, safety helmet, glove) provided and put on - Uncrate and dismantle case in the correct sequence - Using lorry crane to act as support to lower down the side plank - Ensure all workers to stay away from the case - To remove immediately all loose plank - Proper housekeeping to be maintained during work - To ensure no unauthorised personnel is near the working area | 3 | 2 | 6 | Good coordination between workers and close supervision by supervisor Supervisor to ensure nobody is underneath the case before using of lorry crane to push the side plank down | 3 | 1 | 3 | Supervisor on site | 29/08/24 |  |
|  |  | Uncrate top cover of case | Multiple Injuries & Damage to Property | - Provision of scissor lift to work on which is unreachable from ground to uncrate top cover of case - Only trained and certified operator are allowed to operate the scissor lift - Use of safety harness and hook onto strong anchorage point of the scissor lift at all time while working to uncrate the top cover of case | 5 | 2 | 10 | Provision of alternative anchorage point, use of safety harness and secured onto lorry crane hook at all time to act as support while working to uncrtae top cover of case | 5 | 1 | 5 | Supervisor on site | 29/08/24 |  |
| 5 | Lifting operation – to hoist and detach machine from base | Failing of lifting equipment | Multiple Injuries & Damage to Property | - All lifting equipment are tested by approved person and has valid certificate - Ensure LM for mobile/ lorry crane use are valid - Riggers to ensure lifting gears use are in compliance with weight of equipment - No defective gears allowed - Never stand close or under the load - PPE (safety helmet, safety shoe) provided and put on | 5 | 2 | 10 | While moving or slewing the crane, try not to jerk the load Ensure lifting supervisor to be 100% onsite to supervisor the whole lifting operation No lifting is to be carried out during bad weather | 5 | 1 | 5 | Lifting supervisor on site | 29/08/24 |  |
|  |  | Overloading | Multiple Injuries & Damage to Property | - No lifting beyond SWL of equipment - Load weight should be known - To test lift load about 1ft above ground to check on stability and to ensure no overloading of load to be lifted - Implement of Lifting Plan to all lifting personnel on site to ensure that the lifting operation is properly manage | 5 | 2 | 10 | Ensure load, capacity of lifting arm and reach are within limit, when in doubt, always use a higher capacity crane | 5 | 1 | 5 | Lifting supervisor on site | 29/08/24 |  |
|  |  | Struck by objects due to unsafe slinging, rigging | Multiple Injuries & Damage to Property | - Entire lifting crew is trained and to have relevant skill, certificate and experiences required to carry out tasks - Machine must be securely slug, properly balanced and trimmed to prevent from dislodgment - Ensure machine is rig at the centre of gravity - Riggers to check and ensure safe rigging and lifting supervisor to confirm load are safe to be rig | 5 | 2 | 10 | Communication through hand signal or via walkie-talkie by Eric Wong personnel only To provide crane hook with safety latch to prevent displacement of sling or load from the hook | 5 | 1 | 5 | Lifting supervisor on site | 29/08/24 |  |
|  |  | Hand caught by or in between loose part of sling or lifting gears | Minor fracture to hand and arm | - Keep hands off loose part of sling or lifting gears - Ensure signalman give proper means of signal to crane operator - Use glove | 3 | 2 | 6 | Signalman to command crane operator when to lift and supervisor to ensure all worker are hand off any loose part of lifting gears | 3 | 1 | 3 | Lifting supervisor on site | 29/08/24 |  |
| 6 | Signalling of load | View obstruction or load swing | Multiple Injuries & Damage to Property | - Appointed and trained signalman, rigger and lifting supervisor on site - Tagline for load when necessary to prevent of load swing - Clear obstruction and unauthorised personnel in way of lifting path - Never stand close or under the suspended load and away from possible swing path - PPE (safety helmet, safety shoe) provided and put on | 5 | 2 | 10 | Signalman to position himself to be visible to crane operator through hand signalling method or via walkie-talkie whenever signalman is not visible to crane operator | 5 | 1 | 5 | Lifting supervisor on site | 29/08/24 |  |
| 7 | Insert rollers under machine bed | Sharp edges of machine | Cuts from sharp edges | - To keep hand/finger clear of sharp edges - Insert rollers to below machine base with extra care - Use glove - Tighten bolts and nuts with adjustable spanner | 2 | 2 | 4 | Close supervisor by supervisor | 2 | 2 | 4 | Supervisor on site | 29/08/24 |  |
|  |  | Pinch point | Injury to hand | - PPE (safety glove) provided and put on - Never place hand where you can’t see them - If view is obstructed, do not proceed with the work until having a clearer view - To standby solid woodblock below machine and remove only after rollers are properly secured onto machine bed | 3 | 2 | 6 | Always stay vigilant and pay close attention to surrounding area | 3 | 1 | 3 | Supervisor on site | 29/08/24 |  |
| 8 | Transport machine to designated location using rollers | Unstable load during moving or turning | Multiple Injuries & Damage to Property if machine topper | - Ensure rollers are place accordingly on machine base - Ensure rollers are properly secured onto machine base with bolts and nuts - Tighten bolts and nuts with adjustable spanner - Ensure machine load are spread evenly and balance well on the rollers before being transport to location - Workers to be alert and keep close lookout on route of travel from point to point and to ensure rollers are not moving out of position | 4 | 2 | 8 | Worker controlling the roller controller to understand and know how to properly operate the control Slow moving speed | 4 | 1 | 4 | Supervisor on site | 29/08/24 |  |
|  |  | Trip & fall | Sprains & strains | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Keep the floor in the work area dry and clean - Oil/water spill must be clean up immediately - Tape down power cords or cable if any to prevent from tripping | 2 | 2 | 4 | Supervisor to make sure route of transporting before moving and double check clearing of all material and tooling | 2 | 1 | 2 | Supervisor on site | 29/08/24 |  |
|  |  | Obstruction to vision while moving | Damage to Property when hitting other obstacles | - To keep close lookout on route of travel - Ensure movement is guided - To alert personnel controlling the roller steering if potential of other obstacles | 2 | 2 | 4 | Provision of spotter at both end of machine so as to have a clearer view before hitting any obstacles | 2 | 1 | 2 | Supervisor on site | 29/08/24 |  |
| 9 | Unload machine from rollers using jack | Machine topple due to lost of balance upon lifted up | Multiple Injuries & Damage to Property | - Workers to be alert when jack up load - Ensure load are jack up at the right angle of gravity - Jack up load to 4” high from one side to another is carried out in stages with the use of solid woodblock - Supervisor to ensure operation area is clear of unauthorised personnel | 4 | 2 | 8 | Only skill and experience worker to control the jack handling | 4 | 1 | 4 | Supervisor on site | 29/08/24 |  |