Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 61 Science Park Rd | Moving & lifting operation | 1 | Fixing of ‘A’ frame structure |  |
|  |  |  | 2 | Lift up machine using chain block |  |
|  |  |  | 3 | Transportation of machine to loading area |  |
|  |  |  | 4 | Lifting operation |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong | Approved by : Eric Wong Signature: | Reference Number: |
| --- | --- | --- | --- | --- |
| Process: | Moving & Lifting Operation | RA Member: Zailani |  |  |
| Activity Location: | 61 Science Park Rd #01-03 | RA Member: Zaharin |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: Eric Wong |  |
| Last Review Date: | 01/07/2017 |  | Designation: Director |  |
| Next Review Date | 30/06/2020 |  | Date: 26/08/2019 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Fixing of ‘A’ frame structure | Manual handling of heavy equipment | Sprain back when carrying heavy load | - Supervisor to ensure there are adequate manpower to be deploy for the job - Buddy system when handling of heavy equipment | 3 | 2 | 6 | Use of mechanical aid such as lifter if required | 3 | 1 | 3 | Supervisor on site | 28/08/19 |  |
|  |  | Working at height to fix bolt and nut onto frame | Falling from height | - Use of ‘A’ frame ladder to work on which is unreachable from ground - Ladder use must be fully extended and stand on level ground - Buddy system- one to hold onto the ladder while the other is working on top of ladder | 4 | 2 | 8 | Close supervision by supervisor | 4 | 1 | 4 | Supervisor on site | 28/08/19 |  |
| 2 | Lift up engine using chain block | Failing of lifting equipment | Bodily injury & damage to property | - All lifting equipment use are tested by approved person and has valid certificate - To ensure lifting gears use are in compliance with weight of engine - No lifting beyond SWL of equipment - Ensure no unauthorised personnel is near working area | 4 | 2 | 8 | When lift up engine, try not to jerk the load | 4 | 1 | 4 | Supervisor on site | 28/08/19 |  |
|  |  | Struck by load due to unsafe slinging | Bodily injury & damage to property | - Entire crew is trained and to have relevant skill and experience to carry out task - Load must be securely slug and properly balanced to prevent from dislodgment - Ensue load is rig at the centre of gravity | 4 | 2 | 8 | Close supervision and regular reminder by supervisor to ensure all safety precaution are in place | 4 | 1 | 4 | Supervisor on site | 28/08/19 |  |
| 3 | Transportation of engine to loading area | Unstable load during moving or turning | Bodily injury & damage to property | - Ensure pallet truck are place accordingly on engine base - Ensure engine load are spread evenly and balanced well on the pallet truck before being transport to loading bay - Workers to be alert and keep close lookout on route of travel | 4 | 2 | 8 | Slow transportation speed | 4 | 1 | 4 | Supervisor on site | 28/08/19 |  |
|  |  | Passageway block by equipment and or tools | Trip & fall | - Passageway and working area to be keep clear of materials or tooling before moving job - Proper housekeeping to be maintained during work - Oil/water spill must be clean up immediately | 2 | 2 | 4 | Supervisor to make sure route of transporting before moving and double check clearing of all material or tooling | 2 | 1 | 2 | Supervisor on site | 28/08/19 |  |
| 4 | Lifting operation | Failing of lifting equipment | Bodily injury & damage to property | -All lifting equipment are tested by approved person and has valid certificate - Rigger to ensure lifting gears use are in compliance with weight of engine to be hoist - Never stand close or under the load - PPE (safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | While moving or slewing the crane, try not to jerk the load | 4 | 1 | 4 | Lifting supervisor on site | 28/08/19 |  |
|  |  | Struck by objects due to unsafe slinging | Bodily injury & damage to property | - Entire lifting crew is trained and to have relevant skill, certificate and experiences required to carry out tasks - Load must be securely slug, properly balanced and trimmed to prevent from dislodgment - Ensure load is rig at the centre of gravity | 4 | 2 | 8 | Communication through hand signal or via walkie-talkie by Eric Wong personnel only | 4 | 1 | 4 | Lifting supervisor on site | 28/08/19 |  |
|  |  | Load swing | Bodily injury & damage to property | - Appointed and trained lifting supervisor, rigger and signalman on site - Tagline for load when necessary to prevent of load swing - Clear obstruction and unauthorised personnel in way of lifting path - Never stand close or under the suspended load and away from possible swing path | 4 | 2 | 8 | Signalman to position himself to be visible to crane operator through hand signalling method or via walkie-talkie | 4 | 1 | 4 | Lifting supervisor on site | 28/08/19 |  |