Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 16 Seletar Aerospace | Lorry Crane & Moving Operation | 1 | Transport of lifting machinery into or out of site premises |  |
|  |  |  | 2 | Jack up machine using hydraulic/hand jack |  |
|  |  |  | 3 | Insert rollers under machine bed |  |
|  |  |  | 4 | Transport machine to loading area |  |
|  |  |  | 5 | Setting up of crane on site |  |
|  |  |  | 6 | Insert sling to crane hook and secured to rigging point of machine |  |
|  |  |  | 7 | Lifting operation |  |
|  |  |  | 8 | Signalling of load |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by: Eric Wong Signature | Reference Number: |
| --- | --- | --- | --- | --- |
| Process: | Lorry Crane & Moving Operation | RA Member: Zailani |  |  |
| Activity Location: | 16 Seletar Aerospace JEP Precision Engineering Pte Ltd | RA Member: Daniel Wong |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: Eric Wong |  |
| Last Review Date: | 01/07/2023 |  | Designation: Director |  |
| Next Review Date | 30/06/2026 |  | Date: 15/01/2025 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Transport of lifting machinery into or out of site premises | Collision of moving vehicle onto personnel’s or site property | Bodily Injury & Damage to Property | - To coordinate with the occupier for safe route access - Vehicle only to be moved into site with the aid of banks man guiding the driver - Clear all obstruction in way of moving path - Driver to observed speed limit when entering into customer premise | 4 | 2 | 8 | Vehicle must not be left unattended while engine is running | 4 | 1 | 4 | Supervisor on site | 20-21 /01/25 & 24/01/25 |  |
| 2 | Jack up machine using hydraulic/hand jack | Machine topple due to lost of balance upon lifted up | Bodily Injury & Damage to Property | - Workers to be alert when jack up machine - Ensure machine are jack up at the right angle of gravity - Jack up machine to 4” high from one side to another is carried out in stages with the use of solid woodblock to be insert below machine bed - Supervisor to ensure operation area is clear of unauthorised personnel | 5 | 2 | 10 | Only skill and experience worker to control the jack handling | 5 | 1 | 5 | Supervisor on site | 20-21 /01/25 & 24/01/25 |  |
| 3 | Insert rollers under machine bed | Sharp edges of machine | Cuts form sharp edges | - To keep hand/finger clear of sharp edges - Insert rollers to below machine with extra care - Use glove | 2 | 2 | 4 | Close supervision by supervisor | 2 | 2 | 4 | Supervisor on site | 20-21 /01/25 & 24/01/25 |  |
|  |  | Pinch point | Injury to hand | - PPE (safety glove) provided and put on - Never place hand where you can’t see them - If view is obstructed, do not proceed with the work until having a clearer view - To standby solid woodblock below machine and remove only after rollers are properly secured onto machine bed | 3 | 2 | 6 | Always stay vigilant and pay close attention to surrounding area | 3 | 1 | 3 | Supervisor on site | 20-21 /01/25 & 24/01/25 |  |
| 4 | Transport machine to loading area | Unstable load during moving or turning | Bodily Injury & Damage to Property if machine topper | - Ensure rollers are place accordingly on machine base - Ensure rollers are properly secured onto machine base with bolts and nuts - Tighten bolts and nuts with adjustable spanner - Ensure machine load are spread evenly and balance well on the rollers before being transport to location - Workers to be alert and keep close lookout on route of travel from point to point and to ensure rollers are not moving out of position | 5 | 2 | 10 | Slow moving speed Ensure floor is properly sweep and clear of any debris on the floor in case rollers got stuck by the debris | 5 | 1 | 5 | Supervisor on site | 20-21 /01/25 & 24/01/25 |  |
|  |  | Excessive weight load | Sprain back if force moving heavy load | - Use the correct capacity rollers for the move - Proper tool such as manual puller to be brought to site - Supervisor to ensure there are adequate personnel to be deploy for the job - Use of waist belt, if needed, to prevent sprain back or backache | 3 | 2 | 6 | Use of manual puller when needed to assist in pulling the machine | 3 | 1 | 3 | Supervisor on site | 20-21 /01/25 & 24/01/25 |  |
|  |  | Trip and fall | Sprains & strains | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Keep the floor in the work area dry and clean - Oil/water spill must be clean up immediately - Tape down power cords or cable if any to prevent from tripping | 2 | 2 | 4 | Supervisor to make sure route of transporting before moving and double check clearing of all material and tooling | 2 | 2 | 4 | Supervisor on site | 20-21 /01/25 & 24/01/25 |  |
| 5 | Setting up of crane on site | Hit by moving parts of crane | Bodily Injury & Damage to Property | - Operator trained and competent person appointed - Crane operator and lifting supervisor to ensure operation area and crane swing path is clear from any obstruction - Crane operator to be on site when extending or retracting outriggers (must be able to see all operation procedure) - Ensure all outrigger beams must be fully extended and resting on steel plate/timber mat as base plate - PPE (safety shoe, safety helmet) provided and put on | 5 | 2 | 10 | Ensure no unauthorised personnel or vehicle near moving parts of crane Barricade off the work area | 5 | 1 | 5 | Lifting supervisor on site | 20-21 /01/25 & 24/01/25 |  |
| 6 | Insert sling to crane hook and secured to rigging point of machine | Rigger hand caught in between the LG and hook | Minor fracture to hand | - Rigger to attached and secured sling with care - To coordinate well with crane operator to ensure no lifting are done until all sling are properly attached and secured Use of glove | 3 | 2 | 6 | Signalman to command crane operator when to lift and supervisor to ensure all worker are hand off any loose part of lifting gears | 3 | 1 | 6 | Lifting supervisor on site | 20-21 /01/25 & 24/01/25 |  |
|  |  | Pinch point | Injury to hand | - PPE (safety glove) provided and put on - Never place hand where you can’t see them - If view is obstructed, do not proceed with the work until having a clearer view | 3 | 2 | 6 | Always stay vigilant and pay close attention to surrounding area | 3 | 1 | 3 | Lifting supervisor on site | 20-21 /01/25 & 24/01/25 |  |
| 7 | Lifting operation | Failing of lifting equipment | Bodily Injury & Damage to Property | - All lifting equipment are tested by approved person and has valid certificate - Ensure LM for crane are valid - Riggers to ensure lifting gears use are in compliance with weight of equipment - No defective gears allowed - Never stand close or under the load - PPE (safety helmet, safety shoe) provided and put on | 5 | 2 | 10 | While moving or slewing the crane, try not to jerk the load Ensure lifting supervisor to be 100% onsite to supervisor the whole lifting operation No lifting is to be carried out during bad weather | 5 | 1 | 5 | Lifting supervisor on site | 20-21 /01/25 & 24/01/25 |  |
|  |  | Overloading | Bodily Injury & Damage to Property | - No lifting beyond SWL of equipment - Load weight should be known - To test lift load about 1ft above ground to check on stability and to ensure no overloading of load to be lifted - Implement of Lifting Plan to all lifting personnel on site to ensure that the lifting operation is properly manage | 5 | 2 | 10 | Ensure load, capacity of lifting arm and reach are within limit, when in doubt, always use a higher capacity crane | 5 | 1 | 5 | Lifting supervisor on site | 20-21 /01/25 & 24/01/25 |  |
|  |  | Struck by objects due to unsafe slinging, rigging | Bodily Injury & Damage to Property | - Entire lifting crew is trained and to have relevant skill, certificate and experiences required to carry out tasks - Machine must be securely slug, properly balanced and trimmed to prevent from dislodgment - Ensure machine is rig at the centre of gravity - Riggers to check and ensure safe rigging and lifting supervisor to confirm load are safe to be rig | 5 | 2 | 10 | Communication through hand signal or via walkie-talkie by Eric Wong personnel only To provide crane hook with safety latch to prevent displacement of sling or load from the hook | 5 | 1 | 5 | Lifting supervisor on site | 20-21 /01/25 & 24/01/25 |  |
|  |  | Hand caught by or in between loose part of sling or lifting gears | Minor fracture o hand and arm | - Keep hands off loose part of sling or lifting gears - Ensure signalman give proper means of signal to crane operator - To coordinate well with crane operator - Use glove | 3 | 2 | 6 | Signalman to command crane operator when to lift and supervisor to ensure all worker are hand off any loose part of lifting gears | 3 | 1 | 3 | Lifting supervisor on site | 20-21 /01/25 & 24/01/25 |  |
| 8 | Signalling of load | View obstruction or load swing | Bodily Injury & Damage to Property | - Only qualified signalman to execute the operation - Tagline should be used to control load from swinging so as to prevent bodily contact with load - Clear obstruction and unauthorised personnel in way of lifting path - Never stand close or under the suspended load and away from possible swing path - Signalman to position himself to be visible to crane operator through hand signalling method - PPE (safety helmet, safety shoe) provided and put on | 5 | 2 | 10 | Use of walkie-talkie whenever signalman is not visible to crane operator Close supervision by lifting supervisor | 5 | 1 | 5 | Lifting supervisor on site | 20-21 /01/25 & 24/01/25 |  |