Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 26 AMK Ind Pk 2 | Moving Operation | 1 | Reverse lorry to loading bay |  |
|  |  |  | 2 | Transport machine from production to loading bay using pallet truck |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader : Kevin Wong Project Manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Moving Operation | RA Member : Zailani |  |  |  |
| Location: | 26 A.M.K Industrial Pk 2 Meiban Innovation Center | RA Member : Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2020 |  | Designation: | Director |  |
| Next Review Date | 30/06/2023 |  | Date: | 19/01/2021 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Reverse lorry to loading bay | Collision of moving vehicle onto personnel or site property | Bodily injury & damage to property | - Vehicle only to be moved into site with the aid of banks man guiding the driver - Clear all obstruction in way of moving path - Drive at slow speed when reversing lorry to loading bay - Ensure driver is guided by other staff | 4 | 2 | 8 | Vehicle must not be left unattended while engine is running | 4 | 1 | 4 | Supervisor on site | 20/01/21 |  |
| 2 | Transport machine from production to loading bay using pallet truck | Unstable load while moving or turning | Bodily injury & damage to property if load topper | - Ensure pallet truck are place accordingly on machine base - Ensure machine load are spread evenly and balanced well on the pallet truck before being transport to location - Buddy system/one to control the pallet truck steering while the others to push and hold onto the machine - To ensure adequate personnel are to be deploy for the job - Workers to be alert and keep close lookout on route of travel from point to point | 4 | 2 | 8 | Transport machine at slow moving speed | 4 | 1 | 4 | Supervisor on site | 20/01/21 |  |
|  |  | Passage way block by materials and tooling | Trip & fall | - Supervisor to take note on route of transporting - To ensure passage way and working area are to be keep clear of any materials and tooling before moving job - Proper housekeeping to be maintained during work - Oil/water spill must be clean up immediately | 2 | 2 | 4 | Good coordination between workers to ensure all materials or tooling are move aside before any moving are done | 2 | 1 | 2 | Supervisor on site | 20/01/21 |  |
|  |  | Obstruction to vision while moving | Damage to property when hitting other obstacle | - To keep close lookout on route of travel - Ensure movement is guided - To alert personnel controlling the pallet truck steering if potential hitting of other obstacles | 2 | 2 | 4 | Provision of spotter at both end so as to have a clearer view before hitting any obstacles | 2 | 1 | 2 | Supervisor on site | 20/01/21 |  |