Inventory of Work Activities

| Ref No. | Location |  | Process | S/No. | Remarks |
| --- | --- | --- | --- | --- | --- |
|  | 16E Tuas Ave 1 |  | Mobile/lorry crane & moving operation | 1 | Transport of lifting machinery into or out of site property |
|  |  |  |  | 2 | Jack up machine using jack |
|  |  |  |  | 3 | Insert rollers under machine bed |
|  |  |  |  | 4 | Transport machine to loading area |
|  |  |  |  | 5 | Setting up of crane on site |
|  |  |  |  | 6 | Lifting operation |
|  |  |  |  | 7 | Forklift operation – to fork accessories |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by : Eric Wong Signature | Reference Number: |
| --- | --- | --- | --- | --- |
| Process: | Mobile Crane & Moving Operation | RA Member: Zailani |  |  |
| Activity Location: | 16E Tuas Ave 1 #01-61 Hunting Energy Services Pte Ltd | RA Member: Daniel Wong |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: Eric Wong |  |
| Last Review Date: | 01/07/2023 |  | Designation: Director |  |
| Next Review Date | 30/06/2026 |  | Date: 08/04/2025 |  |

| Hazard Identification |  |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard |  | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |  |
| 1 | Transport of lifting machinery into or out of site premises | Collision of moving vehicle onto personnel’s or site property |  | Bodily Injury & Damage to Property | - To coordinate with occupier for safe route access - Vehicle only to be moved into site with the aid of banks man guiding the driver - Clear all obstruction in way of moving path - Driver to drive at slow speed and observe speed limit when entering into customer premise | 4 | 2 | 8 | Vehicle must not be left unattended while engine is running | 4 | 1 | 4 | Supervisor on site | 14/04/25 - 17/04/25 |  |  |
| 2 | Jack up machine using jack | Machine topper due to lost of balance upon lifted up |  | Bodily Injury & Damage to Property | - Worker to be alert when jack up machine - Ensure machine are jack up at the right angle of gravity - Jack up and lower down machine from one side to another is carried out in stages with the use of hard wooden block - Supervisor to ensure operation area is clear of unauthorised personnel | 4 | 2 | 8 | Only skill and experience worker to control the jack handling | 4 | 1 | 4 | Supervisor on site | 14/04/25 - 17/04/25 |  |  |
| 3 | Insert rollers under machine bed | Sharp edges of machine |  | Cuts from sharp edges | - To keep hand/finger clear of sharp edges - Insert rollers to below machine with care - PPE (safety glove) provided and put on | 2 | 2 | 4 | Close supervision by supervisor | 2 | 1 | 2 | Supervisor on site | 14/04/25 - 17/04/25 |  |  |
|  |  | Pinch point |  | Injury to hand | - Never place hand where you can’t see them - If view is obstructed, do not proceed with the work until having a clearer view - To standby solid woodblock below machine and remove only after rollers are properly secured onto machine bed | 3 | 2 | 6 | Always stay vigilant and pay close attention to surrounding area | 3 | 1 | 3 | Supervisor on site | 14/04/25 - 17/04/25 |  |  |
| 4 | Transport machine to loading area | Unstable load during moving or turning |  | Bodily Injury & Damage to Property if machine topper | - Ensure rollers are place accordingly on machine base - Ensure machine load are spread evenly and balanced well on the rollers before being transport to location - Workers to be alert and keep close lookout on route of travel from point to point and to ensure rollers are not moving out of position - Ensure travelling ground is well protected with canvas/metal sheet in contact with the rollers at all time - In case of motor operate roller fail to function, will replace with manual rollers on standby | 4 | 2 | 8 | Slow moving speed Ensure floor is properly sweep and clear of any debris on the floor in case rollers got struck by the debris | 4 | 1 | 4 | Supervisor on site | 14/04/25 - 17/04/25 |  |  |
|  |  | Trip & fall |  | Sprains & strains | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Oil/water spill must be clean up immediately - Tape down any power cords or cable if any to prevent from tripping | 2 | 2 | 4 | Supervisor to make sure route of the transporting before moving and to double check clearing of all materials and tooling | 2 | 1 | 2 | Supervisor on site | 14/04/25 - 17/04/25 |  |  |
|  |  | Obstruction to vision while moving |  | Damage to Property when hitting other obstacles | - To keep close lookout on route of travel - Ensure movement is guided - To alert personnel controlling the rollers steering if potential hitting of other obstacles | 1 | 2 | 2 | Provision of spotter at both end of machine so as to have a clearer view before hitting any obstacles | 1 | 2 | 2 | Supervisor on site | 14/04/25 - 17/04/25 |  |  |
| 5 | Setting up of crane on site | Hit by moving parts of crane |  | Bodily Injury & Damage to Property | - Operator trained and competent person appointed - Crane operator to be on site when extending or retracting outriggers (must be able to see all operation procedure) - Ensure all outrigger beams must be fully extended and resting on steel plate/timber mats as base plate - Crane operator and lifting supervisor to determine the load radius and slewing part of the crane to ensure no hitting of nearby building or equipment or passerby - PPE (safety shoe, safety helmet) provided and put on | 4 | 2 | 8 | Ensure no unauthorised personnel or vehicle near moving parts of crane Check on the crane to be carried out prior to lifting Load chart to be available and studied prior to lifting Overloading device on the crane to be tested before actual lifting operation All hazard pertaining to the operation is to made known to client and workers Barricade off the work area Signage to be put up at the barricade area and if necessary, a stand-by man to direct traffic or people | 4 | 1 | 4 | Lifting supervisor on site | 14/04/25 - 17/04/25 |  |  |
| 6 | Lifting operation | Failing of lifting equipment |  | Bodily Injury & Damage to Property | - All lifting equipment are tested by approved person and has valid certificate - Ensure LM for mobile crane are valid - Riggers to ensure lifting gears use are in compliance with weight of machine - No defective gears allowed - Never stand close or under the load - PPE (safety helmet, safety shoe) provided and put on | 4 | 2 | 8 | While moving or slewing the crane, try not to jerk the load Ensure lifting supervisor to be 100% onsite to supervised the whole lifting operation No one is allowed to stand under the suspended load and to be within the working radius of the crane No lifting is to be carried out during bad weather | 4 | 1 | 4 | Lifting supervisor on site | 14/04/25 - 17/04/25 |  |  |
|  |  | Overloading |  | Bodily Injury & Damage to Property | - No lifting beyond SWL of equipment - Load weight should be known - To test lift load about 1ft above ground to check on stability and to ensure no overloading of load to be lifted - Implement of Lifting Plan to all personnel on site to ensure that lifting operation is properly manage | 4 | 2 | 8 | Ensure load, capacity of lifting arm and reach are within limit, when in doubt, always use a higher capacity crane | 4 | 1 | 4 | Lifting supervisor on site | 14/04/25 - 17/04/25 |  |  |
|  |  | Struck by objects due to unsafe slinging, rigging |  | Bodily Injury & Damage to Property | - Entire lifting crew is trained and to have relevant skill, certificate and experiences required to carry out tasks - Load must be securely slug, properly balanced and trimmed to prevent from dislodgment - Ensure machine is rig at the centre of gravity - Riggers to check and ensure safe rigging and lifting supervisor to confirm load are safe to be rig | 4 | 2 | 8 | Communication through hand signal or via walkie-talkie by Eric Wong personnel only To provide crane hook with safety latch to prevent displacement of sling or load from the hook | 4 | 1 | 4 | Lifting supervisor on site | 14/04/25 - 17/04/25 |  |  |
|  |  | Hand caught by or in between loose part of sling or lifting gears |  | Arm or Hand Injury | - Keep hands off loose part of sling or lifting gears - Ensure signalman give proper means of signal to crane operator - Use glove | 3 | 2 | 6 | Signalman to command crane operator when to lift and supervisor to ensure all worker are hand off any loose part of lifting gears | 3 | 1 | 3 | Lifting supervisor on site | 14/04/25 - 17/04/25 |  |  |
|  |  | View obstruction or load swing |  | Bodily Injury & Damage to Property | - Only qualified signalman to execute the operation - Tagline should be use to control load from swinging so as to prevent bodily contact with the load - Clear obstruction and unauthorised personnel in way of lifting path - Signalman to position himself to be visible to crane operator through hand signalling method | 4 | 2 | 8 | Use of walkie-talkie whenever signalman is not visible to crane operator Close supervision by lifting supervisor | 4 | 1 | 4 | Lifting supervisor on site | 14/04/25 - 17/04/25 |  |  |
| 7 | Forklift operation – to fork accessories | Falling load |  | Bodily Injury & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Known the CG of the load - Elevate the load slightly and fully tilt the mast backward - Ensure accessories load are spread evenly and balanced well on the fork - Ensure o unauthorised personnel is within the working zone | 4 | 2 | 8 | Use of extension fork when needed to give greater stability of load | 4 | 1 | 4 | Supervisor on site | 14/04/25 - 17/04/25 |  |  |
|  |  | Hitting people or property |  | Bodily Injury & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Forklift driver advised to be careful when moving forward or reversing - Ensure working area is clear of unauthorised personnel - Press horn if necessary to warn others that forklift is approaching | 4 | 2 | 8 | Assignment of traffic controller when needed to control movement of vehicle or people within the vicinity | 4 | 1 | 4 | Supervisor on site | 14/04/25 - 17/04/25 |  |  |