Inventory of Work Activities

| Company: Eric Wong Machinery Transportation Pte Ltd |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 249 Jalan Boon Lay | Moving operation | 1 | Cutting of plywood using saw |  |
|  |  |  | 2 | Manual carrying of plywood to paste onto container |  |
|  |  |  | 3 | Forklift operation - to fork up case into container |  |
|  |  |  | 4 | Insert rollers/pallet truck under case |  |
|  |  |  | 5 | Secured chocking wood using battery operate drill |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Moving Operation | RA Member: Zailani |  |  |  |
| Location: | 249 Jalan Boon Lay ST Engineering Synthesis Pte Ltd | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2023 |  | Designation: | Director |  |
| Next Review Date | 30/06/2026 |  | Date: | 25/06/2025 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Cutting of plywood using saw | Cut from sharp edges | Hand / Finger injury | - Ensure operator operating the saw are competent in the use of tool issue to them - Ensure tool issue is in good working condition and properly maintained - Always hold saw firmly with both hand to avoid kickback or slippage - Maintain firm footage and avoid working on uneven slippery ground - Never operate any tools if you’re feeling fatigue - PPE (eye & ear protection) provided and put on | 4 | 2 | 8 | To remain alert and aware of potential risk around surrounding area | 4 | 1 | 4 | Supervisor on site | TBA |  |
| 2 | Manual carry of plywood to paste onto container | Manual handling of heavy load | Musculoskeletal disorder | - Adopt proper technique when carrying task - Apply buddy system when carrying heavy load - Supervisor to ensure there are adequate manpower to be deploy for the job - Minimise carrying distances of load where possible - Reduce weight load when possible by splitting the load into smaller portion | 3 | 2 | 6 | Ensure sufficient rest and recovery time is taken during repeated lift | 3 | 1 | 3 |  | TBA |  |
| 3 | Forklift operation - to fork up case into container | Falling load | Bodily Injury & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Ensure forklift fork are properly aligned directly to the lifting point of load - Known the CG of the load - Ensure plate load are spread evenly and balanced well on the fork - Elevate the load slightly and fully tilt the mast backward - Ensure no unauthorised personnel is within the working area | 5 | 2 | 10 | Use extension fork if required to give greater stability of load | 5 | 1 | 5 | Supervisor on site | TBA |  |
|  |  | Hitting people or property | Bodily Injury & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Operator advised to be careful when moving forward or reversing - Ensure forklift driver is guided by other staff - Press horn if necessary to warn others that forklift is approaching | 5 | 2 | 10 | Assign traffic controller to control movement of vehicle and people within the vicinity | 5 | 1 | 5 | Supervisor on site | TBA |  |
| 4 | Insert rollers/ pallet truck under case | Pinch point | Injury to hand | - PPE (safety glove) provided and put on - Insert roller/pallet truck to below case with extra care - Never place hand where you can’t see them - If view is obstructed, do not proceed with the work until having a clearer view | 3 | 2 | 6 | Always stay vigilant and pay close attention to surrounding area | 3 | 1 | 3 | Supervisor on site | TBA |  |
| 5 | Secured chocking wood using battery operate drill | Contact with drill bit rotation | Hand / Finger injury | - Ensure operator operating the drill are competent in the use of tool issue to them - Keep hand/finger away from the rotating parts of the drill - Always hold drill firmly with both hand to avoid kickback or slippage - Maintain steady pace and avoid applying excessive pressure on the drill as it may result in loss control or kickback - If the drill bit get struck, release the trigger immediately to prevent injury | 4 | 2 | 8 | To remain alert and aware of potential risk around surrounding area | 4 | 1 | 4 | Supervisor on site | TBA |  |