Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 26 Changi North Rise | Relocate machine within same room | 1 | Relocate machine within same room using pallet truck |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong Project Manager | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Relocate machine within same room | RA Member: Zailani |  |  |  |
| Location: | 26 Changi North Rise Safran Electronics & Defense Services Asia PL | RA Member: Daniel Wong |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2023 |  | Designation: | Director |  |
| Next Review Date | 30/06/2026 |  | Date: | 07/08/2025 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Relocate machine within same room using pallet truck | Unstable load during moving or turning | Bodily injury & damage to property if machine topper | - Ensure pallet truck are place accordingly on machine bed - Ensure machine load are spread evenly and balanced well on the pallet truck before being transport to designated new location - Buddy system-one to control the pallet truck steering while the others to push and hold onto the machine - Workers to be alert and keep close lookout on route of travel from point to point | 4 | 2 | 8 | Slow moving speed | 4 | 1 | 4 | Supervisor on site | 18/08/25 |  |
|  |  | Trip & fall | Sprains & strains | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Keep floor in the work area dry and clean - Oil/water spill must be clean up immediately - Tape down any power cords or cable if any to prevent from tripping | 2 | 2 | 4 | Supervisor to make sure route of transporting before moving and to double check clearing of all materials and tooling | 2 | 1 | 2 | Supervisor on site | 18/08/25 |  |
|  |  | Obstruction to vision while moving | Damage to property if hit other obstacles | - To keep close lookout on route of travel - Ensure movement is guided - To alert worker controlling the pallet truck steering if potential hitting of other obstacles | 1 | 2 | 2 | Provision of spotter at both end of machine so as to have a clearer view before hitting any obstacles | 1 | 2 | 2 | Supervisor on site | 18/08/25 |  |