Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 273 Jalan Ahmad Ibrahim | Relocation of machine | 1 | Jack up machine using manual/hydraulic jack |  |
|  |  |  | 2 | Insert pallet truck |  |
|  |  |  | 3 | Transportation of machine to new location |  |
|  |  |  | 4 | Using forklift to assist in pulling/pushing the machine |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader : Kevin Wong | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Relocation of machine | RA Member : Zailani |  |  |  |
| Location: | No 273 Jalan Ahmad Ibrahim Baker Hughes Singapore Pte Ltd | RA Member : Zaharin |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 15/05/2015 |  | Designation: | Director |  |
| Next Review Date | 15/05/2018 |  | Date: | 04/01/2017 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Jack up machine using manual/ hydraulic jack | Machine topple due to lost of balance upon lifted up | Bodily Injury & Damage to Property | - Workers to be alert when jack up machine - Ensure machine are jack up at the right angle of gravity - Ensure jack use are in good working condition and able to lift weight of machine - Lift up machine to 3” high from one side to another is carried out in stages with the use of hard wooden block | 4 | 2 | 8 | Only skill and experience worker to control the jack handling | 4 | 2 | 8 | Supervisor on site | 07/01/17 |  |
| 2 | Insert rollers/ pallet truck | Sharp edges of machine | Hand/Finger Injury | - Keep hand/finger clear of sharp edges - Insert rollers/pallet truck to below machine base with care - Use of suitable hand glove | 2 | 2 | 4 | Close supervision by supervisor | 2 | 2 | 4 | Supervisor on site | 07/01/17 |  |
| 3 | Transportation of machine to new location | Rollers move out of position causing machine to topple | Bodily Injury & Damage to Property | - Ensure rollers are place accordingly on machine base - Ensure rollers are properly secured to machine base with bolts and nuts to prevent from rolling out - Ensure machine load are spread evenly and is stable on the rollers/pallet truck before being transport to new location - Workers to be alert and keep close lookout on route of travel from point to point and to ensure rollers are not moving out of position | 4 | 2 | 8 | Slow transportation speed | 4 | 2 | 8 | Supervisor on site | 07/01/17 |  |
|  |  | Excessive weight load | Sprain Back | - Use the correct capacity pallet truck/rollers to reduce the effort of each man when pushing the equipment - Deploy adequate personnel | 3 | 3 | 9 | Use manual puller or forklift if possible to assist in puling/pushing the machine | 3 | 2 | 6 | Supervisor on site | 07/01/17 |  |
|  |  | Passageway block by equipment and tool | Trip & Fall | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Oil/water spill must be clean up immediately | 2 | 2 | 4 | Good coordination | 2 | 2 | 4 | Supervisor on site | 07/01/17 |  |
|  |  | Obstruction to vision while moving | Damage to Property | - To keep close lookout on route of travel - Ensure movement is guided - To alert worker controlling the pallet truck/rollers steering if potential hitting of other obstacles | 2 | 2 | 4 | Provision of spotter | 2 | 2 | 4 | Supervisor on site | 07/01/17 |  |
| 4 | Using forklift to assist in pulling/ pushing machine | Hitting people or property | Bodily Injury & Damage to Property | - Only certified and competent forklift driver are allowed to operate the forklift - Forklift driver to check whether anyone near or behind the forklift before proceeding to drive - Ensure working area is clear of any obstruction and unauthorised personnel | 4 | 2 | 8 | Assignment of traffic controller | 4 | 2 | 8 | Supervisor on site | 07/01/17 |  |
|  |  | Restricted driver visibility | Bodily Injury & Damage to Property | - Forklift driver advised to be careful when moving forward or reversing - Forklift equipped with side mirror and reverse warning alarm to cautioned nearby people - Ensure forklift driver is guided if needed - Press horn if necessary to warn others that forklift is approaching | 3 | 2 | 6 | Assignment of traffic controller | 3 | 2 | 6 | Supervisor on site | 07/01/17 |  |