Inventory of Work Activities

| Company: ERIC WONG MACHINERY TRANSPORTATION PTE LTD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ref No. | Location | Process | S/No. | Work Activity | Remarks |
|  | 249 Jalan Boon Lay | Relocation of machine | 1 | Jack up machine with pallet truck |  |
|  |  |  | 2 | Transportation of machine to new location |  |

| Company: | Eric Wong Machinery Transportation Pte Ltd | RA Leader: Kevin Wong | Approved by: Eric Wong Signature: |  | Reference Number: |
| --- | --- | --- | --- | --- | --- |
| Process: | Relocation of CMM machine | RA Member: Zailani |  |  |  |
| Location: | No 249 Jalan Boon Lay Advanced Material Engineering Pte Ltd | RA Member: Zaharin |  |  |  |
| Original Assessment Date: | 15/05/2009 |  | Name: | Eric Wong |  |
| Last Review Date: | 01/07/2017 |  | Designation: | Director |  |
| Next Review Date | 30/06/2020 |  | Date: | 24/01/2018 |  |

| Hazard Identification |  |  |  | Risk Evaluation |  |  |  | Risk Control |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref | Work Activity | Hazard | Possible Injury/Ill-health | Existing Risk Controls | S | L | RPN | Additional Controls | S | L | RPN | Implementation Person | Due Date | Remark |
| 1 | Jack up machine with pallet truck | Pallet truck accidentally hit onto machine | Damage to Property | - Insert pallet truck/rollers to below machine base with care - To place rubber mat or any other necessary tools on top pallet truck/rollers before inserting under machine base to protect machine from scratches or crack | 2 | 2 | 4 | Close supervision by supervisor | 2 | 1 | 2 | Supervisor on site | 01/02/18 |  |
| 2 | Transportation of machine | Unstable load during moving or turning | Bodily Injury & Damage to Property when machine topper | - Ensure pallet truck/rollers are place accordingly on machine base - Ensure pallet truck/rollers are properly secured onto machine base with bolts and nuts - Ensure machine load are spread evenly and is stable on the pallet truck/rollers before being transport from point to point - Workers to be alert and keep close lookout on route of travel from point to point | 4 | 2 | 8 | Slow transportation speed | 4 | 1 | 4 | Supervisor on site | 01/02/18 |  |
|  |  | Excessive weight load | Sprain Back when pushing heavy load | - Use the correct capacity pallet truck/rollers to reduce the effort of each man when pushing the machine - Supervisor to ensure there are adequate manpower to deploy for the job | 3 | 3 | 9 | Use manual puller or forklift if possible to assist in puling/pushing the machine | 3 | 2 | 6 | Supervisor on site | 01/02/18 |  |
|  |  | Passageway block by materials and tooling | Trip & Fall | - Passage way and working area to be keep clear of materials and tooling before moving job - Proper housekeeping to be maintained during work - Oil/water spill must be clean up immediately | 2 | 2 | 4 | Supervisor to make sure route of transporting before moving and to double check clearing of all materials and tooling | 2 | 1 | 2 | Supervisor on site | 01/02/18 |  |
|  |  | Obstruction to vision while moving | Damage to Property | - To keep close lookout on route of travel - Ensure movement is guided - To alert worker controlling the pallet truck/rollers steering if potential hitting of other obstacles | 2 | 2 | 4 | Provision of spotter at both end of machine so as to have a clearer view before it hit any obstacles | 2 | 1 | 2 | Supervisor on site | 01/02/18 |  |